/SA WG2 Meeting #162	S2-2405486

# April 15 – April 19, 2024, Changsha, China	(revision of S2-2404245)

Source: 	vivo, CMCC, CATT, Huawei, HiSilicon, Samsung, China Unicom, KDDI, Lenovo, ZTE, InterDigital Inc., Tencent

Title: 	Interim evaluation and conclusion for KI3

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution tries to provide the Interim evaluation and conclusion for KI3.

- Introduction

This paper is trying to evaluate the solutions captured for KI#3(5GS enhancements for network energy saving and efficiency), provide the conclusions for the KI#3 based on the captured solutions.

2	Proposal

It is proposed to capture the following solution in TR 23.700-66.

Start of Changes

8.X	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

Following interim conclusions for KI#3 are proposed:

Editor's note:	The conclusions need to be further revised.

- For Enhancements on NF discovery and (re-)selection based on energy related information :

-	NF profile is extended (e.g., include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

Editor's note: It’s FFS if the information in NF profile is static information or dynamic energy related information.

-  NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

- Enhancements on existing operations and procedures for energy saving and energy efficiency:

-	Access management, session management, UP path adjustment may be enhanced taking the energy related information into account.

-  For Network slice which is subject to energy control, e.g.,

determination of accept/reject of requested S-NSSAIs or PDU session, and modification/release of PDU session may consider energy related information.

- Maximum number of UEs and maximum number of PDU sessions for a network slice maybe adjusted by considering energy related information when it is subject to NSAC

- AMF may retrieve energy related subscription information (e.g., the Green Subscription Indication (GSI) or S-NSSAIs associated with specific energy behaviour) from the UDM during registration and provide it to RAN.

NOTE:	The energy related decision will also consider operator’s policy.

Editor’s note: Whether the enhancement on existing operations and procedures can assist saving energy and energy efficiency is FFS.

- For NWDAF-Based Energy Analytics for network energy saving and efficiency:

-	NWDAF is taken as the role of energy related analysis.

Editor’s note: Whether the NWDAF can assist saving energy and energy efficiency (e.g. considering the EC of generating analytics) will be further evaluated during the conclusion discussion.

Editor's note: Energy related information required and obtained to support 5GS enhancement for energy saving and energy efficient is dependent on KI#1 conclusions.

End of Changes