3GPP SA WG2 Meeting #162	S2-2405540

Changsha, China, April 15th- 19th, 2024

Source:	Huawei, HiSilicon, Samsung

Title:	KI#1 New Sol: Multiple USS support during the UAV flight path

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_ UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes a solution to support multiple USS during the UAV flight path for KI#1 of FS_UAS_Ph3.

Discussion

The aim of the key issue #1 is to study whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting: pre-mission flight planning and in-mission flight monitoring for UAVs, C2 communication reliability and the scenario of multiple USS serving the geographical areas corresponding to the UAV flight path. This paper focuses on the scenario of multiple USS serving the geographical areas corresponding to the UAV flight path. and proposes a solution for the referred part.

The serving USS of UAV may change when the UAV flight path cross the geographical areas served by different USS.

The target USS will take control of the mobile UAV in replacement with the source USS. With the process of USS relocation, here come several problems to be solved.

Q1: How to determine the USS change/relocation?

The USS change is the application server change so the problem can reuse or refer to the EAS relocation solution as described in TS 23.558. Each EAS server has a EAS Geographical Service Area and can requests UE location information to EES. The EES is authorized to use Nnef Event Exposure API for UE location based on service level agreement with MNO. When the S-EAS detects the UE location out of the S-EAS Geographical Service Area, it discovers the T-EAS and determines the T-EAS to use.

Back to the mentioned USS change problem, we can propose that when the serving USS changes due to the UAV mobility, the S-USS can detect the UAV out of the serving geographical areas and determines the T-USS.

Proposal 1: The S-USS can detect the UAV out of the serving geographical areas and determine the T-USS.

Q2: What will be the impacts due to USS change?

When the UAV moves from the S-USS to the T-USS, the S-USS may initiate UUAA revocation and transfer the UAV context to T-USS.

As defined in TS 23.256, a UAV is assigned a CAA-level UAV Identity by functions in the aviation domain (e.g. USS) and this assigned identity is used for Remote Identification and Tracking and to identify the UAV. When the UAV moves to the T-USS coverage area, the T-USS can not use the old CAA-level UAV Identity in the Naf_Authentication_Notification request to UAS NF. The S-USS should indicate the USS change and trigger UVA to report the new CAA-level UAV Identity. To support the current CAA-level UAV Identity report corresponding to the serving USS (e.g., first report in the UUAA registration procedure), the UAV should be configured with the mapping information between the CAA-level UAV ID and the USS information.

Proposal 2: The S-USS should indicate the USS change and trigger UAV to report the new CAA-level UAV Identity for UUAA authorization.

Proposal 3: The UAV should be configured with the mapping information between the CAA-level UAV ID and the USS information for the current CAA-level UAV Identity report.

Proposal

A solution is proposed for KI#1 in the UAS Ph3 TR 23.700-59.

*** Start of first changes ***

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 |  | X |  |
| #5 |  | X |  |
| #6 |  | X |  |
| #7 |  |  | X |
| #8 |  |  | X |
| #9 |  |  | X |
| #x | X |  |  |

*** Start of Second changes ***

6.X	Solution #X: Multiple USS support during the UAV flight path

6.X.1	Key Issue mapping

This solution addresses Key Issues #1 which focuses on the scenario of multiple USS serving the geographical areas corresponding to the UAV flight path.

The USS change is to consider the two potential problems: how to detect the USS change and what will be the impacts due to USS change.

6.X.2	Description

The serving USS of UAV may change when the UAV flight path cross the geographical areas served by different USS.

The target USS will take control of the mobile UAV in replacement with the source USS. With the process of USS relocation, how to detect the USS change and what will be the impacts due to USS change need to be discussed in the solution. After establishing connection with the UAV, the serving USS(S-USS) may request UAV location from the 5GS so that it can detect whether the UAV is about to leave its coverage area and subsequently there is a need to relocate the UAV to another USS(target USS, T-USS).

Proposal 1: The S-USS can detect the UAV out of the serving geographical areas and determine the T-USS.

As described in TS 23.256, a CAA-level UAV Identity may be assigned by functions in the aviation domain (e.g. a single USS).

CAA (Civil Aviation Administration)-Level UAV Identity: a UAV identity assigned by USS/UTM, and uniquely identifies a UAV at least within the scope of a USS.

Due to this reason, when UAV moves to the T-USS coverage area, the T-USS may not use the old CAA-level UAV Identity. In this case, the S-USS should indicate to UAV of the USS change and trigger UAV to report the new CAA-level UAV Identity for UUAA authorization.

It is assumed that UAV is configured with the mapping information between the CAA-level UAV ID and the USS information and send the corresponding CAA-level UAV Identity to the target USS for UUAA authorization.

Editor's note:	if UAV is configured with only one CAA-level UAV ID by a certain USS, how to ensure this CAA-level UAV ID can be used in other USS domain is FFS.

Proposal 2: The S-USS should indicate the USS change and trigger UAV to report the new CAA-level UAV Identity for UUAA authorization.

Proposal 3: The UAV should be configured with the mapping information between the CAA-level UAV ID and the USS information for the current CAA-level UAV Identity report.

6.X.3	Procedures

Figure 6.X.3-1: Procedure for USS change

- The UAV is configured with the mapping information between the CAA-level UAV ID and the USS information for the current CAA-level UAV Identity report. For each configured CAA-level UAV ID, it has the corresponding geographical area for its usage.

- The UAV performs Authentication and Authorization procedures. Details can be seen in TS 23.256 clause 5.2.2/5.2.3. When providing the CAA-level UAV ID to the network for authentication, and if configured multiple CAA-level UAV IDs locally, UAV selects the CAA-level UAV ID based on its current location and the applicable geographical area corresponding to the CAA-level UAV ID.

2.	The S-USS detects the UAV out of the serving geographical areas and determines the T-USS.

Editor’s note: other information, in addition to UAV location can be used to determine the T-USS is FFS.

3.	The S-USS transfers the UAV context to the T-USS. This interaction is out of 3GPP scope.

4.	The S-USS sends a Nnef_Authentication_Notification  request to UAS NF for authentication of the UAV. The USS includes GPSI, CAA-Level UAV ID report indication and the T-USS address, if available in the request. The CAA-Level UAV ID report indication and T-USS address indicates the UAV to report the new CAA-Level UAV ID corresponding to the T-USS address. The CAA-Level UAV ID report is sent when the S-USS determines the CAA-level UAV ID cannot be used in the T-USS domain.

5.  UAS NF retrieves the stored UUAA context for the UE. From the stored UUAA context the UAS NF determines the target AMF or SMF for sending the notification.

6.  The UAS NF sends Nnef_Authentication_Notification request to notify the target NF, i.e. either the AMF or the SMF, to initiate CAA-Level UAV ID report of the UAV.

7.  Either the AMF or the SMF triggers the UAV to report the current CAA-level UAV Identity. UAV provides the CAA-level UAV ID UAV corresponding to the T-USS address indicated in step 4 to the network.

8. The AMF or the SMF receives the CAA-level UAV Identity sent by UAV for the communication with T-USS and  use it to invoke UAS NF/NEF service operation for UUAA Authentication/Authorization.

Editor’s note: to communicate with the T-USS, it is FFS whether the PDU session used for communication with the S-USS can be re-used.

6.X.4	Impacts on services, entities and interfaces

AF (USS/UTM):

-	Indicate the USS change and trigger UAV to report the new CAA-level UAV Identity for UUAA authorization with the target USS.

UE(UAV):

-	configured with more than one CAA-level UAV ID, and for each CAA-level UAV ID, has the corresponding applicable geographical area and the USS information

-	support to report the CAA-level UAV Identity corresponding to the TSS address

NEF (UASNF):

- Receive the CAA-Level UAV ID report indication and the T-USS address from S-USS

*** END of changes ***