3GPP TSG-SA2 Meeting #162	S2-2405560

# 15 - 19 April, 2024, Changsha, China							revision of S2-2400580 was 4684

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4800 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to assistance for member UE selection functionality and service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As it has been agreed in normative stage, the member UE selection update including three scenarios, e.g. update the parameters for the Member UE filtering criteria, the Member UE filtering criteria, or the Member UE filtering criteria values. However, this is not clear in the current spec. It should be also clarified that in all three scenarios, AF includes the Subscription Correlation ID in the request, but does not include the list of target member Ues In addition, regarding the change in 5.2.6.32.4: As specified in clause 4.15.13.4 of TS 23.502, in step 11-12, ‘If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify the AF of the given UE which is moving out the coverage’. Based on the description, the NEF should inform AF of the UEs that cannot fulfil UE location filtering criteria or the UEs that are not recommended anymore for FL. However, using the current Nnef_MemberUESelectionAssistance_Notify, the NEF can only include one or more list(s) of candidate UEs to the AF. The description of step 11-12 should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that three scenarios of member UE update and the content of the corresponding AF request. Correct step 11-12 of clause 4.15.13.4: if the UE is out of the FL area, the NEF update the candidate UEs and notify to the AF. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear functionality description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, 4.15.13.4, 5.2.6.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

#### 4.15.13.0	General

With the initial subscription request, the AF provides a list of target UEs in the form of a list of GPSIs or a list of UE IP addresses and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve from 5GC NFs the information for each UE in the list of target member UEs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF.

AF may subsequently update the parameters for the Member UE filtering criteria, the Member UE filtering criteria, or the Member UE filtering criteria values. In all cases, AF includes the Subscription Correlation ID in the request, but does not include the list of target member UEs. AF may request to update the parameters (e.g. expiry time, time window for selecting the candidate UEs) for the subscribed Member UE filtering criteria. AF may also request to update the Member UE filtering criteria by adding new Member UE filtering criteria or removing part of the subscribed Member UE filtering criteria. The AF may also request to update the Member UE filtering criteria values.

The Member UE selection assistance capability can be used to assist the AF to select the list of member UEs to support application service (e.g. FL operation) as described in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how the AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

AF may also unsubscribe the previous subscription for Member UE selection assistance information as described in clause 4.15.13.7.

* * *Next Change * * *

#### 4.15.13.4	Specific procedure for the 5GC assistance to member UE selection based on the UE's current location, historical location and direction and UE separation distance

Figure 4.15.13.4-1: 5GC assistance to Member UE selection based on the UE's current and historical location, direction and separation distance

1.	AF requests 5GS assistance to support the Member UE selection by considering the UE's historical location, UE's current location, direction and UE separation distance. AF includes the UE lists and the following criteria as part of the member UE selection request:

-	UE historical location: The Target AoI where the UEs have been roving over the historical nomadic period before moving into the FL coverage area.

-	UE current location: The current AoI which is the coverage area of the FL training server where the selected UEs located in to participate in the FL operation.

-	UE Direction: Select the UE with the different direction in the FL coverage Area.

-	UE separation distance: Select UEs that are geographically separated, fulfilling the separation distance no smaller than the certain predefined separation distance.

When providing a target area for e.g. FL operation, the AF may provide sub-areas, and provide a maximum number of UEs that should take part in FL from each sub-area.

2.	NEF translates the GPSIs to SUPIs and maps the filtering criteria into the corresponding UE filtering information.

3-4.	The NEF invokes Namf_EventExposure_Subscribe service operation with the UE list, the current AoI and event ID = Location report. AMF will provide a list of UEs that are within the current AoI to the NEF using the Namf_EventExposure_Notify service operation. The NEF obtains the list of possible member UEs from AMF within the current AoI.

5.	In order to identify the appropriate NWDAF which can provide analytics output to derive the visited AOI info and get the UE direction for the possible target member UEs above, the NEF initiates NWDAF discovery request (Analytics ID = UE mobility/Relative Proximity, AOI = Target AOI) with UE list received from step 4.

6-7.	The NEF invokes NWDAF Analytics Info request (UE list received from step 4, Analytics ID = UE mobility, Relative Proximity, Filters include "Visited AoI = Target AOI", "target period = historical nomadic period" and "proximity attributes"). In a response, NWDAF provides a list of UEs that were ever roving within the target AOI, at the minimum, over the historical nomadic period. Additionally, NWDAF provides a list of UEs location in the order of which the UE passes through. Thus, the NEF gets the corresponding statistics of UE mobility, the UE's direction and distance between UEs in the group/list.

8-9.	Based on the information provided by the AMF and NWDAF, the NEF can determine the FL candidate UEs which are now within the FL coverage area but were roving within the target AOI over the historical nomadic period and UE with the different direction and separated from each other fulfilling the minimum distance threshold as requested by AF. The NEF notifies AF for such UE candidate list.

10.	The NEF also needs to consider the list of UEs which are now within the FL coverage area, but may move out of the FL coverage area. Therefore, for each UE in the candidate list in steps 8-9, the NEF invokes the Namf_EventExposure_Notify service with UE ID = SUPI, Event ID=UE moving in/out of AOI in order to keep tracking the movement of the UE(s).

11-12.	If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may update the candidate UEs and notify to the AF.

* * *Next Change * * *

##### 5.2.6.32.4	Nnef_MemberUESelectionAssistance_Notify service operation

Service operation name: Nnef_MemberUESelectionAssistance_Notify

Description: NEF reports the Member UE selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information.

Inputs, Conditional Required:

At least one of the following inputs is required:

-	One or more list(s) of candidate UEs in the form of a list of GPSIs or a list of UE IP addresses.

Inputs, Optional: Recommended time window for performing the application operation per list of candidate UEs as described in clause 4.15.13.1, specific values of the parameters that NEF gathered for the Member UE filtering criteria per candidate UE, a number for each filtering criterion that indicates the amount of UEs in the initial list which do not meet the criterion.

NOTE:	This number can be an indication for AF to revise one or multiple of the corresponding filtering criteria.

Outputs, Required: Operation execution result indication.

* * *End of Changes * * *