3GPP TSG-WG SA2 Meeting #162	S2-2405314

Changsha, China, April 15th – 19th, 2024	(revision of S2-2404412)

Source:	Huawei, HiSilicon

Title:	KI#1: Initial summary and evaluation of the solution to KI#1

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: This contribution proposed an initial summary and evaluation of the solution to KI#1

1. Introduction

There are 7 solutions for the KI#1. In this paper we try to give an initial summary and evaluation of the solution to this KI.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-49.

# * * * * First change (all new text) * * * *

7.x    Evaluation for KI#1: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs

There are 7 solution related to KI#1, i.e. solution #1-#7. The solution can be categorized into 4 groups:

- Group 1: I-SMF based offloading solution.

- Group 2: L-SMF based offloading solution.

- Group 3: Two PDU sessions based solution.

- Group 4: EDI provisioned solution.

G1: I-SMF based offloading solution

I-SMF based offloading solution is that the SMF which control the offloading traffic is put before the anchor SMF, i.e. I-SMF. The Edge related information and handling, i.e. DNS context management, DNS message handling, UL-CL/BP and L-PSA insertion is handled by I-SMF. This includes the solution#1, #5.

NOTE 1: 	In this category even it is called as I-SMF based offloading solution, it does not mean the functionalities define in the I-SMF in this category solution is same as existing defined I-SMF.

G2: L-SMF based offloading solution

L-SMF based offloading solution is that the SMF which control the offloading traffic is put after the anchor SMF. The Edge related information and handling, i.e. DNS message handling, L-PSA insertion is handled by L-SMF. The DNS context management, UL-CL/BP insertion is handled by SMF. This includes the solution#3, #4.

G3: Two A-SMF based offloading solution

Two A-SMF based offloading solution is that the two A-SMF are selected and interact with PCF from SMF/L-SMF separately but combined as one PDU session. The UL CL/BP/L-PSA insertion, interaction with NG-RAN are handled by L-SMF. This includes the solution#2.

G4: EDI provision solution

Solution #6 and #7 introduces EDI provision methods.

# * * * * Next change (all new text) * * * *

8.X	Interim conclusion of KI#1

The following principles are concluded for the normative work.

- Offloading SMF selects L-PSA UPF based on the EAS information within the DNS response message.

Editor’s Note: it is FFS whether the offloading SMF is I-SMF as defined in Group 1 or L-SMF as defined in Group 2 solution.

# * * * * End of changes * * * *