3GPP TSG-SA WG2#162	S2-2403984

Changsha, China, 15 – 19 April 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	KI 2.2: Update of Solution 2.7 to address Editor's Notes

Document for:	Discussion/Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution addresses the open issues of Solution 2.7 by resolving the editor’s notes currently captured in TR 23.700-54 clause 6.2.7.

# 1.	Discussion

Currently the description of Solution #2.7 for KI#2.2 (Simplified ATSSS architecture over non-3GPP access) includes the following Editor's Notes:

EN#1 (clause 6.2.7.1.2): Security aspects related to exposing a new IP communication endpoint to be reachable directly by the UE over non-3GPP access may need further investigation by SA WG3.

Since SA3 will have to study any security aspects related to architectural enhancements agreed by SA2, the proposal is to convert this Editor's Note into a NOTE claiming that security aspects related to exposing the UPF to a new IP communication endpoint are assumed to be investigated by SA3. This NOTE can be considered when concluding the study phase and, potentially, if the solution is selected for normative work, during the normative phase.

EN#2 (clause 6.2.7.1.4): Whether only the MPQUIC functionality or both the MPQUIC and the MPTCP steering functionalities are going to be supported, can be decided based on the conclusion of KI#2.1 (MPQUIC steering functionality to steer, switch and split non-UDP traffic).

The principles of Solution 2.7 apply irrespectively to MPQUIC and to MPTCP. Since it seems that KI#2.1 may conclude with the standardization of MPQUIC functionalities extended to support, among others, IP and IP/TCP traffic, the Editor's Note can be converted into a NOTE stating that, if KI#2.1 concludes with the standardization of such extensions, then Solution 2.7 applies only to MPQUIC. It is also proposed to add a NOTE in the conclusion clause to reinforce this aspect.

EN#3 (clause 6.2.7.1.5): Whether and how there is a need to establish, manage and maintain communication between UE and UPF via NIN3A when the UE is not connected via 3GPPA is FFS.

It is proposed to remove this Editor's Note and to address the issue in the new subclause 6.2.7.2.4. The solution is based on the SMF instructing, during PDU Session Establishment performed via 3GPP access, both the UE and the UPF on how to handle the NIN3A leg of the PDU Session when the UE becomes temporarily unreachable via 3GPP access.

EN#4 (clause 6.2.7.1.5): Security aspects, including aspects related to the exposure of a new IP communication endpoint to be reachable directly by the UE over non-3GPP access, are assumed to be further investigated by SA WG3

This Editor's Note addresses the same issue of Editor's Note #1. Since it is proposed to replace Editor's Note #1 with a NOTE, it is then proposed to remove Editor's Note #4.

EN#5 (clause 6.2.7.1.6): Details of how URSP rules, ATSSS and MAR/N4 rules can be extended are FFS.

More details on how URSP rules and/or ATSSS/N4 rules can be extended are provided in clause 6.2.7.1.6. Because of that it is proposed to remove this Editor's Note.

EN#6 (clause 6.2.7.3): Whether this is indicated only in SM signalling at PDU session establishment or also to AMF is FFS.

It is proposed to clarify that the impact on the UE to indicate its ATSSS-Lite Capability to the AMF applies to Option 1 only, while the signaling towards the SMF applies to both Option 1 and Option 2. In addition, it is proposed to replace the Editor's Note with a NOTE stating the decision of which option to select is to be taken during the evaluation/conclusion phase of the study.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-54:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	IETF RFC 9298: "Proxying UDP in HTTP".

[7]	IETF RFC 9484: "Proxying IP in HTTP".

[8]	IETF draft-ietf-masque-connect-ethernet: "Proxying Ethernet in HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	IETF RFC 9114: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

[10]	IETF draft-ietf-httpbis-connect-tcp: "Template-Driven HTTP CONNECT Proxying for TCP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[11]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[12]	IETF RFC 9000: "QUIC – A UDP based Multiplexed and Secured Protocol".

[13]	IETF RFC 9001: "Using TLS to Secure QUIC".

[14]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[15]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[16]	IETF draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[17]	3GPP TR 23.700-53: "Study on access traffic steering, switching and splitting support in the 5G system architecture; Phase 3".

[18]	IETF RFC 9369: "QUIC Version 2".

[19]	IETF RFC 9220: "Bootstrapping WebSockets with HTTP/3".

[20]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[x]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting (ATSSS); Stage 3".

[y]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

# >>>>NEXT CHANGE<<<<

### 6.2.7	Solution #2.7: Architecture for ATSSS-Lite

#### 6.2.7.1	Description

##### 6.2.7.1.1	Introduction

This solution addresses KI 2.2 (Simplified ATSSS architecture over non-3GPP access) and introduces the architecture for a lightweight version of ATSSS, name ATSSS-Lite. This new architecture is based on the principles described in the following subclauses. The solution does not apply to ATSSS-LL.

##### 6.2.7.1.2	Architecture without TNGF/N3IWF

This solution proposes an architecture in which there is no gateway for access to 5G CN via non-3GPP access, such as TNGF (for trusted non-3GPP access) or N3IWF (for untrusted non-3GPP access). In particular, the UE does not establish any IPSec connection to the 5G CN. Instead, the UE connects to the UPF via the public IP network (Internet) over a new interface named Nx. The existing UPF and ATSSS related traffic handling is extended in such way that a PDU session over the NIN3Aused in ATSSS is logically replaced by a secure connection that is used to transfer UP traffic between UE and UPF. The connection between the UE and the UPF over the Nx interface is secured using TLS. There are no explicit resources reserved in the NIN3A, which just provides IP connectivity to the UE, allowing the UE to connect to the UPF in a secure manner. Traffic handling is done like in ATSSS, but with only difference that the non-3GPP access leg of the MA PDU Session is now replaced with the secure connection over NIN3A.

The ATSSS-Lite architecture, in case of non-roaming and roaming scenario is described in Figures 6.2.7.1.2-1 and 6.2.7.1.2-2, respectively.

Figure 6.2.7.1.2-1: Baseline ATSSS-Lite architecture (non-roaming scenario)

Figure 6.2.7.1.2-2: Baseline ATSSS-Lite architecture (roaming scenario)

It is assumed that the UPF acting as anchor for the ATSSS-Lite connectivity supports the new Nx interface. The UPF processes the traffic according to the traffic handling rules (MAR/N4 rules) provided by the SMF (this aspect is inherited from ATSSS and is described more in detail in clause 6.2.7.1.4).

’	NOTE:	Security aspects related to exposing a public IP address to the UE are to be further investigated by SA3.

##### 6.2.7.1.3	Non-Integrated Non-3GPP Access and re-use of the MA PDU Session concept

The ATSSS-Lite architecture leads to the notion of Non-Integrated Non-3GPP Access (NIN3A) (see clauses 3.1 and 3.2). The solution proposes to reuse the definition of Multiple Access PDU Session described in TS 23.501 [3] clause 3.1 for NIN3A, which reads as follows:

MA PDU Session: A PDU Session that provides a PDU connectivity service, which can use one access network at a time, or simultaneously one 3GPP access network and one non-3GPP access network.

Figure 6.2.7.1.3-1 depicts a simplified diagram of a MA PDU Session applied to one 3GPP access and one Non-Integrated Non-3GPP Access.

Figure 6.2.7.1.3-1: MA PDU Session with Non-Integrated Non-3GPP Access

##### 6.2.7.1.4	Re-use of high layer steering functionality and traffic security

The assumption for ATSSS-Lite is that the steering functionalities in the UE and in the UPF are high layer steering functionalities. They are based on the existing MPTCP and MQUIC steering functionalities that are used in the legacy ATSSS (see clauses 5.32.6.2.1 and 5.32.6.2.2 of TS 23.501 [3]).

’

NOTE 1:	If Key Issue #2.1 concludes with the standardization of extensions of the MPQUIC functionality to support IP and IP/TCP traffic, then Solution 2.7 supports only the MPQUIC functionality.

The traffic exchanged between UE and UPF (in terms of confidentiality and integrity protection) can be protected using the Transport Layer Security protocol (TLS).

NOTE 2:	Further security details can be discussed by SA WG3, if and when needed.

##### 6.2.7.1.5	Protocol stacks connectivity via Non-3GPP

The solution is based on the following assumptions:

-	The NAS signalling between UE and CN is carried only over the 3GPP access. This implies that there is no need to have NAS signalling between UE and CN exchanged over NIN3A when the UE is connected via 3GPPA. Consequently, no CP protocol stack between UE and CN over Nx interface is needed.

’

-	Since, as explained in clause 6.2.7.1.4, there is no need for establishment of an IPSec tunnel between UE and network over NIN3A, the UE UP protocol stack for the non-3GPP access can be simplified. Figure 6.2.7.1.5-1 depicts the user plane protocol stack over Nx interface. The solution does not use primary (AKA based) authentication via non-3GPP access and relies on that the UE has been authenticated via 3GPP access.

’

Figure 6.2.7.1.5-1: UP Protocol Stack for Nx interface

##### 6.2.7.1.6	Re-use of steering modes, MPTCP/MQUIC measurements and extension URSP/ATSSS/N4 rules

###### 6.2.7.1.6.1	Introduction

Since the steering functionality for ATSSS-Lite is based on MPTCP or MPQUIC, the MPTCP/MPQUIC measurements mechanisms can be used instead of the 3GPP defined PMF (Performance Measurement Function) to measure Round Trip Time (RTT) and Packet Loss Rate (PLR). Because of that, at least in principle, the existing ATSSS steering modes (with thresholds, where applicable), namely, Load Balancing, Shortest Delay, Active-Standby, Priority Based and Redundant Steering Mode can be applied also to MA PDU Sessions with NIN3A. Two Options are proposed, namely Option A and Option B, which are described in the subsequent clauses.

###### 6.2.7.1.6.2	Option A – Extension of ATSSS/N4 rules

With Option A, the URSP rules sent by the PCF to the UE would not need to be extended, while the ATSSS and MAR/N4 rules would need to be extended to explicitly include the 3GPPA+NIN3A access combinations for the relevant steering modes. An example of how the ATSSS rules can be extended for the active-standby steering mode is shown in Figure 6.2.7.1.6 below (the new combinations for 3GPPA + NIN3A are marked in bold, italic text). Similar changes can be applied to the Priority based, the redundant (only in case of usage of thresholds) and the Load balancing steering modes. The Steering mode information for the smallest delay steering mode does not need to be extended. The same type of extensions can be applied to the N4 rules for the UPF. With Option A there is no need to extend the URSP rules.

| Steering mode (octet f+3) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The steering mode descriptor field shall be encoded by one octet (octet f+3) as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 1 |  |  |  | Active-standby |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | 0 |  |  |  | Smallest delay |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | 1 |  |  |  | Load balancing |  |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | 0 |  |  |  | Priority based |  |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | 1 |  |  |  | Redundant |  |
| All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steering mode information (octet f+4) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If the steering mode is defined as active-standby, octet f+4 shall be defined as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 |  | 1 |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 |  | 0 |  | 1 |  |  |  | Active 3GPP and no standby |
| 0 | 0 | 0 | 0 | 0 | 0 |  | 1 |  | 0 |  |  |  | Active 3GPP and non-3GPP standby |
| 0 | 0 | 0 | 0 | 0 | 0 |  | 1 |  | 1 |  |  |  | Active non-3GPP and no standby |
| 0 0 0 0 | 0 0 0 0 | 0 0 0 0 | 0 0 0 0 | 0 0 0 0 | 1 1 1 1 |  | 0 0 1 1 |  | 0 1 0 1 |  |  |  | Active non-3GPP and 3GPP standby Active 3GPP and NIN3A standby (new) Active NIN3A and no standby (new) Active NIN3A and 3GPP standby (new) |
| All other values are spare. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 6.2.7.1.6.2-1: Example of extension of Active-Standby steering mode to support 3GPPA+NIN3A MA PDU Sessions (based on modification of Encoding of ATSSS rules in TS 24.193 [x]).

###### 6.2.7.1.6.3	Option B – Extension of URSP rules

Alternatively to Option A, in Option B the URSP rules, currently used in the 5GS by the PCF to indicate to the UE which traffic is subject to ATSSS, can be extended to indicate which traffic is subject to ATSSS-Lite (i.e. which traffic can be transferred using NIN3A). In this way, the ATSSS rules (for the UE) and the MAR/N4 rules (for the UPF) would not to be modified/extended but they would be applied, as they are, to the 3GPPA + NIN3A combinations instead of to the 3GPP + (integrated) N3GPPA combinations. An example of how the URSP rule Route Selection Descriptor could be extended in TS 24.526 [y] clause 5.2 is shown in Figure 6.2.7.1.6-2 (the new code value for 3GPPA + NIN3A is marked in bold, italic text):

Route selection descriptor component type identifier Bits
8 7 6 5 4 3 2 1

0 0 0 0 0 0 0 1		SSC mode type
0 0 0 0 0 0 1 0		S-NSSAI type
0 0 0 0 0 1 0 0		DNN type
0 0 0 0 1 0 0 0		PDU session type type
0 0 0 1 0 0 0 0		Preferred access type type (NOTE 2)
0 0 0 1 0 0 0 1		Multi-access preference type (NOTE 2)

0 0 0 1 0 0 1 0		Multi-access with NIN3A preference type (new)
0 0 1 0 0 0 0 0		Non-seamless non-3GPP offload indication type (NOTE 9)
0 1 0 0 0 0 0 0		Location criteria type
1 0 0 0 0 0 0 0		Time window type
1 0 0 0 0 0 0 1		5G ProSe layer-3 UE-to-network relay offload indication type (NOTE 9)

1 0 0 0 0 0 1 0		PDU session pair ID type (NOTE 5, NOTE 9)

1 0 0 0 0 0 1 1		RSN type (NOTE 5, NOTE 9)

1 0 0 0 0 1 0 0		5G ProSe multi-path preference type
All other values are spare. If received they shall be interpreted as unknown.

Figure 6.2.7.1.6.3-1: Example of extension of the route selection descriptor component to support 3GPPA+NIN3A MA PDU Session (based on modification of current TS 24.526 [y] definitions).

’

#### 6.2.7.2	Procedures

##### 6.2.7.2.0	Introduction

The procedures in this clause describe how the MA PDU Session can be established for ATSSS-Lite and how the appropriate SMF and UPF can be selected according to the indicated UE capability. Figures 6.2.7.2.1-1 and 6.2.7.2.1-2 describe two options of how a MA PDU Session over the Nx interface can be established. Such Figures are based on a simplified version of Figure 4.3.2.2.1-1 of TS 23.502 [4] and on the procedure described in clause 4.22.2.1 of TS 23.502 [4], with the key enhancements highlighted.

In the procedures, SMF selects the UPF based on an ATSSS-Lite indication from the UE, and then the SMF requests the UPF for its Access Info data to be used with this MA PDU session. Access Info data includes communication parameters of how the UPF may be reached, such as IP addresses, port numbers or FQDNs, credentials, security keys and other relevant information depending on the used steering functionality, i.e. MP-QUIC. One Access Info data may be associated with one MA PDU session, and it is steering functionality specific. In addition, the Access Info data is dynamic in nature, and it may have lifetime during which the UPF is reachable for the UE to establish a secure connection via the NIN3A. If the UE has not managed to establish a secure connection within the given timeframe, then the given Access Info data is expired, and the UE needs a new one by requesting PDU Session modification from the SMF (as shown in Figure 6.2.7.3 2). It is the UPF's responsibility to provision and monitor this lifetime.

Clause 6.2.7.2.4 describes how the 5GS handles the situation in which, after a certain MA PDU Session with NIN3A has been established, the UE becomes unreachable via 3GPP access and/or the 3GPP access becomes unavailable for that UE.

##### 6.2.7.2.1	Enhancement to MA PDU Session Establishment, Option 1: ATSSS capable AMF

This option assumes that the AMF supports ATSSS-Lite.

Figure 6.2.7.2.1-1: Simplified ATSSS-Lite MA PDU Session Establishment – Option 1

The main steps of the MA PDU Session Establishment procedure for ATSSS-Lite can be summarized as follows:

1.	When sending the PDU Session Establishment Request to the CN, the UE Includes a request to establish a MA PDU Session Request as well as its ATSSS-Lite capability indication.

2.	Based on such indication and request, the AMF selects an ATSSS-Lite capable SMF (which may, but does not have to, support legacy ATSSS), or if no such SMF is available, rejects the request and notifies the UE.

3.	The AMF forward the indications from the UE to the SMF.

4.	The SMF, based on the ATSSS-Lite capability indication of the UE forwarded by the AMF and with the help of NRF, selects a UPF that is configured to receive/send traffic over the Nx interface. If no such UPF is available, the session cannot be established, and this is notified to the UE.

5.	The SMF establishes SM policy association with the PCF and receives ATSSS rules. The SMF establishes the UP resources in the UPF and instructs it with the related N4 Rules. The SMF instructs the UPF to activate MPTCP or MPQUIC functionality, and requests Access Info data from the UPF and the UPF sets new timer for the Access Info data. The UPF allocates the UE "MPTCP link-specific multipath" or the UE "MPQUIC link-specific multipath" addresses/prefixes.

6.	The SMF establishes the necessary UP resources over the 3GPP access network, indicates the ATSSS rules to the UE. The SMF provides to the UE the Access Info data, including the MPTCP or MPQUIC link-specific multipath addresses/prefixes of the UE and the MPTCP or MPQUIC proxy and the necessary information (e.g. TLS security material including any required certificates for MPTCP/MPQUIC Proxy) for the UE to establish the MPTCP/MQUIC session over NIN3A.

After step 6, the UE is able to establish the communication with the UPF via NIN3A/Nx interface. For MP-QUIC, once the PDU Session is established the UE first establishes an MP-QUIC connection with the UPF and adds the first path of the MP-QUIC connection via the 3GPP access. If the UE also has another IP connectivity via a NIN3A, the UE then creates a second MP-QUIC path to the UPF over the NIN3A using the Access Info data received from the SMF during PDU Session establishment. The QUIC layer takes care of using the existing secure context that was created when the MP-QUIC connection was established over 3GPP access.

As soon as the communication is established, the UE and the UPF can exchange UP data over both accesses according to the ATSSS rules and the MAR/N4 rules received from the SMF.

##### 6.2.7.2.2	Enhancement to MA PDU Session Establishment, Option 2: ATSSS capable SMF supports ATSSS-Lite

This option assumes that an ATSSS capable SMF supports ATSSS-Lite, whereas the AMF is transparent to ATSSS-Lite.

Figure 6.2.7.2.2-2: Simplified ATSSS-Lite MA PDU Session Establishment – Option 2

With the assumptions above, Option 2 is based on the same steps of Option 1, with the key difference that the (legacy) ATSSS capability indication is sent by the UE to enable the AMF to select the SMF, while the SMF selects an ATSSS-Lite capable UPF (i.e. a UPF capable to receive/transmit traffic over the Nx interface) based on the ATSSS-Lite capability indication included in the extended protocol configuration option IE (ePCO) of the PDU Session Establishment message.

##### 6.2.7.2.3	Enhancements to MA PDU Session Modification

Figure 6.2.7.2.3-1: MA PDU session modification for using NIN3A

1.	The UE issues new PDU session modification request (for the MA PDU session supporting ATSSS between 3GPP and NIN3A) to get a new Access Info data.

2.	The SMF requests Access Info data from the PSA UPF of MA PDU Session.

3.	The SMF provides Access Info data in Session Modification Accept to the UE.

4.	Using Access Info data, the UE adds a new path (e.g. to the MP-QUIC connection, QUIC takes care of security etc.).

NOTE:	While Access Info data is valid, the UE may try to repeat step 4, via different NIN3A.

##### 6.2.7.2.4	Handling of MA PDU Session when UE is no longer reachable via 3GPP Access.

As described above, ATSSS-Lite is defined with the assumption that 3GPP NAS signaling can be exchanged only via 3GPP access. This means that no 3GPP NAS signaling is transported over NIN3A and this clause describes how to handle the scenario in which at some point during the lifetime of the MA PDU Session the UE unreachable via 3GPP access (e.g., due to the UE being out of 3GPP access coverage). Figure 6.2.7.2.4-1 below describes the main steps of such procedure (for the sake of simplicity, the AMF has been omitted from the message flow).

The assumption is that the 3GPP access is at least available when the MA PDU Session with NIN3A is to be established (for the case in which the 3GPP access it not available even at that moment in time, see Option 2 of Solution 2.X in clause 6.2.X – NIN3A only PDU session).

Figure 6.2.7.2.4-1: Simplified ATSSS-Lite MA PDU Session Establishment with provision of indication of how to handle NIN3A in case 3GPPA becomes unavailable

1.	The UE initiates the establishment of MA PDU Session with NIN3A as per clauses 6.2.7.2.1 or 6.2.7.2.2.

2.	The SMF may retrieve the PCC rules from the PCF and such rules include indications on how to handle the NIN3A connection of the MA PDU Session in case the 3GPP access becomes unavailable or the UE becomes unreachable over 3GPP access.

3.	As Step 4 of Figure 6.2.7.2.1-1 and of Figure 6.2.7.2.2-1.

4.	The SMF provides the UPF with the instructions on how to handle the NIN3A connection when the 3GPP access becomes unavailable.

5.	Similarly to Step 4 for the UPF, in this Step the SMF instructs the UE on how to handle the NIN3A connection when the 3GPP access becomes unavailable.

The instructions provide to the UPF and UE in Step 4 and Step 5, respectively, may include the following options:

-	release the NIN3A connection as soon as it is known that the UE is unreachable via 3GPPA;

-	release the NIN3A connection after a certain period since it is known that the UE is unreachable via 3GPPA;

-	maintain the NIN3A connection indefinitely/until further notification.

Later, when the SMF becomes aware that the UE is not reachable over 3GPP access (e.g., based on notification from the AMF), it notifies the UPF of this event. The UE may become aware of the unavailability of the 3GPP access based on implementation specific mechanisms. At this point, UPF and UE apply the instructions provided by the SMF in Step 4 and Step 5, respectively, and maintain or release the NIN3A connection (and the related MA PDU Session).

#### 6.2.7.3	Impacts on services, entities and interfaces

NOTE 1:	The decision on whether to support the MPQUIC steering functionality or also the MPTCP one is to be taken during the evaluation/conclusion phase of the study.

UE:

-	Indicates UE ATSSS-Lite Capability to AMF (for Option 1 only) and to SMF (for both Option 1 and Option 2).

NOTE 2:	The decision on which Option to select is to be taken during the evaluation/conclusion phase of the study.

-	Receives Access Info data from the SMF.

-	Creates new MPTCP/MP-QUIC path to the existing MA PDU session using the received Access Info data.

-	Ability to request new Access Info data from the SMF via PDU Session modification procedure.

-	Supports extended URSP rules (Option B) and/or extended ATSSS rules (Option A).

AMF (for Option 1 only):

-	Uses UE ATSSS-Lite Capability to select ATSSS-Lite capable SMF.

SMF:

-	Uses UE ATSSS-Lite Capability provided by the UE to select ATSSS-Lite capable UPF

-	Provisioning of new MPTCP/MPQUIC information between UPF and UE.

-	Support of MA PDU Sessions via non-3GPP access without a SM NAS connection via non-3GPP access. This includes handling of the MA PDU Session in case the UE is not reachable via 3GPP access.

-	Indicates to UE whether ATSSS-Lite MA PDU Session is accepted or not.

-	Receives in PDU Session establishment request, an indication for support of ATSSS-Lite and selects an UPF that supports this feature.

-	Query Access Info data from the UPF and provide it to the UE.

-	Receives request from UE, in PDU Session Modification, to fetch fresh Access Info data from UPF, Query Access Info data from the UPF and provide it to the UE.

-	Generates extended ATSSS/N4 rules (Option A)

PCF:

-	Generates extended URSP rules (Option B)

UPF:

-	Supports Nx interface by exposing new IP communication endpoint(s) (i.e. Access Info data) reachable via NIN3A network for adding a new path for the MPTCP/MP-QUIC connection.

-	Provides Access Info data when requested by SMF.

-	Provisions and monitors of Access Info data lifetime.

-	Supports extended N4 rules (Option A)

# >>>>END OF CHANGES<<<<