SA WG2 Meeting #162	S2-2404120

# April 15thth – April 19th, 2024; Changsha, China

Source: 	Samsung

Title: 	KI#2, New Sol: UPF Discovery and Direct Subscription to UPF for AF

Document for:	Approval

Agenda Item: 	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract of the contribution: This contribution proposes new solution for KI#2.

# 1	Discussion

As per KI#2 description specified in TS 23.700-63 following aspects need to be studied:

-	Identify the specific use case and scenarios that require optimizing the procedures related to UPF data collection with enhancements on UPF direct or indirect subscription;

-	Whether and how the consumer NF can directly or indirectly contact the UPF for its subscription;

-	How to authorize the consumer NF for directly subscribing to UPF event exposure service(s), and how to update/release the subscription;

-	How to support the UPF relocation when some NFs have subscribed the direct or indirect subscription of UPF information;

-	Which Event ID(s) can be subscribed/requested directly or indirectly to the UPF exposure service;

-	Whether there are use cases that require other enhancements on UPF exposure services.

# 2 	Proposal

It is proposed to adopt the following changes to TR 23.700-63.

*** Start of the change***

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| Solution #1: Provisioning of information for header handling |  |  | x |
| Solution #2: UPF provision and selection based on new UPF functionality #2 | x |  |  |
| Solution #3: Selection on UPF with extended user plane capabilities | x |  |  |
| Solution #4: Selection of UPF providing specific user plane functionalities | x |  |  |
| Solution #5: Direct subscription of UPF event exposure service for TSC management |  | x |  |
| Solution #6: UPF selection based on the status of the supported functionalities | x |  |  |
| Solution #7: Translating SUPI/GPSI to NATed IP address |  | x |  |
| Solution #X |  | X |  |

*** Next change (all new text) ***

## 6.X	Solution #X: UPF Discovery and Direct Subscription to UPF for AF

### 6.X.1	Description

When an AF needs to subscribe to UPF event exposure service, the current Rel-18 procedure as described in clause 4.15.3.2.3 of TS 23.502 [2] is as follows.

- AF (who knows the UE’s GPSI) requests Nnef_EventExposure service to NEF

- NEF sends the request Nudm_EventExposure to UDM

- UDM finds the serving SMF corresponding to the request from NEF, and invokes Nsmf_EventExposure to UPF

- SMF then finally sends Nupf_EventExposure Subscribe message to relevant UPF

Furthermore, if the AF is unaware of UE’s GPSI, it needs to query using UE’s IP address, which further requires many additional steps. Also, each of the entities involved in the Event Exposure subscription call flow need to store the context of the relevant subscription, thus optimizations are needed in order to efficiently find the serving UPF for the corresponding target application flow for the AF.

AF can utilize the already established user plane between the UE and the application server (which is controlled by the same authority as the AF) to find the serving UPF for the corresponding session.

AF and operator need to have some service agreement in order for the UPF to detect particular header/protocol in the user plane packets of the UE’s application flow that contain information related to subscribing the serving UPF by AF. AF may provide its AF ID, along with notification address, Event IDs etc. in order to subscribe for particular UPF event exposure service. UPF may check its configured information to see if the AF request is valid and the AF is authorized or not in order to subscribe for UPF event exposure service.

Once the packet with relevant Header/tag is detected for a UE’s application flow, corresponding AF (which had service agreement with the operator) can be notified of the UPF’s address and optionally particular UE’s private address in order for the AF to directly subscribe for the UPF event exposure service via NEF.

### 6.X.2	Procedures

#### 6.X.2.1	General call flow

Figure 6.X.2-1

- UPF/SMF/PCF are pre-configured by the AF to detect certain headers/protocol type in user plane.

NOTE:	The policy/configuration may be stored for Any UE, and may require the use of some aspects of Solution#1. In addition to just detecting the header/tag UPF may also be configured to detect and process the content of the payload of the packet containing the relevant header/tag.

- UE and the application server (same authority of the AF) have an established application flow between them.

- The AF decides to subscribe UPF event exposure service for the particular UE.

- AF (via the application server) sends user plane packet to UE with specific header/tag information

The headers (or the contents in payload) may indicate to UPF information related to AF ID, notification address etc.

- Based on the UPF configuration (in Step 1), UPF detects the specific header related to UPF discovery present in UE’s flow. The contents of the packet may be in a format of request, containing AF ID, notification address etc.

EN: The exact format of header/ information in packet is to be determined by Stage-3.

Case A:	If the configured action of the UPF (configuration done in Step 1) in case the header/tag is detected, is to notify the AF regarding serving UPF ID, Steps 6a, 7a, 8a are performed.

Case B:	Else if configured action of the UPF (configuration done in Step 1) is to implicitly subscribe to AF event exposure, UPF proceeds with Step 6b.

6a.   UPF based on the header information in Step 5, notifies the AF of the UPF ID and/or optionally UE’s private IP address. UPF may utilize the

7a.   AF to NEF: AF after knowing the serving UPF ID, subscribes via NEF for UPF event exposure

8a.   NEF directly subscribes for the UPF event exposure service for the relevant event, without the need for performing serving UPF discovery

6b.	UPF and other NFs behaviour in Step 6b is described in detail in clause 6.X.2.3

#### 6.X.2.2	UPF discovery by utilizing Header information and Subscription via CP

This clause describes the details of case A as mentioned in clause 6.X.2.1.

Suppose in Step 1, UPF is configured to detect header X in UE’s data traffic. UPF action may be further defined to drop the packet/process the packet contents instead of forwarding it to UE

Now the application flow is assumed to be established between UE and Application server. Application server sends a user plane packet with the Header/Tag X. Once the UPF detects that packet, based on it’s configured action, it notifies the UPF ID to the AF.

This notification can be done via control plane as done in Solution#1

- UPF sends the N4 session report to SMF

- SMF notifies the NEF about the header detection including information of the serving UPF.

- NEF notifies the AF the serving UPF information for the AF session.

Second option is to perform this notification by inserting relevant header/tag information in user plane traffic and including information related to UPF in the contents, i.e. the configured action of UPF in Step 1 would be to detect Header/Tag X info and insert Header/Tag Y info.

Once the AF gets the information of the serving UPF for the AF session, it subscribes to UPF event exposure service, by including the relevant UPF ID in the Nnef request, and NEF can directly subscribe to UPF for event exposure. In case AF is internal, AF directly subscribes to UPF (without NEF).

#### 6.X.2.3	UPF Discovery and subscription by utilizing Header information

This clause describes the details of case B as mentioned in clause 6.X.2.1.

For this case, instead of AF being notified about the serving AF, AF is implicitly subscribed to UPF event exposure when the particular header/tag is detected in UE’s application traffic.

AF may include the relevant information (e.g. Event ID, notification target etc.) as the payload contents of the packet, or particular header/tag may implicitly map to particular info (e.g. Header X = 01 maps to Subscribe, Header X = 02 maps to Unsubscribe) based on SLA between AF and operator.

In case the AF request for Event subscription needs to be authorized, UPF may contact the NEF which can perform authorization of AF request.

NOTE: 	AF may have obtained an authorization token beforehand from the NEF and included it in the contents of packet. In case AF is internal this task may be done by NRF as done when a general consumer NF subscribes for event exposure services.

In case the authorization of relevant Event needs to be coordinated by other NFs (e.g. PCF), that NF may just be notified about the header/tag detection and subscribe to the UPF on behalf of the AF.

Example call flow is shown in Figure 6.X.2.3-1

Figure 6.X.2.3-1

### 6.X.3	Impacts on services, entities and interfaces

UPF impacts

- Detection of specific header pertaining to AF request in UE’s user plane packets.

- Notifying the UPF ID/UPF information to the AF either via N6 or via CP

NEF impacts

- Directly subscribing to particular UPF for event exposure based on AF’s request.

*** End of the changes ***