3GPP TSG-WG SA2 Meeting #162	S2-2404166

Changsha, China, April 15th – 19th, 2024	(revision of S2-240xxxx)

Title:	[DRAFT] LS reply on LCS user plane connection binding to the UE

Response to:	LS (S2-2403871/C1-241722) on LCS user plane connection binding to the UE from CT1

Release:	Rel-18

Work Item:	5G_eLCS_Ph3

Source:	[Huawei to be] SA2

To:	CT1

Cc:	CT4

Contact Person:

Name:	Guanglei Li

Tel. Number:

E-mail Address:	liguanglei3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR 0519

1. Overall Description:

SA2 thanks CT1’s LS on LCS user plane connection binding to the UE with following spotted issues:

CT1 has found following issues: - After the TLS connection is established, the UE and the LMF initiate LPP or LCS-SS procedure. For the LMF-initiated case, given that multiple UEs are served by the LMF and multiple TLS connections are active, the LMF needs to determine a TLS connection for the target UE. However, it has not been specified how to link the association between the TLS connection and the UE, e.g., how the LMF knows which TLS connection is for the UE who has received the UPP-CM command message. If the TLS connection can be linked to the ‘wrong UE’ then there could be a risk of location related data from one UE which is sent to another UE. - Once the binding of the TLS connection to the UE is done, if there is a new LCS service request (e.g., MT-LR) for the same UE, the AMF invokes Nlmf_location_determinelocation service operation to the LMF. However, it has not been specified how the LMF associates the LCS service request to the TLS connection of the UE. In this case, the LMF cannot determine which TLS connection can be used for the LCS service request. Therefore, the LMF cannot reuse the TLS connection of this UE for subsequent LCS service request(s).

To solve the above problems, SA2 proposed following principles and agreed the attached corresponding CR:

- When LMF provides the user plane information to the UE via the UPP-CM command message, a UE-level temporary LCS-UP UE ID for LCS-UP connection binding should be assigned by LMF and included in the user plane information sent to UE. When UE performs user plane connection establishment with the LMF using an LCS-UPP message, the temporary LCS-UP UE ID should be carried in the LCS-UPP message so that LMF can bind the TLS connection with the UE.

- When AMF sends LCS service request to the LMF, if UE supports user plane positioning, UE identifier (GPSI, SUPI) should be carried in the LCS service request, so that LMF can associate LCS service request to the TLS connection of the UE in case LMF determines to perform positioning via user plane.

- The LMF should maintain the mapping relationship for the assigned temporary LCS-UP UE ID and UE identifier (GPSI, SUPI) received from AMF (via Nlmf_Location_DetermineLocation request in the case of LMF initiated UP connection establishment procedure, or via Nlmf_Location_UPConfig request in the case of UE initiated UP connection establishment procedure).

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #163		27th May – 31st May, 2024			Jeju, KR

TSG-SA2 Meeting #164		19th – 23rd August, 2024			Maastricht, NL