SA WG2 Meeting #162	S2-2404197

April 15 – 19, 2024, Changsha, China

Source:	OPPO

Title:	FS_UIA_ARC Updating Soln#9 for AMF based User ID authentication

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes some principles for solution evaluation for the FS_UIA_ARC TR 23.700-32.

1	Discussion

Compared to previous version, the updates include:

- Resolve the two leftover ENs

Editor’s note: 	It needs be further clarified regarding the roaming scenario.

Editor’s note: 	It needs be further clarified regarding the case when User Id is changed.

- Update the procedure to introduce UIAF interfacing with AAA-Server for User ID based authentication.

2	Proposal

It is proposed to add the principles in discussion into clause 7 “Overall Evaluation” in TR 23.700-32 V0.2.0.

* * * * Start of Changes (All text new) * * * *

6.9	Solution #9: Solution for AMF based User ID authentication

6.9.1	Key Issue mapping

6.9.2	Description

The solution has the following assumptions:

-  	One SUPI is corresponding to one or more User IDs. The value of User IDs has been negotiated and preconfigured in UE, Core network, and AAA-Server.

-  	It is assumed each node performs the following functions for User-ID based authentication

-	UE: Peer

-	AMF: Authenticator

-	UIAF: it is a new CN NF owned by HPLMN, which is to support interfacing with AAA server for EAP-based User ID authentication. The UIAF can be collocated with other CN NFs such as NSSAAF.

-	AAA-Server: EAP server

-  	Reusing EAP based authentication (e.g. MD5 challenge, Cogent Systems Biometrics Authentication EAP) to authenticate User-Id by 3rd party’s AAA-Server. The User-id and the authentication result can be aware by 5GC

6.9.3	Procedures

6.9.3.1	Initial registration

Figure-6.9.3-1: User ID based authentication after initial registration

(Steps/Parameters for potential impact is in Italic)

- User-A switch on a UE and the UE performs initial registration

- UE send initial registration including SUPI (Concealed as SUCI) and capability of User-Id based authentication

- AUSF triggers the Primary authentication to authenticate the SUPI reported by UE is correct. Specific procedure has been defined in TS 33.501 [9].

  - During the registration, AMF retrieves the SUPI’s subscription which indicates 1) whether an User-ID based authentication is allowed, and whether the authentication is allowed in roaming case.

NOTE 1: 	The other omitted messages for 5G registration follows the existing procedure.

5.  	Registration accept is sent to UE with AMF allocated 5G-GUTI.

6.  	AMF determines to perform User-ID based authentication by sending DL NAS message including EAP-Request (e.g. type=1, identity)

7.  	UE sends UL NAS message including EAP-Response (e.g. type=1, identity=user id)

NOTE 2: 	The step 6 and 7 can be combined into one message.

8. 	AMF invokes Nuiaf_UIAuth_Request to request the User ID based authentication.

9.	UIAF sends AAA Protocol message including GPSI and EAP-Response: identity=user id

10.		More interactions between EAP server and UE may be needed for authentication. E.g., triggers MD5-Challenge to authenticate the user id

11.	 EAP Server responds authentication result of the user id (success/failure) via AAA protocol message

12.	UIAF sends the response of authentication result  to AMF

13.	AMF sends the Authentication result to UE

14.	AMF sends the authenticated User ID to UDM for storing

6.9.3.2	Registration update

When a different human user uses the UE (User ID on the UE is changed), the UE performs registration request repeating the procedure from step-7 to step-14 in Figure-6.9.3-1.

6.9.4	Impacts on services, entities and interfaces

UE:

- Send capability of User-Id authentication in registration request

- Support EAP peer and send EAP message via NAS

AMF:

- Support EAP authenticator

- Support determination of performing User-id authentication, sending EAP message via NAS

UDM:

- SUPI’s Subscription indicates whether User-Id authentication is required

- Store the authenticated User ID

* * * * End of Changes * * * *