SA WG2 Meeting #162	S2-2404307

15-19, April 2024, Changsha, P. R. China

Source:	OPPO

Title:	KI#1, Sol#2: Updates to Resolve ENs

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This contribution proposes to update Solution #2 to resolve ENs.

# Discussion

This contribution proposes to update Solution #2 to resolve ENs.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-32.

* * * * Start of Changes * * * *

## 6.2	Solution #2: Support of User Identification in 5GS

### 6.2.1	Key Issue mapping

This is a solution for KI#1.

### 6.2.2	Description

This solution meets the following architecture assumptions outlined in clause 4.1:

-	The subscription is a 5GS subscription.

-	When the user identifier applies to a human, only a single user identifier is active with a UE subscription at a given time and it is assumed that the specific user identifier is associated with all the UE's traffic during the time that specific user identifier is active with the UE's subscription.

-	A user is considered active if the associated user identifier has been authenticated and authorized to use a linked subscription to access the 5GS.

-	The User Identifier and any subscription that it links to are assumed to be associated with the same PLMN (e.g. the operator that manages the User Identifier and the operator that manages the subscription is assumed to be the same).

-	Application layer interaction between an application client of the UE and application server is assumed not in scope of this study.

The solution includes the following architecture principles:

-	A User Identity Profile, associated with the UE's 5GS subscription, is proposed to be stored in UDR.

-	The logics handling the User Identity Profile could be done in UDM.

-	The User Identity Profile should include User Identifier (UID). The uniqueness of the user identifier is maintained within a specific PLMN and may be linked to any subscription.

-	PDU session procedures may associate User Identifier (UID) to PDU sessions, PCC rules, policies, etc.

As specified in this study's architectural assumption, subscriber/subscription information will not be moved into a User Identity Profile and information from the User Identity Profile should not be used to override information in a subscription.

It is proposed to introduce a new Data Subset "User Identity Profile" in UDR. There may be one or multiple User Identities for each User Identity Profile. Each User Identity maybe associated with the following attributes:

-	User Identifier (UID) and linked 5GS subscription identified by a SUPI

-	User Identity specific service settings and parameters

-	Subscriber categories

-	UID specific network resources (e.g., network slice) in accordance with the subscription(s) the UID belongs to

-	QoS information

User identity refers to information representing a user in a specific context. It is not uncommon for a single identity to be associated with multiple identifiers, however each User Identity contains one UID which is used for authentication and authorization to receive 5GS services.

A User identity profile consolidates information associated with a user’s multiple User Identities. The User Identity Profile Data may be pre-provisioned by operator and stored in UDR.

The UID may be provisioned for a specific subscriber category (e.g., gold, silver, premium, etc.), with each subscriber category enabling the association of operator-defined polices to the UIDs within that category.

The UDM that is capable of handling User Identity Profile is in the HPLMN of the human users it serves and access the information of the UDR located in the same PLMN.

The UDM should include the support of the following new functionality:

-	Handling of User Identity Profile, including storage and management for each User Identity Profile in 5G system

-	The User Identity Profiles associated with a subscription are provisioned by the operator.

-	Operator when updating a User Identity Profile can link or unlink UID with the specific subscription, i.e., SUPI, in an operator-controlled manner, as described in clause 6.2.3.3.

### 6.2.3	Procedures

#### 6.2.3.1	PDU Session Establishment procedure for specific user identifier

Figure 6.2.3.1-1 provides a high-level procedure of PDU session establishment for specific UID to support identifying the human user of a UE's 3GPP subscription when the human user access services via the 5GS using a User Identifier (UID).

Figure 6.2.3.1-1: High-level procedures to support identifying the human user of a UE's 3GPP subscription when the human user access services via the 5GS using a user identifier (UID)

0.	The UE registers with the network with its subscription and establishes a NAS security context with the AMF. The UID associated with the subscription has been authenticated and authorized.

1.	If UID is supported on this UE, when the user is active, during PDU Session Establishment procedure, the UE shall indicate the current UID in N1 SM container.

2.	The AMF selects a SMF.

3.	In steps 3a-3c, if the AMF does not have an association with a SMF for the PDU session ID associated with the UID sent by UE (e.g., when request type is "initial request"), the AMF invokes the Nsmf_PDUSession_CreateSMContextRequest with PDU session ID, UID, and other relevant parameters to create the SM context.

If the AMF already has an association with an SMF for the PDU Session ID associated with the UID provided by the UE (e.g. when Request Type indicates "existing PDU Session"), the AMF invokes the Nsmf_PDUSession_UpdateSMContextRequest with PDU Session ID, UID and other relevant parameters to update the SM context.

The SMF is responsible of checking whether the UE requests are compliant with the User Identity Profile. If User Identity Profile for corresponding subscription is not available, the SMF retrieves the User Identity Profile and UE SM subscription data from UDM/UDR and may subscribe to be notified when the User Identity Profile is modified.

If User Identity Profile for corresponding subscription is not available, the SMF retrieves the User Identity Profile and UE SM subscription data from UDM/UDR, and may subscribe to be notified when the User Identity Profile is modified.

4.	During SM Policy Association Establishment procedure, the SMF may provide relevant parameters about the User Identifier (UID) and QoS information to the PCF to establish UID-related policy association with the PCF and get PDU session policy information and/or PCC rules associated with the UID. If the PCF does not have the UID-related information, it sends a request to the UDR to retrieve the subscription that the UID is linked to get policy control subscription information associated with the UID.

If the Request type is existing PDU session, the SMF may provide UID-related information on the Policy Control Request Trigger condition that have been met by an SMF initiated SM Policy Association Modification procedure. The PCF may provide UID-related policy information to SMF.

If a Subscriber Category identifier has been associated with the UID, the operator-defined policies for this Subscriber Category can be used for this UID in this step.

5.	The SMF continues with PDU Session Establishment Request procedure in clause 4.3.2.2.2 of TS 23.502 [5].

#### 6.2.3.2	User State

One of the requirements identified in TR 22.904 [8] for managing UID within the 5GS is the capability to determine the user’s activity status, including transitions between active, inactive, and suspended states. Furthermore, reactivating a suspended UID on a UE where it was suspended could involve simplified authentication on the device itself. To support these requirements, some procedures to determine user states are proposed in this clause.

To support the User State scenarios, UDR may include a new Data Subset "User State Data". Associated with a specific subscription and UID, the user state information is stored by the UDR with Data Key SUPI and/or UID for one or multiple UIDs in the specific SUPI. The "User State Data" may be updated by NEF.

A User Identifier (UID) associated with a specific subscription can be in different states, such as activated, deactivated, or suspended. For example, a UE may detect a change in the UID state if a user has been inactive on the UE for a predetermined period, or the user may request to temporarily suspend their UID. The UE may then request the network to determine and update user state. The inactivity period is defined by the home operator's policy. Reactivating a suspended UID on a UE where it was suspended could involve simplified authentication on the device itself.

During Session Management procedures, the SMF may store and update user's state information in local configurations according to the home operator's policy. Changes in the user states are communicated to subscribed NFs via the Nsmf_EventExposure_Subscribe function. Additionally, the AMF may maintain and update the user state information in local configurations as reported by the UE.

In Figure 6.2.3.2-1, a high-level procedure to describe user state update is provided.

Figure 6.2.3.2-1: User State Change

1.	UE detects user activity with UID and identifies state change. A UID maybe suspended upon human user’s request from the UE.

NOTE:	How UE detects user activity and state change are implementation specific, not in the scope of 3GPP.

2.	UE requests to update user state in User Identity Profile, by sending a NAS message containing the UID.

3.	In steps 3a-3c, AMF uses UE's report and its local UE context data to determine UID state change. If user state change occurs, AMF sends the event report, by means of Namf_EventExposure_Notify message, to the associated notification endpoint of the NEF, along with the time stamp. NEF may store the information in the UDR along with the time stamp using either Nudr_DM_Create or Nudr_DM_Update service operation as appropriate.

4.	In steps 4a-4c, during Session Management procedures, the SMF may monitor user activities, and determine UID state change occurs, e.g. , when the PDU Session associated with a specific UID has no data transfer after a predetermined period of inactivity, in accordance with the home operator’s policy.

SMF sends the user state change event report, by means of Nsmf_EventExposure_Notify message, to the associated notification endpoint of the NEF, along with the time stamp. NEF may store the information in the UDR along with the timestamp using either Nudr_DM_Create or Nudr_DM_Update service operations as appropriate.

The User State information could be taken into account in the authentication/authorization procedure, e.g. when reactivating a suspended UID on a UE where it was suspended, a simplified authentication procedure on the device itself could be performed.

#### 6.2.3.3	Linking and Unlinking

Figure 6.2.3.3-1: Registration to begin Association of a User to a Subscription

1.	The AF sends a Link Request to the NEF. The request includes a User Identifier, a subscription identifier (e.g. a SUPI or External ID), and an indication of whether the request is to "link" or "unlink".

2.	The NEF invokes the Nudm_UIP_Modify service operation to send the "link" or "unlink" request to the UDM. If the UDM authorizes the request to "link", then the UDM forms the user identifier link by updating the User Profile to include the SUPI. If the NEF authorizes the AF request to "unlink", based on NEF request, the UDM makes decision to delete the user identifier link by updating the User Identity Profile to remove the SUPI. The UDM responds to the service invocation by indicating if the "link" or "unlink" request was successful.

3.	The NEF responds to the AF with an indication of whether the "link" or "unlink" request was successful.

### 6.2.4	Impacts on services, entities and interfaces

UE:

-	Detect UID state change.

-	Handle UID during PDU session procedures.

SMF:

-	Handle UID during PDU session procedures.

-	Determine UID state.

AMF:

-	Determine UID state based on UE report and UE context.

PCF:

-	Handle UID specific rules and policies.

UDM/UDR:

-	Handling of User Identity Profile, including storage and management for each subscriber's User Identity Profile in 5G system.

-	Linking/Unlinking and updating User Identity Profile within the specific subscription.

* * * * End of Changes * * * *