SA WG2 Meeting #162	S2-2404545

# Changsha China, April 15th–April 19th, 2024          	(revision of S2-240xxxx)

Source: 	Nokia

Title: 	KI#3: Updates to Solution #16

Document for: 	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This pCR updates solution #16.

# 1	Discussion

This document resolves the following EN.

Editor's note: 	It is FFS, if and what needs to be further studied with respect to transferring of data to NWDAF/ADRF.

NWDAF shall register for Event Notification with the UDM for learning the Authentication and Authorization results for the needed User Identity (or any User Identity). This solution could be extended to allow the NWDAF to learn about the Authentication and Authorization results for a UE too.

# 2 	Proposal

It is proposed to update TR 23.700-32 as follows.

# * * * * First change * * * *

#### 6.16.3.2	Exposure of Authentication and Authorization historical Results

As per the above parameters detailed in 6.16.2, this section describes further on how an AF requests and receives information related to Authentication and Authorization details.

The AF uses the Nnef UIA Service Get of the NEF, and the NEF in turn invokes Nnwdaf_DataManagementService Fetch towards NWDAF.

The query parameter may consist of

- Query Type = Authorization_Res/Authentication_Res/Auth_Authentication_Both

- User Identifier or UE identifier

- Optional parameter as defined in section 6.16.2

The query parameter then relates towards NWDAF with a corresponding data specification pointing to UIA Service.

The data in this case, is based on the historical Authentication and Authorization data.

Accordingly, the NWDAF shall construct a response container, based on the results available and based on the operator policies on the number of reports.

The framed container takes the route of Nnwdaf_DataManagement Service Response and then further by Nnef UIA Service Get Response, accordingly, based on the request.

A prior, the Authentication / Authorization (registration) results are stored in the DM of the NWDAF/ADRF. Corresponding results could be organized and stored by the NWDAF/ADRF in its DM, and the same is fetched from UDM by the NWDAF by using the Nudm_EventExposure Service.

Figure 6.16.3.2-1

# * * * * End of Changes * * * *