SA WG2 Meeting #162	S2-2404552

Changsha China, April 15th–April 19th, 2024

# Title:	[draft] Reply LS on clarification related to the RAT type only change impact to AM/UE Policy association.

# Response to:	LS (C3-241578) Clarification related to the RAT type only change impact to AM/UE Policy association.

# Release:	Rel-18

# Work Item:	TEI18

Source:                  [Nokia, Will be] SA2

To:                          CT3

Cc:                          CT1

Contact Person:

Srinivas Garikipati

Srinivas.Garikipati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

CR: 1290

1. Overall Description:

SA2 thanks CT3 for their LS on Clarification related to the RAT type only change impact to AM/UE Policy association.

SA2 provides the following updates to CT3.

CT1 Query Details:

Question 1: As per above requirements, there are multiple RAT types defined within one Access type, and there is possibility of UE mobility between RAT types (Inter RAT) within same Access. In which case, do we need the PCF to distinguish between different RAT types to provide AM/UE policies specific to RAT type?

SA2 Answer:

SA2 considers the following:

- We do not see any specific use case for UE policies to be changed when there is change in RAT Type.

- Referring to 23.501 “5.3.2.3 Registration Area management” and “5.31.11 Inter-RAT idle mode mobility to and from NB-IoT”, the PCF may implement different RFSP using the AM policies, when there is change of RAT. Change of RFSP based on RAT type is possible and is left to the deployments if they are required.

In view of the above, a PCRT based on RAT Type is recommended for the AM policies.

2. Actions:

To CT3 group.

ACTION:

SA2 kindly asks CT3 to take SA2 responses and attached CRs into account and accordingly drive the Stage 3 specifications.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | CITY |
| --- | --- | --- | --- |
| SA2#163 | May 27th – May 31st, 2024 | Korea | Jeju |
| SA2#164 | Aug 19th – Aug 23rd, 2024 | NL | Maastricht |

SA2 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-S2.htm