3GPP TSG-SA WG2 Meeting #S2-162	S2-2404639

Changsha, China, April 15  –  19, 2024                           (was  S2-2402930)

Source: 	Samsung

Title: 	Update for Solution #7: Enhancing alternative QoS profile with UL and/or DL PDU set QoS parameters

Document for: 	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This contribution proposes to update the solution #7 to cover handover scenarios in alternative QoS profile with PDU Set QoS parameters.

- Introduction

Solution #7 was agreed but the handover aspects are not described and captured as FFS.

That is, considering non-homogenous NG-RAN deployment scenario, even if the serving NG-RAN supports PDU Set QoS parameters, another NG-RAN UE moves to may not support PDU Set QoS parameters.

In that sense, when alternative QoS profile with PDU Set QoS parameters is used, if the UE moves to NG-RAN not supporting PDU Set QoS parameters, the alternative QoS profile can not be used anymore. So in that case the alternative QoS profile should fall back to alternative QoS profile without PDU Set QoS parameters.

Additionally, due to the procedures the situation is not clear whether PDU Session establishment or PDU Session modification scenario step 4 is a bit modified for more clarification.

2 Proposal

It is proposed to add the following contents to TR 23.700-70.

# * * * * First change * * * *

6.7	Solution #7: Enhancing alternative QoS profile with UL and/or DL PDU set QoS parameters

6.7.1	Key Issue mapping

This solution targets KI#1, which is listed below:

This key issue will study PDU set based QoS handling enhancements considering both control plane and user plane perspectives. In particular, this KI will address:

- whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced.

NOTE: This will require close coordination with SA4 and RAN WGs

6.7.2	Description

The present solution is based on the following principles:

The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled from SMF to the NG-RAN. An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. For delay-critical GBR QoS flows, an Alternative QoS Profile may also include MDBV. When the NG-RAN sends a notification to the SMF that the current QoS profile is not fulfilled (GFBR can no longer be guaranteed), the NG-RAN shall, if the currently fulfilled values match an Alternative QoS Profile, notify the SMF about the reference to the Alternative QoS Profile to indicate the QoS that the NG-RAN currently fulfils.

Same for the PDU set QoS parameters in the QoS profile, i.e., PSDB, PSER and PSIHI, the NG-RAN should also be provided with a list of alternative UL and/or DL PDU set QoS profiles in the prioritized order, which is the combination of UL and/or DL PSDB, PSER and PSIHI, and can be enabled with notification control. When the current PDU set QoS parameters cannot be fulfilled, the NG-RAN can check the received alternative PDU set QoS profiles for UL and/or DL respectively. If a match is found, the NG-RAN can also provide the reference to the alternative UL and/or DL PDU set QoS profile to the SMF to adjust the traffic accordingly, along with the indication that the current PDU set QoS parameters cannot be fulfilled.

There can be two possible ways to make enhancements to the legacy alternative QoS profiles regarding the PDU set QoS parameters:

Option 1:  Separate UL, DL alternative PDU set QoS profile, and legacy alternative QoS profile.

For this option, two more alternative PDU set QoS profiles can be created, and each is the combination of PSDB, PSER and PSIHI for the DL and UL of the QoS flow respectively. When notifying about the preferred alternative PDU set QoS profile, the reference to each preferred profile will be sent from the NG-RAN to the SMF if a match can be found.

Editor’s note: Whether the alternative PDU set QoS profiles and legacy alternative QoS profiles can share the same priority order is FFS.

Option 2: Add PDU set QoS parameters into the alternative QoS profile to get the upgraded alternative QoS profile.

For this option, the PDU set QoS parameters are added into the legacy alternative QoS profile and the Upgraded Alternative QoS profile can be obtained, which is the combination of: PER, PDB, GFBR (UL/DL), averaging window, the maximum data burst volume (MDBV), PSDB, PSER and PSIHI for UL and DL, respectively.

When notifying about the preferred profile, the reference to the upgraded alternative QoS profile will be provided.

NOTE: The upgraded Alternative QoS profile mentioned here still refers to the legacy alternative QoS profile that includes the PDU set QoS parameters.

6.7.3	Procedures

The Figure 6.7.3-1 shows the procedure for provisioning the NG-RAN with alternative UL and/or DL PDU set QoS profile and enable the notification control for it.

Figure 6.7.3-1: Procedures for supporting alternative UL and/or DL PDU set QoS profile with notification control enabled

The steps of Figure 6.7.3-1 are described as follows:

Step 1: An AF requests to establish an AF session with QoS by invoking the Nnef_AFSessionWithQoS Create service operation as described in clause 4.15.6.6 of 3GPP TS 23.502. Apart from the flow description, QoS parameters, PDU set QoS parameters, the AF additionally includes the alternative UL PDU set QoS parameters sets and alternative DL PDU set QoS parameters sets in a prioritized order.

Step 2: The NEF authorizes the AF request that contains a single UE address and may apply policies to control the overall amount of QoS authorized for the AF.

Step 3a: The NEF forwards the received parameters to the PCF in the Npcf_PolicyAuthorization_Create request, including the alternative UL PDU set QoS parameters sets and/or alternative DL PDU set QoS parameters sets in a prioritized order. If the AF is considered to be trusted by the operator, the AF uses the Npcf_PolicyAuthorization_Create request message to interact directly with PCF to request reserving resources for an AF session. Alternatively, PCF can obtain the information based on local configuration in Step 3b.

Step 4: The PCF creates PCC rules that includes the PDU set QoS Notification Control for UL and/or DL respectively,  the alternative UL and/or DL PDU set QoS parameters sets in a prioritized order, and sends it to the SMF if there is a corresponding PDU Session.

NOTE: In case of PDU Session establishment procedure, the SMF receiving the PDU Session establishment request from UE fetchs SM policy from the PCF, which may include the above the QNC related information.

Step 5: Based on the PCC rules received from the PCF, for the corresponding QFI SMF generates the corresponding QoS profile that includes the QoS parameters and PDU set QoS parameters, the notification control parameters, the PDU set QoS notification parameters for UL and/or DL respectively, the upgraded alternative QoS profiles including the UL and/or DL PDU set QoS parameters (or the and alternative UL and/or DL PDU set QoS profiles) in the prioritized order. SMF sends the information to the NG-RAN via AMF.

NOTE: In step 5, the upgraded alternative QoS profiles including the UL and/or DL PDU set QoS parameters refers to Option 2, and the alternative UL and/or DL PDU set QoS profiles refers to Option 1. The corresponding reference to the preferred alternative UL and/or DL PDU set QoS profile, or the reference to the preferred upgraded alternative QoS profile, should be used when sending notification in the following step.

Step 6: If NG-RAN supports the PDU set based handling and determines that the UL and/or DL PDU set QoS parameters cannot be fulfilled, then the NG-RAN may check the alternative UL and/or DL PDU set QoS profiles list to find if there is any match, in the prioritized order. If there is a match, the NG-RAN shall indicate the SMF about the reference to the matching Alternative UL and/or DL PDU set QoS Profile, or the reference to the upgraded alternative QoS profile, with the highest priority.

Moreover, for option 1, an optional idication can be sent from the NG-RAN to SMF indicating that the current UL and/or DL PDU set QoS parameters cannot be fulfilled, together with the reference to the alternative UL and/or DL PDU set QoS profile.

Step 7: The SMF notify the PCF and AF about the current fulfilled situation at the NG-RAN for the QoS flow, together with the preferred reference of the alternative UL and/or DL PDU set QoS profiles, or the reference to the upgraded alternative QoS profile, if applicable.

When such alternative UL and/or DL PDU set QoS profile is provided in the NG-RAN, due to the UE mobility the serving NG-RAN is changed to target NG-RAN, but if the target NG-RAN has no capability of PDU Set handling, the alternative QoS profile is applied in place of UL and/or DL PDU set QoS profile.

6.7.4	Impacts on services, entities, and interfaces

AF:

-	Provision the alternative UL and/or DL PDU set QoS Parameters sets in a prioritized order.

PCF:

-	The PCF may enable PDU set QoS Notification Control for UL and DL respectively, and include the Alternative UL/DL PDU set QoS parameter sets in the PCC rule sent to the SMF.

SMF:

-	The SMF sends a prioritized list of the upgraded alternative QoS profiles including the UL and/or DL PDU set QoS parameters, or the Alternative UL and/or DL PDU set QoS Profile(s) to the NG-RAN, together with the notification control parameters.

NG-RAN:

-	The NG-RAN sends a notification to the SMF when the QoS profile cannot be fulfilled, an optional indication that the current UL and/or PDU set QoS parameters cannot be fulfilled, together with the reference to the preferred alternative UL and/or DL PDU set QoS profile, or the reference to the upgraded alternative QoS profile, if applicable.

# * * * * End of changes * * * *