3GPP TSG-SA WG2 Meeting #161	S2-2404683

Changsha, China April 15– April 19, 2024								rev of S2-2402887

Source:	NEC

Title:	KI #1, Addressing ENs of solution 3.

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: The contribution addresses the ENs of solution 3.

1. Introduction

This contribution addresses the following ENs.

2. Discussion

Editor's note:	It is FFS how to manage the heart beat management procedure of SCTP protocol in steps 4 and 5.

Editor's note:	The detailed step of resume procedure is FFS.

When a gNB determines based on that the  Ephemeris information that the satellite is moving away from a particular location then it disable the heartbeat of the SCPT connection with the AMF as described in the RFC and notify it to the NGAP layer. Similarly, the AMF based on the feederlink availability time disasble the heartbeat of the SCTP connection the gNB.

When the gNB determines that the moving to a location then the gNB enable the heartbeat of the SCTP connection with the AMF and also the AMF enable the heartbeat of the SCTP connection with the gNB based on the feederlink available time provided by the gNB during the NG SETUP procedure. The gNB and AMF based on the indication from respective underlying SCTP layer that the path is up then gNB and AMF resumes the NGAP procedure

Editor's note:	It is FFS how to perform the UE context management.

This solution proposes to release the UE connection and context but suspend and resume the path between the gNB and the AMF and maintain the gNB context in the AMF and AMF context in the gNB.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-29.

* * * Start of change * * * *

## 6.3	Solution #3: Support for suspend and resume of NG interface

### 6.3.1	Key Issue mapping

This solution addresses the scenarios mentioned in the KI#1.

### 6.3.2	Functional Description

This solution follows following basic principles and assumptions:

The solution assumes that Earth-fixed Tracking Areas are used.

Figure 6.3.2-1

1.	The gNB is in the payload of the satellite.

2.	When the feeder link becomes available, the gNB sends NG SETUP REQUEST message including feeder link available time i.e. gNB available time to the AMF.

3.	The AMF stores the feeder link available time for this gNB. The gNB and the AMF communicate the NGAP messages over the NG interface during the feeder link available time.

4.	When the gNB is moving away from the location, then gNB disable the heartbeat of the path with the AMF (refer RFC9260). When the AMF based on the feederlink available time determines that satellite is moving away the AMF disable the heartbeat of the path with the gNB. After the heartbeat is disabled, the gNB and the AMF suspends the NG connection i.e the gNB and the AMF do not communicate any NGAP message at this interface. The gNB deletes releases the RRC connection of the UE which are in connected mode and delete the UE contexts.

5.	When the feederlink availability time indicates that the feeder link is available both the gNB and the AMF re-enable the heartbeat of the respective path. Upon indication from the respecrtive layers that path between the gNB and the AMF are up, the gNB and the AMF resume the NG connection implicitly.

The same principle is applied to the EPS at the S1 interface between eNB and MME.

NOTE:	Adverse weather condition might impact the feeder link establishment procedure.

* * * End of change * * * *