3GPP TSG-WG SA2 Meeting #162	S2-2404785

Changsha, China, April 15th – 19th, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#8, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_ph2 / Rel-19

Abstract: This paper proposes to add the conclusion section for KI#8 in FS_NG_RTC_Ph2 TR.

# 1. Discussion

The evaluation and conclusion for KI#8 are proposed.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-77.

# * * * * First change * * * *

## 7.x Evaluation Related to Key Issue #8

Six solutions were proposed for KI#8: #17, #23, #24, #25, #26, #27.

Solution #17 mainly proposes the following:

-	Based on data channel architecture, a new NF called DAC is added to store Avatar representation, DAC is connected to XR AS. The XR AS download the user’s avatar representation using Avatar-id from DAC.

-	Three high level call flows are described which include: UE-A centric, network centric and UE-B centric. The decision of which call flow to be used is negotiated between UE-A, network and UE-B.

-	In the call flows, the XR AS downloads Avatar representation and sends to UE-A/UE-B/MF, UE-A/UE-B/MF performs media rendering based on the metadata generated locally or received from UE.

Solution #23 mainly proposes the following:

-	Based on data channel architecture, a new NF called Avatar Repository is added to store Avatar representation, the Avatar Repository is connected to DCSF.

-	Introduces high level call flow of downloading avatar representation using BDC.

Solution #24 mainly proposes the following:

-	Based on data channel architecture, a new NF called Avatar Repository is added to store Avatar representation, the Avatar Repository is connected to DCSF.

-	Introduces the transition between audio/video and avatar communication, and its signalling and media processing procedure are almost the same as solution #17.

Solution #25 mainly proposes the following:

-	Based on data channel architecture, a new NF called BAR is added to store Avatar representation, the BAR is connected to XR AS.

-	A new reference point MC1 is added, which is used for instruction of service related media processing.

-	Network-based avatar call flow is introduced, the XR AS instruct MF/MRF through DC4 and MC1 to perform avatar call related media processing, the MF/MRF downloads Avatar representation from BAR and performs media processing based on the metadata received.

Solution #26 mainly proposes the following:

-	Based on data channel architecture, a new NF called DAR is added to store Avatar representation, the DAR is connected to MF and UE.

-	Two high level call flows used in AR call are described which include: network centric and UE-B centric. In the UE-B centric mode, UE-B extract UE-A’s Avatar-id from the scene description. The other signalling and media processing procedure are almost the same as solution #17.

Solution #27 mainly proposes the following:

-	Based on data channel architecture, a new NF called DAC is added to store Avatar representation, the DAC is accessed by NEF.

-	This solution has defined the sending and receiving Avatar capability of the UE, this capability is negotiated between UE-A and UE-B during the avatar session establishment. Two different call flows are described according to the negotiation result, which include:

-	No transcoding on MF/MRF if UE-B support this Avatar capability, UE-A and UE-B retrieval Avatar representation and animate the Avatar representation locally.

-	Transcoding on MF/MRF when UE-B doesn’t support this Avatar capability, MF/MRF retrieval Avatar representation and animate the Avatar representation.

In the following table solutions are classified based on following categories:

-	The NF connect to DAC;

-	How the UE downloads the avatar model;

-	How the IMS network downloads the avatar model;

-	The NF controls the avatar service logic;

-	The role of animation and rendering the avatar;

|  | The NF connect to DAC | How the UE downloads the avatar model | How the IMS network downloads the avatar model | The NF controls the avatar service logic | The role of rendering the avatar |
| --- | --- | --- | --- | --- | --- |
| Solution #17: Solution for support of IMS avatar communication using data channel | XR AS | ADC | XR AS | XR AS | UE-A or MF/MRF or UE-B |
| Solution #23: Download avatar representation in bootstrap data channel from avatar repository | DCSF | BDC | N/A | N/A | N/A |
| Solution #24: Transition Between Avatar and Audio/Video Communication | DCSF | ADC | DCSF | DCSF | MF/MRF or UE-B |
| Solution #25: Supporting network-based avatar communication by media capability invocation | XR AS | ADC | MF/MRF | XR AS | MF/MRF |
| Solution #26: Support for Avatars in AR Calls | MF | unclear | unclear | XR AS | MF/MRF or UE-B |
| Solution #27: Early capability negotiation for IMS Avatar Communication | NEF | NEF | DCSF | DCSF | UE-A or MF/MRF or UE-B |

# * * * * Second change * * * *

## 8.x Conclusions Related to Key Issue #8

The following conclusions are agreed for supporting IMS Avatar communication:

-	Avatar communication is implemented based on the data channel architecture.

-	A new NF DAR is added to store Avatar representation and related data of users who subscribed this service. The DAR can be a device of the operator or a third-party device. DAR related interfaces are out of 3GPP scope.

-	The Avatar representation stored in the DAR should be accessed only for authorized UE.

-	The Avatar representation and metadata (such as action and facial expression data) are transmitted between the UE and the IMS network using data channel.

-	The Avatar representation can be animated and rendered by audio/video or metadata. This animation and rendering can be independently completed on UE, or collaboratively completed on the UE and IMS network through negotiation.

-	The metadata media format and the negotiation procedure for Avatar representation animation and rendering should be further defined in SA4.

-	When network centric mode is used, the XR AS instructs Avatar communication service control instruction to the MF/MRF, the MF/MRF needs to support media processing, e.g. ASR/TTS/SLR/SSL, and Avatar animation and rendering.

# * * * * End of changes * * * *