3GPP TSG-SA WG2 Meeting #161	S2-2402888

Changsha, China, 15TH – 19th April, 2024

Source:	NEC

Title:	KI #1: Update to Architecture Assumption

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto/ Rel-19

Abstract of the contribution: The contribution addresses Key Issue #1: Support of UE moves between CAG cell of 5G Femto and CSG cell.

1. Introduction

This contribution aim to provide solution to the key issue#1.

### 5.1.1	General description

This key issue investigates any needed enhancements to support the UE moving between CAG cell of 5G Femto and CSG cell.

The KI should cover:

-	The mobility scenarios to be studied for the UE move (e.g. CSG cell to CAG cell of 5G Femto).

-	Whether and how control signalling procedures are enhanced to support the mobility scenarios.

NOTE 1:	This Key Issue assumes that the existing CAG concept defined for PNI-NPN is re-used for access control when a UE wants to access a 5G Femto cell without impacts to PNI-NPN.

NOTE 2:	This Key Issue expects that solutions avoid impacts on EPC (e.g., MME), E-UTRAN and NG-RAN.

2. Discussion

Assumption and facts:

- It is assumed that the existing CAG concep will be used to support the CSG feature in 5G. The CAG identity will be used to identify the 5G CG Cells.

- An enterprise can deploy a CSG cell in LTE and EPS as well. In this case the same CSG ID is broadcasted in the CSG cell and CAG Cell.

- While in 5G, the UE allowed CAG list will contain both allowed CSG list and allowed CAG list. This will be sent to the NG RAN and other network node.

- The length of the CAG ID in 5GS is 32bits and the length of the CSG in Femto cell is 28bit.CAG.

Considering the above assumptiions and facts:

Some enterrprise can have both 5GS Femto and EPS Femto. 5GS Femto Cell is implemented as CAG Cell and EPS Femto will be implemented as CSG cell. A enteprise want to have one identity for both 5GS and EPS Femto. This can be achieved by reserving first 5bits of CAG ID for a 5G Femto. e.g. reserving first 5bits to 00000 for 5G Femto. so a CSG ID can be used in both 5GS and EPS.

3. Conclusion

It is proposed to add following solution in the TR.

* * * Start of change * * * *

## 4.1	Architectural Assumptions

The architecture for support of 5G NR Femto shall be based on the following architectural assumptions:

-	the 5GS defined as part of Rel-18 is used as basis for further potential enhancements;

-	the need for potential architecture enhancements for supporting 5G NR Femto deployments depends on the outcome of RAN WG3 study;

-	It is expected that MME, E-UTRAN and NG-RAN impacts are avoided;

-	It is assumed that the existing CAG concept defined for PNI-NPN is re-used for Femto access control without impacts to PNI-NPN.

NOTE: A particular value can be assigned to first 5bits of CAG ID for 5G Femto cell e.g. first 5bits of CAG ID can be assigned to 00000 for femto cell.

-	It is assumed that a UE will treat a EPS CGS cell in same way both in Idle mode or connected mode i.e.either CSG cell or open HeNB

* * * End of change * * * *