/SA WG2 Meeting #162	S2-2405146

# April 15 – April 19, 2024, Changsha, China	(revision of S2-2404245)

Source: 	vivo

Title: 	Interim evaluation and conclusion for KI3

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution tries to provide the Interim evaluation and conclusion for KI3.

- Introduction

This paper is trying to evaluate the solutions captured for KI#3(5GS enhancements for network energy saving and efficiency), provide the conclusions for the KI#3 based on the captured solutions.

2	Proposal

It is proposed to capture the following solution in TR 23.700-66.

Start of Changes

Next change

8.X	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

Following interim conclusions for KI#3 are proposed for normative work:

Editor's note:	The conclusions need to be further revised by considering additional solutions.

- For Enhancements on NF (re)selection based on energy related information, following principles can be considered in normative work:

-	NF profile is extended to include the energy related information and maybe updated to reflect energy related states.

-	NF discovery is enhanced to support energy-related NF discovery query parameters

-	NF selection is enhanced to consider the new energy related information from the NF profiles.

Editor's note:	Additional energy related information elements included in the NF profile is FFS.

- Enhancements on existing operations and procedures for energy saving and energy efficiency, following principles can be considered in normative work:

-	Access management and session management enhancement e.g., AM policy and SM policy generation by PCF taking the energy related information/Analytics into account.

NOTE:	Policy control enhancement is relative to conclusion of KI#2.

-  The UP path of PDU sessions e.g., DNAI, UPF (re-)selection maybe adjusted based on energy related information.

-  For Network slice which is subject to energy control, e.g.,

determination of accept/reject of requested S-NSSAIs or PDU session, and modification/release of PDU session may consider energy related information.

- Maximum number of UEs and maximum number of PDU sessions for a network slice maybe adjusted by considering energy related information when it is subject to NSAC

Editor's note:	Whether all the NSAC architecture options are applicable to this feature is FFS.

NOTE:	The energy related decision of bullet 1 and bullet 2 is based on operator’s policy.

- For NWDAF-Based Energy Analytics for network energy saving and efficiency, following principles can be considered in normative work:

-	NWDAF is taken as the role of energy related analysis/prediction/statistics, considering the energy consumption for generating the analytics and corresponding data collection.

Editor's note:	The input/output data for energy related analytics will be concluded during study phase.

End of Changes