3GPP TSG-WG SA2 Meeting #162	S2-2405258

# 15 – 19 April 2024, Changsha, CN

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4794 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 4.3.2.2.3.3 and 4.17.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedures in clause 4.3.2.2.3.3 have the following issues: - unclear how vNSSF finds hNSSF - does not fully take into account what has been specified in 29.510 for hiearchies of NRFs. - - Discovery provides NF profiles - "selection" used instead of "discovery" - The address can be on service level. Thus better to use NF profile. - inconsistent language - unclear what local NRF means. NRFs may verywell be located very central, and thus "local" has no real meaning. It is really the NF profiles they hold that is important |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates according to reason for change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not specified how vNSSF finds hNSSF, Stage 3 and stage 2 missalignment with regards to NRF hierarchies and by that not fully specified procedure. This may lead to interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2.3.3, 4.17.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Since very few if any 5GC roaming is deployed, it is probably enough to only correct Rel-18. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3.2.2.3.3	Home routed roaming

The discovery and selection of the SMF in VPLMN is performed in the same way as for non-roaming and roaming with local breakout (see clause 4.3.2.2.3.2). The discovery and selection of the H-SMF in HPLMN is performed by means of either using NSSFs in VPLMN and HPLMN to discover a NRF in HPLMN (hNRF) or by having the hNRF configured in VPLMN. Which of these two options to use is based local configuration in the VPLMN. The configuration depends on Service Level Agreements between the operators.

NOTE 1:	The procedures described in this clause are not limited to SMF discovery and selection, but can be used to discover and select any NF/NF service in the HPLMN part of a Network Slice instance.

The discovery and selection of the H-SMF in HPLMN is performed by means of the procedure depicted in Figure 4.3.2.2.3.3-1 for the option where  NSSFs are used for hNRF discovery. In this case the steps 1 to 4 in Figure 4.3.2.2.3.3-1 are required. In the option where VPLMN are having hNRF configured, then either AMF or NRF in VPLMN (vNRF) has the endpoints of the Nnrf_NFdiscovery service(s) of the hNRF(s) as described in Figure 4.3.2.2.3.3-2.

Figure 4.3.2.2.3.3-1: Option 1 for SMF selection for home-routed roaming scenarios

1.	Based on the operator's configuration, if the AMF is not aware of the appropriate NRF to be used to discover NFs/NF services in the HPLMN, the AMF invokes the Nnssf_NSSelection_Get service operation from the NSSF in VPLMN (vNSSF) with the VPLMN S-NSSAI from the Allowed NSSAI or Partially Allowed NSSAI requested by the UE for this PDU Session, the HPLMN S-NSSAI that maps to the VPLMN S-NSSAI, PLMN ID of the SUPI, the TAI of the UE and the indication that the request is within a procedure of PDU Session establishment in the home-routed roaming scenario.

2.	If slicing configuration information for the S-NSSAI in the HPLMN is not available (e.g. the vNSSF has no cached information), the vNSSF invokes the Nnssf_NSSelection_Get service operation from NSSF of the HPLMN (hNSSF) according to the PLMN ID of the SUPI by including the HPLMN S-NSSAI. The vNSSF may use NRF to discover hNSSF, or the vNSSF is configured with the address(es) of the Nnssf_NSSelection_Get service(s) of the hNSSF.

3.	The NSSF in HPLMN may include the NSI ID, if needed, for the Network Slice instance in HPLMN selected for the corresponding S-NSSAI of the HPLMN in the Nnssf_NSSelection_Get response. The NSSF in HPLMN also includes the appropriate hNRF to be used to discover NFs/NF services within HPLMN in the Nnssf_NSSelection_Get response.

4.	The vNSSF includes in the Nnssf_NSSelection_Get response all the information that has been received from the NSSF in HPLMN in the response to the AMF.

The steps 5-8 below apply SMF discovery of the general procedure for NF/NF service discovery across PLMNs in the case of discovery made by NF service consumer defined in 4.17.5

5.	The AMF queries the vNRF using the Nnrf_NFDiscovery_Request by including PLMN ID of the SUPI, DNN, HPLMN S-NSSAI, the hNRF (if discovered in steps 1-4, cached or configured) and possibly an HPLMN NSI ID for the selected Network Slice instance corresponding to the HPLMN S-NSSAI if available in the AMF (obtained from the HPLMN NSSF in steps 3 and 4 or cached from a previous H-NSSF query, or configured).

6.	The vNRF invokes the Nnrf_NFDiscovery_Request service operation from hNRF (address aquired in step 5, or configured) according the procedure in Figure 4.17.4-1 to get the NF profiles of candidate H-SMF instance(s) in the HPLMN. As the vNRF sends an Nnrf_NFDiscovery request  on behalf of the AMF, the vNRF shall not replace the information of the requesting NF i.e. AMF ID, in the Nnrf_NFDiscovery_Request message. If hNRF does not hold the NF profiles of the candidate H-SMF instances, hNRF may further interrogate other NRF(s) of the HPLMN to discovery H-SMF instances. For further information about NRF-NRF interactions, see clauses clauses 5.3.2.2.4 and 5.3.2.2.5 of TS 29.510 [37].

7-8. The hNRF provides to the AMF, via vNRF, the NF profiles  of the discovered H-SMF instance(s) in Nnrf_NFDiscovery_Request response message, which may also include an NSI ID for the selected Network Slice instance corresponding to the S-NSSAI of the HPLMN, which can be used for subsequent NRF queries.

The second option is depicted in Figure 4.3.2.2.3.3-2.

Figure 4.3.2.2.3.3-2: Option 2 for SMF selection for home-routed roaming scenarios

1.	Based on the operator's configuration, the AMF queries the vNRF with PLMN ID of the SUPI, PLMN ID of the serving PLMN, DNN, the HPLMN S-NSSAI that maps to the S-NSSAI from the Allowed NSSAI or Partially Allowed NSSAI of the Serving PLMN the UE has requested, the endpoint of the discovery service of hNRF if available and if applicable and available, an HPLMN NSI ID (if the AMF has storedhNRF information and if applicable and available, an HPLMN NSI ID for the selected Network Slice instance corresponding to the S-NSSAI of the HPLMN) and DNN.

2.	The vNRF queries, on behalf of the AMF in VPLMN, the hNRF identified by means of the PLMN ID of the SUPI (if no endpoint address of hNRF discovery service is received from the AMF, the hNRF detmines the endpoint address of hNRF based on local configuration using  information received in step 1). The vNRF in VPLMN sends an Nnrf_NFDiscovery requestto the hNRF according the procedure in Figure 4.17.4-1 to get the NF profiles of candidate H-SMF(s). As the vNRFsends an Nnrf_NFDiscovery request on behalf of the AMF, the vNRF  shall not replace the information of the requesting NF, i.e. AMF ID, in the Nnrf_NFDiscovery_Request message.

The steps 3a-3d below apply SMF discovery of the general procedure for NF/NF service discovery across PLMNs in the case of discovery made by NF service consumer defined in 4.17.5

Depending on the available information and based on configuration, the hNRF may either execute steps in 3(A) or in 3(B).

3(A)	The hNRF provides to the AMF, via vNRF, the NF profile(s) of the discovered SMF instance(s) and possibly an NSI ID for the selected HPLMN part of the Network Slice instance corresponding to the S-NSSAI of the HPLMN for subsequent NRF queries in Nnrf_NFDiscovery_Request response message(steps 3a and 3b).

3(B)	The hNRF queries, on behalf of the AMF, an another NRF in HPLMN (e.g. a slice level NRF); this other NRF provides NF profiles of SMF instance(s) and possibly an NSI ID for the selected HPLMN part of the Network Slice instance corresponding to the S-NSSAI of the HPLMN for subsequent NRF queries (steps 3a and 3b) that the hNRF returns, via vNRF, to the AMF (steps 3c and 3d).

NOTE2:	In 3(B) the NF profile is provided by a second NRF, however the candidate NF profile(s) might recide in a third or fourth NRF etc. How NRF hierarchies are used is described in clauses 5.3.2.2.4 and 5.3.2.2.5 of TS 29.510 [37].

### 4.17.5	NF/NF service discovery across PLMNs in the case of discovery made by NF service consumer

In the case that the NF service consumer intends to discover the NF/NF service in home PLMN, the NRF in serving PLMN needs to request "NF Discovery" service from NRF in the home PLMN. The procedure is depicted in the figure below:

Figure 4.17.5-1: NF/NF service discovery across PLMNs

1.	The NF service consumer in the serving PLMN invokes Nnrf_NFDiscovery_Request (Expected Service Name, NF type of the expected NF, home PLMN ID, serving PLMN ID, NF type of the NF service consumer) to an appropriate configured NRF in the serving PLMN. The request may also include optionally producer NF Set ID, NF Service Set ID, S-NSSAI, NSI ID if available and other service related parameters. A complete list of parameters is provided in service definition in clause 5.2.7.3.2.

NOTE 1:	The use of NSI ID within a PLMN depends on the network deployment.

2.	The NRF in serving PLMN identifies NRF in home PLMN (hNRF) based on the home PLMN ID and it requests "NF Discovery" service from NRF in home PLMN according the procedure in Figure 4.17.4-1 to get the expected NF profile(s) of the NF instance(s) deployed in the home PLMN. As the NRF in the serving PLMN triggers the "NF Discovery" on behalf of the NF service consumer, the NRF in the serving PLMN shall not replace the information of the service requester NF, i.e. NF consumer ID, in the Discovery Request message it sends to the hNRF.

The hNRF may further query another  NRFs within the home PLMN based on the input information received from NRF of the serving PLMN. The  Address(es) of  the NF Discovery service(s) of this NRF in the home PLMN may be configured in the hNRF or may need to be discovered based on the input information. For further information about NRF-NRF interactions and NRF hiararchies, see clauses clauses 5.3.2.2.4 and 5.3.2.2.5 of TS 29.510 [37].

3.	The NRF in serving PLMN provides same as step 3 in clause 4.17.4 applies.

******************* END of Changes *******************