SA WG2 Meeting #162	S2-2405271

April 15 – 19, 2024, Changsha, China

Source:	Ericsson

Title:	KI#2.1: New solution for detail description of CONNECT-TCP

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#2.1.

# Discussion

In the TR there are candidate solutions for solutions based on CONNECT-IP and CONNECT-Ethernet describing to a certain level of detail how each connect method can be used in an ATSSS context. However, there is no similar solution description for CONNECT-TCP. Whether or not CONNECT-TCP should be included as part of the rel-19 conclusion is a separate discussion, but it would be good to at least capture a description of how solution for CONNECT-TCP works, on a similar level of detail as for the IP and Ethernet connect methods.

# Proposal

It is proposed to update TR 23.700-54 as follows:

**** First Change (all new text) ****

### 6.2.X	Solution #2.X: MPQUIC steering functionality using TCP proxying over HTTP

#### 6.2.X.1	Introduction

The solution in this clause  addresses the objective of KI#2.1 for a QUIC-based steering functionality to allow the transport of TCP packets over multiple accesses between UE and UPF. It generalizes on the current solution for ATSSS Phase 3 [17] for proxying UDP packets over HTTP over MP QUIC and extends it to TCP traffic.

Editor's note: It is FFS if this would become a new Steering Functionality or be part of the existing MPQUIC Steering Functionality.

The solution is primarily based on the IETF draft-ietf-httpbis-connect-tcp [10], which specifies how TCP traffic can be transferred between a client (UE) and a proxy (UPF) using the HTTP/3 protocol [9]. The HTTP/3 protocol operates on top of the QUIC protocol [12], which supports simultaneous communication over multiple paths, as defined in draft-ietf-quic-multipath [16].

The solution considers the following mode for transmitting TCP payload (see further details in clause 6.2.X.3, step 5):

-	Stream mode: This mode encapsulates the TCP payload into QUIC STREAM frames. It provides reliable transport (based on the existing mechanisms supported by the QUIC protocol) and supports data delivery without out-of-order packets and without duplicated packets.

Editor's note:	How this solution interacts with other "connect-x" based solutions to completely fulfil the scope of the KI #2.1 is FFS.

#### 6.2.X.2	High-level Description

The key principles of the solution are summarized below.

-	After the MA PDU Session establishment, the UE creates one or more multipath QUIC connections with the UPF. Each multipath QUIC connection is associated with a QoS flow, i.e. it carries the traffic mapped to a QoS flow.

-	The UE operates as a connect-tcp client and the UPF operates as a connect-tcp proxy, both defined in IETF draft-ietf-httpbis-connect-tcp [10]. Therefore, the UE supports an HTTP/3 client and the UPF supports an HTTP/3 proxy, both of them operating over QUIC.

-	On each of the established QUIC connections, the UE sends an extended HTTP CONNECT request to UPF (as defined in [9]) indicating that TCP proxying over HTTP is needed.

-	When the UE wants to transmit an uplink packet of an TCP flow, the UE:

-	Selects which QUIC connection will be used for the uplink traffic of the TCP flow based on the QoS flow associated with the TCP flow;

-	Creates a new bidirectional QUIC stream on the selected QUIC connection and sends an extended HTTP CONNECT request on this stream according to IETF draft-ietf-httpbis-connect-tcp [10];

-	Configures the QUIC stream to apply a transport mode and a steering mode (i.e. the transport mode and steering mode that should be used for the uplink traffic of the TCP flow based on the ATSSS rules); and

-	Forwards to UPF the uplink TCP payload of the TCP flow using multipath QUIC transport.

NOTE: In case of connect-tcp, only stream transport mode is supported (see below).

-	When the UPF wants to transmit a downlink packet of an TCP flow, the UPF:

-	Selects which QUIC connection will be used for the downlink traffic of the TCP flow (this QUIC connection is the same as the one selected by UE for the TCP flow, assuming the QoS flow in UL and DL directions is the same);

-	Selects a bidirectional QUIC stream on the selected QUIC connection (this QUIC stream is the same as the one created by the UE for the TCP flow);

-	Configures the QUIC stream to apply a transport mode and a steering mode (i.e. the transport mode and steering mode that should be used for the downlink traffic of the TCP flow based on the N4 rules); and

-	Forwards to UE the downlink TCP payload of the TCP flow using multipath QUIC transport.

The following figure illustrates how the traffic of a TCP flow is transferred between the UE and UPF using multipath QUIC transport. In this figure, it is assumed that the uplink traffic and the downlink traffic of the TCP flow are mapped to the same QoS flow (QFI-1). Therefore, both the uplink traffic and the downlink traffic of the TCP flow use the same multipath QUIC connection.

Figure 6.2.X.2-1: Using multipath QUIC transport for a TCP flow

The bidirectional QUIC stream is established by the UE to enable transmission of uplink TCP payload and downlink TCP payload of the TCP flow. The UE configures this stream to send uplink traffic with a steering mode determined based on the ATSSS rules in the UE (uplink steering mode). The UPF configures this stream to send downlink traffic with a steering mode determined based on the N4 rules in the UPF (downlink steering mode). The TCP byte stream of the TCP flow is transmitted in stream mode, i.e. transmitted directly over the bidirectional stream encapsulated in QUIC STREAM frames.

Figure 6.2.X.2-2 and Figure 6.2.X.2-3 illustrate the components of the MPQUIC steering functionality used to support proxying of TCP traffic in the uplink and downlink direction respectively. The MPQUIC steering functionality for TCP proxying is composed of three components:

1)	QoS flow selection & Steering mode selection: This component in the UE initiates the establishment of one or more QUIC connections, after the establishment of the MA PDU Session and, for each uplink TCP flow, it selects a QoS flow (based on the QoS rules), a steering mode (based on the ATSSS rules) and a transport mode (see further details below). This component in the UPF selects, for each downlink TCP flow, a QoS flow (based on the N4 rules), a steering mode (based on the N4 rules) and a transport mode (see further details below).

In the UE, this component is only used in the uplink direction, while, in the UPF, this component is only used in the downlink direction.

2)	HTTP/3 layer: Supports the HTTP/3 protocol defined in RFC 9114 [9] and the extensions defined in:

-	IETF draft-ietf-httpbis-connect-tcp [10] for supporting TCP proxying over HTTP; and

-	RFC 9297 [11] for supporting HTTP datagrams.

The HTTP/3 layer selects a QUIC connection to be used for each TCP flow and allocates a new QUIC stream on this connection that is associated with the TCP flow. It also configures this QUIC stream to apply a specific steering mode (further details are provided below).

In the UE, the HTTP/3 layer implements an HTTP/3 client, while, in the UPF, it implements an HTTP/3 proxy.

3)	QUIC layer: Supports the QUIC protocol as defined in the applicable IETF specifications (RFC 9000 [12], RFC 9369 [18], RFC 9001 [13], RFC 9002 [14]) and the extensions defined in:

-	draft-ietf-quic-multipath [16] for supporting QUIC connections using multiple paths simultaneously.

Figure 6.2.X.2-2: Components of MPQUIC steering functionality used for UL data transmission

Figure 6.2.X.2-3: Components of MPQUIC steering functionality used for DL data transmission

The protocol stack of the solution is depicted in Figure 6.2.X.2-4 below.

Figure 6.2.X.2-4: UP protocol stack of the solution

#### 6.2.X.3	Procedures

Figure 6.2.X.3-1 below depicts the key steps of the procedure that enables data traffic to be exchanged between the UE and UPF using the MPQUIC steering functionality. The figure shows a 1:1 relation between QUIC packets sent between UE and UPF and TCP payload segments received from the UE's application or from N6, but this is not necessary since the proxy copies bytes from the TCP stream onto the QUIC stream and there is no assumption that this has to be 1:1 on packet/segment level. This is different from the proxying of datagrams when e.g. connect-udp is used.

Figure 6.2.X.3-1: Procedure for enabling data traffic using the MPQUIC steering functionality

1.	The UE establishes a MA PDU Session with the 5G core (5GC) network. During the MA PDU Session establishment:

-	In the PDU Establishment Request message, the UE indicates that it supports the MPQUIC steering functionality. This indication can be used by the network (a) to select a UPF that also supports the MPQUIC steering functionality and (b) to decide whether the ATSSS/N4 rules for the MA PDU Session may use the MPQUIC steering functionality.

-	The UE receives MPQUIC proxy information, i.e. one IP address of UPF, one UDP port number and the proxy type (e.g. "connect-tcp"). This information is used by the UE for establishing QUIC connections with the UPF, which is also referred to as "MPQUIC proxy".

-	The UE receives one IP address/prefix for the MA PDU Session and two additional IP addresses/prefixes, called "link-specific multipath QUIC" addresses; one associated with 3GPP access and another associated with the non-3GPP access. These two addresses can be used by the UE to create two paths in a multipath QUIC connection.

-	The UE receives QoS rules and ATSSS rules to be applied for the MA PDU Session, for QoS enforcement and traffic steering enforcement respectively. Similar rules (N4 rules) are received by UPF.

2.	After the MA PDU Session is established and the UE identifies that one or more ATSSS rules require traffic steering using the MPQUIC steering functionality, the UE determines the number of multipath QUIC connections to be established with the UPF (MPQUIC proxy). For example, the UE may determine to establish as many multipath QUIC connections, as the number of QoS flows of the MA PDU Session, i.e. one multipath QUIC connection per QoS flow. The QoS rules provided to UE include downlink QoS information and the UE applies the downlink QoS information to establish QUIC connections for the QoS flows used for downlink traffic only.

3.	The UE establishes the number of multipath QUIC connection with the UPF (MPQUIC proxy) determined in the previous step. This results into several multipath QUIC connections between the UE and UPF, each one composed of multiple paths, e.g. one path over 3GPP access and another path over non-3GPP access. Data transmitted over a multipath QUIC connection must be encrypted according to RFC 9001 [13].

During a QUIC connection establishment, the UE and UPF negotiate QUIC transport parameters and indicate (a)support of multipath. They indicate support of multipath by providing the "enable_multipath" transport parameter (see draft-ietf-quic-multipath [16]).

After a QUIC connection establishment, the HTTP/3 client and the HTTP/3 proxy negotiate HTTP settings support of Extended CONNECT (see RFC 9220 [19]).

The QoS flow associated with a QUIC connection is also negotiated between the UE and UPF. This is done by using a new QUIC transport parameter (defined by 3GPP) when the QUIC connection is established.

NOTE 1:	QUIC transport parameter needs to be registered in IANA (by stage 3).

4.	The UE generates a new TCP payload (TCP payload #1) that should be sent via the MA PDU Session. This TCP payload initiates a new TCP flow. For this new TCP flow (and for each new TCP flow):

-	The UE selects a QoS flow (QFI) over which the TCP flow should be transmitted. This is selected by using the received QoS rules.

-	The UE selects a steering mode that should be applied for the TCP flow. This is selected by using the received ATSSS rules.

-	The UE selects stream transport mode since connect-tcp is used.

5.	The UE selects a multipath QUIC connection to be used for the new TCP flow (e.g. based on the selected QFI). The UE allocates a QUIC stream on this QUIC connection to be used for this new TCP flow.

6.   The UE sends an extended HTTP CONNECT to UPF (as defined in RFC 9484 [7]). This extended HTTP CONNECT request, essentially indicates that TCP proxying over HTTP is needed. If this is accepted by UPF, it responds with a 200 status.

UE configures the allocated stream to use the selected Steering and Transport mode.

7.	The UE sends the TCP payload using the allocated new stream on the selected QUIC connection.

When the stream transport mode is selected, the UE encapsulates the TCP payload within an HTTP frame that is transferred inside a QUIC STREAM frame.

8.	When a TCP/IP packet is received by UPF (MPQUIC proxy) from the remote host (TCP/IP packet #2), the TCP payload is transferred to the UE using the established context information for the TCP flow, i.e. using the selected multipath QUIC connection, the selected stream on this connection, the selected steering mode, and the selected transport mode. Such context information is stored in the UPF and in the UE and is applied for all the forwarded TCP payload of the TCP flow.

9.	Similarly, when additional TCP payload is generated by the UE app (TCP payload #3), this TCP payload is transferred to UPF (MPQUIC proxy) using again all the stored context information for the TCP flow.

NOTE 2:	The context information for a TCP flow in the UE and in the UPF is created when the initial TCP payload (i.e. TCP payload #1) of this TCP flow is transferred. All subsequent TCP payloads of the same TCP flow are transferred between the UE and the UPF using this context information.

When the UE identifies that the context information for an TCP flow is no longer needed, the UE deletes this context information and releases the associated QUIC stream, which cause the UPF to delete the context information stored in UPF.

The following figure 6.2.X.3-2 illustrates an example of how the uplink traffic of various TCP flows is transferred from the UE to UPF using QUIC multipath transport, and how the UPF relays this traffic to a final destination (remote host), although this solution does not preclude other forms of deployment in which the final destination may be associated with any IP address or a specific subnetwork.

Each TCP flow is associated to a QUIC connection and to an associated bidirectional QUIC stream, which is configured to apply a specific steering mode for the uplink traffic. For example, the red TCP flow is associated with the multipath QUIC connection #1 and with the red Stream Y, which is configured to apply the Smallest delay steering mode in the uplink direction.

All TCP flows shown in this figure are transferred with the stream transport mode, i.e. their data packets are transferred inside QUIC STREAM frames.

The downlink traffic of IP flows is transferred between the UE and UPF in a similar way.

Figure 6.2.X.3-2: Example of user-plane operation using the MPQUIC steering functionality (UL direction)

**** End of Changes ****