SA WG2 Meeting S2#162	S2-2405357

April 15 – 19, 2024, Changsha, PR China	revision of S2-2404593/5053

Source:	Nokia

Title:	KI#4: Conclusion on solution principles

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes to conclude on solution principles.

# Introduction

KI#4 is mainly about support of third party IDs in IMS which was intensively discussed in TR 23.700-87 (Rel-18).

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	KI#4 Principles for normative work

Editor's note:	Following points are FFS:

A) refer to either IMS AS or S-CSCF in bullet 1), 6) and 7)

B) the wording “the operator/third party can store” in bullets 4*

For KI#4 on "Extensible IMS framework to support authorization and authentication of third-party identities in IMS sessions" the following solution principles are agreed:

- Support authorization check in IMS AS / S-CSCF whether the calling party (PBX / user) is authorized to use Third Party specific identity information based the user profile retrieved from the HSS.

- The definition of Third Party specific user identity information used in IMS follows the definitions in draft-ietf-sipcore-callinfo-rcd-09 [19].

3.	Signing and verification of Third Party specific user identity information follows draft-ietf-stir-passport-rcd-26 [18].

4a.	Support configuration of an RCD address or optionally RCD information in the HSS per IMPU including wildcard IMPU.

4b.	For some users (i.e. that have individual subscriptions in HSS), the operator/third party can store per IMPU trusted RCD information from the Third Party network like caller name, job title, organization, and location information in the HSS.

4c	For some users behind a PBX (i.e. managed via a wildcarded IMPU in HSS), the operator/third party can store per wildcarded IMPU RCD information in the HSS.

4d	For some users, the operator/third party can store in HSS per IMPU an RCD address where Third Party specific user identity information can be fetched from.

Editor's note:	: The nature of RCD information storage is FFS

5.	Based on operator policy, either the fetched RCD address or the fetched RCD information is signed and provided in SIP messages (SIP INVITE) towards the terminating party network.

6	The RCD address or optional RCD Information is fetched by the IMS AS / S-CSCF from the HSS during IMS session setup.

7.	The IBCF or IMS AS /S-CSCF invoke signing and verification of Rich Call Data information based on local policies and trust relationship with the Third Party network.

8.	The fetched RCD address or RCD information is used by the Signing AS for signing the RCD PASSporT and by the Verification AS in the terminating network to verify the signed RCD PASSporT.

9). An user may be associated multiple RCD records. This does not require any normative work in this release.

Editor’s Note: it is FFS whether Informative text may be added during the normative phase to depict example(s) on how this can be made to work.

.

# * * * * End of changes * * * *