3GPP TSG-WG SA2 Meeting #162	S2-2405404

# Changsha, China, April 15- April 19, 2024

Source: 	China Mobile, Vodafone, SK Telecom, Deutsche Telekom, Nokia, Nokia Shanghai Bell,

Title:	Evaluation and Conclusion for KI#1

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Propose Evaluation/Conclusion for KI#1.

1. Discussion

This proposal proposes the evaluation and conclusion for KI#1 as baseline for normative work.

2. Proposal

It is proposed to capture the following changes in TR 23.700-63.

FIRST CHANGE

8	Conclusions

8.X	Interim Conclusion for Key Issue #1: Selection of UPF providing a selected user plane functionality

The following aspects are concluded as principles for normative work:

-	Reusing procedures in clause 4.17.1 and clause 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF.

-	The following UPF functionalities are added in UPF profile stored in NRF:

-	The functionality of NAT information exposure;

-	Packet Inspection functionality (to differentiate between IP or MAC filter based packet detection, and the packet detection based on other means, e.g. layer 7 DPI);

-  Defining a generic and operator configurable parameter in the N4 capabilities and UPF NF profile which can be used for non-standard or partially supported features and configured by operator to extend the baseline UPF capabilities. The format of the parameter is to be determined by stage 3.

-	for example, the operator may configure this parameter to represent customized configuration (e.g, hardware accelerators for example GPU, DPU etc.)

Editor’s NOTE: It is FFS whether the SMF selection logic of UPF can be based on the required or desired functionalities.

# * * * * End of changes * * * *