SA WG2 Meeting #162	S2-2405211

15 Apr - 19 Apr, 2024, Changsha, China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4805 | rev | 0 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Nnef_UEId Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, EDGEAPP |  |  |  |  |  | Date: |  |  | 2024-03-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The text “…AF is allowed and authorized” requires a reference to the appropriate clause in TS 33.501 (clause 12.4 annex V) to further indicate how AF authorization and user consent, as needed, can be obtained (based on different OAuth 2.0 flows) to allow retrieval of the MSISDN by a trusted/authorized AF. The text: “The AF is HPLMN or SNPN operator owned and operated within the operator domain and the AF does not re-expose the GPSI in MSISDN format outside the operator domain)” is removed so that an AF whether operator-owned or 3rd party-owned (e.g. a bank) so long as it is considered trusted and has obtained appropriate authorization and user consent can receive the MSISDN using Nnef_UEid service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies the conditions on the usage of the MSISDN retrieval (clause 4.15.10A) to allow the usage of Nnef_UEId Service for retrieving MSISDN by a trusted and authorized AF by adding reference to TS 33.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading condition limits the usage of the MSISDN retrieval (through Nnef_UEId Service) only to the operator-owned AFs and not to the trusted and authorized 3rd-party AFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.10A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

### 4.15.10A	MSISDN retrieval

This clause contains the detailed description and procedures for UE ID retrieval in the GPSI format of MSISDN as defined in TS 23.003 [33] for trusted AF. Depending on operator policy and local regulation exposure of GPSI in MSISDN format can be allowed when:

-	The trusted AF is allowed and authorized, using authorization code grant flow of OAuth-based mechanism as per clause 12.4 of TS 33.501 and a user consent mechanism e.g. as defined in Annex V of TS 33.501, to retrieve the UE ID.

The figure and procedures of clause 4.15.10 shall be applicable for MSISDN retrieval with the following differences:

-	description of the AF specific UE ID retrieval is replaced as the MSISDN retrieval; and

-	description of the AF specific UE Identifier represented as an External Identifier is replaced as the UE Identifier in the GPSI form of MSISDN.

*** End of Changes ***