SA WG2 Meeting #S2-167	S2-2501382

17 - 21 February, 2025, Athens, Greece

# Liaison Statement

| Liaison Statement Title: | LS from GSMA NG to 3GPP on SMS to emergency center |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| NRG#24 | 13/01/2025 | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| NRG 024_202 | 2024-12-17 | Eddy Goffin |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG/NRG | 2025-04-01 |  |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | Copy: GSMA REG, WAS, TSG, IDS |
| External Recipients: | To : 3GPP SA1 Copy : 3GPP SA2/CT1/CT4 |

# Access to PSAP via SMS in home PLMN

Short Message to contact the emergency services (PSAP) is widely deployed in Europe where 22 countries have enabled this service. The current functionality is based on sending a normal SMS to 112 that is rerouted to the Emergency Centre. This service is only available when in the home PLMN and requires the UE to be successfully registered.

Other countries, including United States, Canada, India, Australia, etc., have also implemented this service.

Please refer to Annex 1 for more details about deployment of access to PSAP via SMS.

# Roaming status

The current solution to access to PSAP via SMS is working only in the domestic scenarios due to the fact that the SMS is home routed. Roaming subscribers are only able to reach the local PSAP via an emergency call and this has impacts in terms of accessibility and useability.

Some regulators are recommending to enhance the access to PSAP via SMS to also work for the roaming subscribers (see for example EU regulation in Annex 2).

# Requirements for access to PSAP via SMS for roaming subscribers

GSMA NRG has discussed the access to PSAP via SMS for roaming subscribers and agreed that the following service requirements apply:

- Service-Rq#1: roaming subscribers shall be able to access the local PSAP via SMS if required by local regulation

- Service-Rq#2: SMSoNAS and SMSoIMS shall be supported

Annex 3 presents potential roaming architecture to support SMS to PSAP.

# Actions to 3GPP

To 3GPP SA1: GSMA NRG kindly ask 3GPP SA1 to consider the requirements defined in §3.

References

[1] ETSI EMTEL TS 103 625 Transporting Handset Location to PSAPs for Emergency Communications -Advance Mobile Location specification, v1.3.1, March 2023

[2] GSMA NG.119, Emergency Communication, v3.0, February 2024

NEXT NG/NRG MEETINGS:

NRG#25:	April 1st, 2025

NRG#26:	June 30th, 2025

Annex 1 - Emergency text to PSAP is today mainly based on SMS

Emergency Text to PSAP using SMS is currently used in multiple countries around the world (EU, United States, Canada, India, Australia, South Africa, Brazil, …).

The following figure describes some examples.

The figure hereafter described the SMS usage in Europe (EU27), where 80% of the European countries have implemented SMS to Emergency Centre.

Annex 2 - EU regulation recommends enhancing emergency access using also SMS in roaming condition

One of the major challenges is to enrich and improve the existing emergency service using IMS and enable the migration and operation of those new IMS emergency services (end-to-end) making possible to European Mobile Operators using Short Message Service to access PSAPs to be compliant to Recital 285 of the European Electronic Communications Code (EECC)

- Directive (EU) 2018/1972 of the European Parliament and of the Council of 11 December 2018 establishing the European Electronic Communications Code (Recast).

- Adopted in 2018 and must be transposed in the Member States by December 2020) states that:

“end-users should be able to access emergency services through emergency communications free of charge and without having to use any means of payment, from any device which enables number-based interpersonal communications services, including when using roaming services in a Member State. Emergency communications are a means of communication that includes not only voice communications services, but also SMS, messaging, video or other types of communications, for example real time text, total conversation and relay services” - (emphasis added).

Annex 3 - Potential roaming architecture to support SMS to PSAP

Potential roaming architecture discussed in GSMA is described in this annex.

Annex 3.1 - SMS over NAS

A potential roaming architecture is presented hereafter using SMSoNAS.

This architecture (already used for AML SMS for roamers) could be reused for SMS to Emergency Center in roaming and is described in the following references:

- ETSI TS 103 625 [1], Section 6.3.9

- GSMA PRD NG.119 [2], Section 6 and Annex D.3

The key features of this architecture are as follows:

- Reuse of current Mobile originated SMS architecture. no priority, valid registration, no need for PDN Connection/PDU Session

The impact with this potential solution on the UE is as follows:

- when an emergency number is detected, the UE performs the following steps:

  - generate SMSC address to access the local emergency service via visited SMSC connected to the visited PSAP
For example, SMSC address=+CC (VPLMN) + “Emergency number” 
(Typical example for Belgium: SMSC = +32-112)

  - send SMS over NAS using the SMSC address in step 1

The impact with this potential solution on the network is as follows:

- configuration change (SMS is routed to a new SMSC address using network node configuration)

- (note) other potential impacts can exist for SMSC to accept roaming traffic limited to emergency service

Note that the proposed architecture would also work in case of NTN access.

Annex 3.2 - SMS over IMS

A potential roaming architecture evolution is presented hereafter using SMSoIMS

SMSoIMS could be used to provide priority access for SMS to Emergency Centre (using the LBO emergency data session).

PSAP connection variant could be studied per protocol used.