3GPP TSG-WG SA2 Meeting #167	S2-2501765

17th – 21st February 2025, Athens, Greece	(revision of 25XXXXX)

Source:	Lenovo

Title:	pCR to TS 23.xyz: NRF Functionality

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes to add the descriptions of the NRF functionality in the TS 23.xyz regarding the Ambient IoT service.

1	Discussion

This paper intends to document the standard texts based on the agreed conclusion principles in TR 23.700-13, regarding the functional descriptions about the NRF.

2. Text Proposal

It is proposed to capture the following changes to TS 23.xyz for Ambient IoT.

# * * * * First change * * * *

4.5.6	NRF

In addition to the functions defined in clause 6.2.6 of TS 23.501 [x], the NRF further supports the following functions about Ambient IoT service:

-	Support storing and maintaining the registration of the new NF, i.e., AIOTF, and its corresponding NF profiles, that includes at least:

-	The AIOTF instance ID, IP address/FQDN, NF type;

-	The location information or the serving area information of the AIOTF, available data types of the AIoT devices within its serving area, available AIoT device information and its associated serving AIoT reader information within its serving area, AIoT service customer information (i.e. to indicate that the AIOTF is used for a dedicated service customer, e.g. a name like DNN may be used), etc.

-	Support the discovery of the AIOTF instances, based on the information provided, e.g., target area, and provide the discovered AIOTF instances to the service consumer of the NRF, e.g., NEF.

# * * * * End of changes * * * *