SA WG2 Meeting #167	S2-2501946

17 - 21 February, 2025, Athens, Greece	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.xyz: Service Operation Result Aggregation

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper adds service operation result aggregation principal.

1. Discussion

The TR 23.700-13 has concluded the aggregation can be performed by AIoT RAN based on the aggregation assistance information received from the AIOTF. This paper adds service operation result aggregation principal as a high level feature in clause 5 of TS 23.xyz.

2. Proposal

It is proposed to capture the following changes into TS 23.xyz.

* * * * Start of Changes * * * *

5.X	Service Operation Result Aggregation

The AIoT RAN may perform service operation result aggregation based on the assistance information received from the AIOTF. An AIoT service operation with a specific Correlation ID may involve many AIoT Devices, for the purpose of efficient reporting the service operation result the AIoT RAN may aggregate the result before sending it to the AIOTF.

The AIOTF may determine the aggregation assistance information based on operator policy and/or request from the AF, which includes:

-	Aggregation per time window (e.g., aggregate and report data for a time period);

-	Aggregation per number of AIoT devices (e.g. for inventory, threshold for the maximum number of AIoT Device ID(s) required for reporting);

-	Any combination of above (if needed).

Editor's note:	The details of the aggregation assistance information needs to be updated according to the conclusion in TR 23.700-13.

* * * * End of Changes * * * *