3GPP TSG-WG Meeting # SA2 167	S2-2502114

# Athens, Greece,  February 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6151 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 EN removal and MPQUIC recomendation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | See S2-2502109 for motivation for the proposed way forward |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following task is included in XRM Phase 2 exemption request SP-241514: • Support differentiated QoS handling of ciphered multiplexed XRM traffic other than SRTP. Scope is DL only. It is contentious whether this feature must be done in Rel-19. The task is addressed with this CR that removes the Editor Note in TS 23.501 clause 5.37.9.1. In addition, 5.37.11 is updated to point to MPQUIC as a technology to achieve different 5-tuples per media flow. Further details are provided in a new annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One task in approved Exemption request is not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.9.1, 5.37.11, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** First change **************

### 5.37.9	Support of transferring media related information over N6

#### 5.37.9.1	General

This clause describes the procedure to support transferring of media related information (i.e. information related to the media being transmitted, either explicitly provided by the AS or inferred from protocol headers or payload, e.g. PDU Set Information, Data Burst Size, etc.) over N6. Such support is necessary is when the end-to-end connection for XR media between the UE and Application Server is fully encrypted thus UPF not being able to identify, for example, PDU set information.

On-path N6 signalling methods are used to transfer media related information over N6 where media related information is sent together with the encrypted payload allowing media related information to be received synchronously with the media packet. The following on-path N6 signalling methods are supported:

-	Using Media over QUIC (MoQ), IETF draft-ietf-moq-transport [201], whose usage is further defined in clause 5.37.9.2.

-	Use of connect-UDP for Using Proxying-UDP-in-HTTP, IETF RFC 9298 [170] and QUIC-Aware Proxy, draft-ietf-masque-quic-proxy [200], whose usage is further defined in clause 5.37.9.3

-	Using UDP-Options, IETF draft-ietf-tsvwg-udp-options [202], whose usage is further defined in clause 5.37.9.4.

When Media over QUIC is used the UPF identifies PDU Set Information and End of Data Burst Indication from media related information specified in [201].

When Proxying-UDP-in-HTTP RFC 9298 [170] or QUIC-Aware Proxy [200] or UDP-Options, IETF draft-ietf-tsvwg-udp-options [202], is used, the following media related information are supported:

-	PDU Set Information (as described in clause 5.37.5.2).

-	End of Data Burst Indication (as described in clause 5.37.8.3).

-	Expedited Transfer Indication (as described in clause 5.37.9).

-	Data Burst Size (as described in clause 5.37.9).

-	Time to Next Burst (as described in clause 5.37.9).

The mechanisms defined in clause 5.37.9 address only UE to XRM server (AS) communication and do not address UE-UE XRM communications. The mechanisms defined in clause 5.37.9 support only downlink media related information delivery to the UPF.

The media related information is encrypted and integrity protected between the UPF and the AS, as defined in TS 33.501 [29].

************ Next Change ************

### 5.37.11	Traffic identification and differentiated QoS for multiplexed media flows

5.37.11.x	General

XR and interactive media services can send data traffic of different media components with different QoS requirements. Several media flows could be multiplexed on the same end-to-end transport layer connection. Clause 5.37.11.x describes how 5GS can provide differentiated QoS handling for multiplexed (S)RTP media flows.

NOTE:  an aplication that is multiplexing media in same QUIC transport connection with support for the multipath QUIC extensions (see [174]) can use different paths with different 5-tuples for the different media flows. In this case, the multiplexed encrypted media flows can be differentiated in uplink and downlink using IP address and port. For further information see Annex X.

5.37.11.y	Multiplexed (S)RTP media flows

. In order to uniquely identify each media flow, the IP Packet Filter can further include multiplexed media specific (S)RTP Multiplexed Media Identification Information to differentiate the media flow among multiple media flows that share the same transport layer related elements in the packet filter (see clause 5.7.6.2).

The AF may provide individual QoS requirements for each individual RTP media flow within the multiplexed data flows and for the (S)RTCP that controls the (S)RTP transmission, by including the (S)RTP Multiplexed Media Identification Information associated with the individual QoS requirements within the flow description in the "AF session with the required QoS" procedure specified in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3]. The policy control related to multiplexed media flows identification is specified in clause 6.1.3.22 of TS 23.503 [45].

If a UE indicates support of (S)RTP Multiplexed Media Identification Information in the IP packet filter, it shall indicate this to the SMF at PDU Session establishment and the SMF provides this UE capability information to the PCF. The PCF considers such UE capability information when providing the PCC rules to the SMF.

If (S)RTP Multiplexed Media Identification Information is received in the PCC rule, the SMF provides (S)RTP Multiplexed Media Identification Information to the UPF in the corresponding PDR(s) and also to the UE in the QoS Rule if the UE has indicated that it supports (S)RTP Multiplexed Media Identification Information in IP packet filter. If the UE has not indicated so, the SMF shall not provide the (S)RTP Multiplexed Media Identification information in the QoS Rule to the UE. Reflective QoS shall not be applied to SDFs which are identified with (S)RTP Multiplexed Media Identification Information defined in clause 5.7.6.2.

************ Next Change ************

Annex X (informative):
Media flow multiplexing and Multipath QUIC

An application can multiplex media flows with different QoS requirements in same QUIC transport connection. IETF has specified extensions for multipath QUIC (MP-QUIC) that can be used to send the multiple media flows multiplexed in the same end-to-end transport layer connection (i.e. one QUIC connection and single security context) on different paths, where each path can use a different 5-tuple. As an example, these paths could share the server IP address and have different port numbers.

Using multipath QUIC, the several multiplexed encrypted media flows can be differentiated for QoS handling in uplink and in downlink using the 5-tuple. The congestion control contexts are independent for each path. And as a result, delays, loss and reordering on one QoS flow do not directly impact media flows on other QoS Flows.

At the endpoint, the QUIC transport layer with Multipath QUIC support needs application guidance to associate the different media flows to paths. One alternative is to base this association in a media flow relative priority. On the other hand, the application layer (and more specifically, AF), needs to know the 5-tuples that apply to the media flows to provide the QoS requirements accordingly. As an example, the application layer could receive information of how media flows or priorities have been associated to ports (if paths share IP address but have different port).

************ End of changes ************