3GPP TSG-WG SA2 Meeting #167	S2-2501942

17th – 21st February, 2025, Athens, Greece

Source:	HONOR

Title:	pCR to TS 23.xyz: AIoT Reader Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: This paper proposes the AIoT RAN reader selection high level functionality.

1. Introduction/Discussion

Based on the conclusion in TR 23.700-13, this contribution is prepared to add AIoT Reader Selection functionality in clause 5.3.

2. Text Proposal

It is proposed to approve the following change.

FIRST CHANGE

5.3	AIoT Reader Selection

5.3.1	General

AIoT Reader selection is the means for the AIOTF to determine multiple or an individual AIoT Reader to handle the service operation request(s). The AIOTF can determine which AIoT readers to use based on the information provided by the AF and/or AIoT device context and/or AIoT device profile. If no AIoT readers can be selected, then the service operation request is rejected.

For the initial AIoT reader selection, if the information used for AIoT reader selection is not provided by the AF, then how the AIOTF determines which AIoT readers to use is based on implementation.

5.3.2	AIoT RAN Reader Selection

An A-IoT RAN node may support one or more AIoT RAN Readers which provide communication services towards Ambient IoT Devices. The AIOTF selects A-IoT RAN node(s) and optionally indicates A-IoT RAN node(s) to select which AIoT RAN Readers to handle the service operation request(s). If the AIOTF cannot indicate A-IoT RAN node(s) to select which AIoT RAN Readers to use, the A-IoT RAN node selects AIoT RAN reader(s) based on the local configuration or implementation.

If the AF provides the target area information, the AIOTF can determine A-IoT RAN node(s) based on the target area information. Furthermore, the AIOTF determines AIoT RAN Reader(s) optionally based on the target area information and A-IoT RAN Readers information if available. The OAM may configure the AIOTF with AIoT RAN Readers information including e.g. reader ID(s) and their corresponding service area when AIoT RAN Readers are integrated in the network.

Editor’s note:	Whether RAN3 will specify AIoT Readers information (e.g. AIoT RAN Reader/BS reader ID, service area) reporting to the AIOTF over NGAP depends on RAN3 decision and if so SA2 may include this feature in SA2 specifications.

If the AF provides AIoT device ID(s), the AIOTF can determine A-IoT RAN node(s) or AIoT RAN Reader(s) based on AIoT device ID(s) and AIoT device context (e.g., the requested target AIoT Devices last known location) stored in the network.

If the AF provides AIoT RAN readers identifier information, the AIOTF can determine AIoT RAN Reader(s) based on AIoT RAN readers identifier information and the AF subscription data.

NOTE: The AIoT RAN readers identifier information is assumed to be pre-negotiated between the AF and the operator.

END OF CHANGE