3GPP TSG-WG SA2 Meeting #167	S2-2502338

Athens, GR, 17th Feb – 21st Feb, 2025	(revision of S2-2501971, 0932)

Source:	Huawei, HiSilicon, NEC

Title:	pCR to TS 23.xyz: Profile Management for Ambient IoT Device

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract: add the support of Profile Management for Ambient IoT Device.

1. Discussion

Following the conclusion in clause 8.2.2 of TR 23.700-13, this proposal introduces the support of Profile Data Management for Ambient IoT Device.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.xyz.

# * * * * First change (all new texts) * * * *

5.5	AIoT Device Profile Data Management

The network may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g., Device ID or credentials) is stored external to the network.

The AIoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in TS 23.502 [4] clause 5.2.3, it is stored in the ADM (AIoT Data Management) network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

Editor’s note: Whether ADM support ID security mechanism pending on SA3 decision.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Unique identify the AIoT Device. |
| AIoT Device Status | Indicate the status of the AIoT device, e.g., is to be permanently disabled or not, or unknown. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

Editor’s note:	SA WG2 will align security related materials with SA WG3 decision later.

# * * * * End of changes * * * *