3GPP TSG-SA WG2 Meeting #168	S2-2503316

Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	Key issue for WT#1: Architecture enhancement to support UE data collection.

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 on architecture enhancement to support UE data collection.

- Discussion

In the RAN LS RP-242389, the following agreements on UE side data collection are captured as bellow:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

In RAN LSs RP-243316, it is clearly mentioned transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements.

The LPP (LTE Positioning Protocol) is defined in the LCS feature. It serves as the dedicated protocol for carrying positioning-related signaling and measurement data between the UE and the LMF. Both the UE and LMF should support LPP to properly interpret positioning-related signaling and data, ensuring seamless execution of the LCS procedure.

In the LCS architecture, positioning can also be performed over the user plane (UP). In such cases, LPP messages are exchanged between the UE and LMF via the UP connection.

To ensure full visibility and controllability of UE data collection over the UP, both the UE and the 5GC NF should support a common protocol. This protocol is responsible for provisioning the types of data the UE needs to collect and transmitting the collected data from the UE to the 5GC NF.

The 5GC NF must be capable of interpret this data to determine how it should be exposed to 3rd party, ensuring proper data handling while maintaining compliance with privacy and security requirements.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

5.x	Key Issue #x: Architecture enhancement to support UE data collection.

5.x.1	Description

To ensure full visibility and controllability of UE data collection over the UP, both the UE and the 5GC NF should support a common protocol. This protocol is responsible for provisioning the types of data the UE needs to collect and transmitting the collected data from the UE to the 5GC NF.

The 5GC NF must be capable of interpret this data to determine how it should be exposed to 3rd party, ensuring proper data handling while maintaining privacy and security requirements.

This key issue studies how to enhance the architecture to support UE data collection between the UE and 5GC. In particular, this key issue will study:

-	Whether the existing protocol or the new protocol is needed to support the UE data collection.

-	How to coordination and configuration of the UEs to participate in the UE data collection including signalling control and configuration information between the UE and the 5GC NF to support the MNO controllability.

-	What configuration information are exchanged between the UE and the 5GC NF for UE-side model training.

NOTE 1:	Any data to be collected from UE by 5GC NF for AI/ML for NR air interface operation with UE-side model training is assumed to be defined by RAN WGs.

NOTE 2: Whethe the existing protocol or the new protocol should be used to support the UE data collection will be cooredinate with RAN WGs.

* * * End of Change * * * *