3GPP TSG SA WG2#167	S2-2503352

# Goteborg, Apr 7th – 11th 2025	                                                         (revision of S2-2501871)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6128 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB-UE authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the following descriptions from clause of 5.49.3.2, AMF behaviours are not clear. When the MWAB-UE can no longer use S-NSSAI(s) dedicated to MWAB operation,, e.g. the dedicated S-NSSAIs and DNNs for MWAB operation are removed from the subscription (i.e. the MWAB becomes unauthorized) and a UE subscription update (including a UE configuration update) step happens, or the MWAB-UE moves to an area where the S-NSSAI is not available, if the AMF is configured to do so for the dedicated S-NSSAI and DNN for MWAB operation, the AMF delays the corresponding PDU session release based on an operator configuration (e.g. a local configured timer to allow time for the MWAB-gNB to handover the UE(s) it serves to other cells). When S-NSSAI is not available due to MWAB mobility, AMF decides the MWAB operation becomes unauthorized by removing the dedicated S-NSSAI from current Allowed NSSAI and provides the new Allowed NSSAI to MWAB via UE Configuration Updated procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on MWAB-UE authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF behaviours are not clear when S-NSSAI is not available due to MWAB mobility |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.49.3.2	MWAB-UE authorization

The MWAB-UE is authorized based on its subscription information which includes dedicated S-NSSAI(s) and DNN for MWAB operation. Such dedicated S-NSSAI(s) and DNN(s) shall be part of the subscription information for the MWAB-UE as specified in clause 5.2.3.3.1 of TS 23.502 [3].

The authorization of the MWAB-UE to establish connectivity in a BH PLMN/SNPN is based on whether dedicated S-NSSAI(s) for MWAB operations are received as part of Allowed NSSAI in BH PLMN/SNPN following existing procedures as described in clause 5.15 (i.e., the MWAB becomes authorized when Allowed NSSAI in BH PLMN/SNPN includes dedicated S-NSSAI(s) for MWAB) and whether it can establish BH PDU session(s) in the BH PLMN/SNPN using the dedicated S-NSSAI(s) and DNN(s).

The MWAB-UE authorization supports time-based or location-based control.

-	For time-based control, existing time-based network slice subscription control can be reused (including the enhancements in clause 5.15.16).

-	For location-based control, existing mechanism, e.g. service area restriction, LADN based control (see e.g. clauses 5.6.5 and 5.6.5a) can be reused.

NOTE 1:	When the MWAB-UE can no longer use S-NSSAI(s) dedicated to MWAB operation,, e.g. the dedicated S-NSSAIs and DNNs for MWAB operation are removed from the subscription (i.e. the MWAB becomes unauthorized) and a UE subscription update (including a UE configuration update) step happens, or when the MWAB-UE moves to an area where the S-NSSAI is not available, if the AMF is configured to do so for the dedicated S-NSSAI and DNN for MWAB operation, the AMF provides the new Allowed NSSAI  to the MWAB-UE via UE Configuration Updated procedure and delays the corresponding PDU session release based on an operator configuration (e.g. a local configured timer to allow time for the MWAB-gNB to handover the UE(s) it serves to other cells).

NOTE 2:	If the S-NSSAI(s) used by BH PLMN for MWAB operation is not dedicated for MWAB, the behaviours of AMF as in NOTE 1 cannot be limited to MWAB-UEs only. Additionally, it may also not be possible to configure specific treatment for MWAB-UEs (different from other UEs) in the NG-RAN.

The MWAB-UE may be connected to the BH PLMN/SNPN for other services, if allowed by the subscription data, even if the S-NSSAIs and DNNs for Backhaul PDU sessions are no longer authorized.

# * * * * End of changes * * * *