3GPP SA WG2 Meeting #168	S2-2503638

07 - 11 April, 2025, Goteborg, Sweden

Title:	KI #X: New Solution: CST reduction by pre-negotiate SDP parameters with an early SIP INVITE

Source:	MediaTek Inc.

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract:	The contribution proposes a solution to KI #X: XXXXXXX

Introduction

This solution aims to reduce the call setup time by negotiate pre-configured SIP call SDP parameters with an SIP INVITE request after IMS registration procedure.

Discussion

By moving the SDP negotiation (i.e. offer/answer) procedure during the Call Setup procedure “earlier”, i.e., move the negotiation phase to an early SIP INVITE which the UE sends after IMS registration, can reduce later Call Setup procedure number of messages and message size. This can reduce the overall CST (Call Setup Time). Once this procedure is complete, the SDP parameters would be static in all the following calls, we may regard this “early” SDP negotiation procedure as a SDP pre-configuration procedure.

Proposal

A solution is proposed for KI#X for incorporation in the FS_5GSAT_Ph4-ARC TR23.700-19.

*** Start of changes (all new text) ***

6.X	Solution #X: CST reduction by pre-configured SDP parameters with an early SIP INVITE.

6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #X, " XXXXXXXXXXXXXXX".

6.X.2	Description

This solution aims to reduce the call setup time by pre-configured SIP call SDP parameters with an early SIP INVITE.

6.X.3	Procedures

Figure 6.X.3-1: Pre-configuration Procedure with an early INVITE

Figure 6.X.3-1 is a signaling diagram for UE using GEO satellite access to successfully activate the “SDP pre-negotiation” mechanism, which includes the following steps:

0.	The UE is ATTACHED to a PLMN over GEO via NB-IoT NTN connecting to EPC and has established IMS PDN Connection and registered to IMS. The UE acuiqres the IP address of the UE and the P-CSCF during the IMS PDN Connection establishment procedure. The UE during the SIP registration procedure indicates its support of "SDP pre-config" and the IMS Server responds to accept to use the "SDP pre-config".

1.	The UE sends a SIP INVITE message, which includes a “SDP pre-config” feature tag to indicate that this SIP INVITE message is aim to do the “SDP pre-config” and a SDP offer containing the requested SDP parameters of UE side (e.g., IP address, port, and voice codec), to the P-CSCF.

2.	The P-CSCF supports “SDP pre-config” and selects the final SDP parameters, saves it in the P-CSCF and responds with a 200 OK message that includes SDP answer containing the pre-configured SDP parameters of P-CSCF side.

3.	The pre-configured SDP parameters is saved in both the UE and P-CSCF sides, and the pre-configured SDP parameters determined during previous steps (i.e., with an early SIP INVITE message) will be used in all the following IMS calls during the IMS registration lifecycle

NOTE 1:	Early INVITE pre-configuration can extend to support adding more pre-configured media configurations and modify pre-configured media configurations..

Figure 6.X.3-1a: Example of SDP negotiation with an early SIP INVITE message

Figure 6.X.3-1a is a more detailed SDP negotiation example with an early SIP INVITE message.:

Figure 6.X.3-2: Call Setup Procedure with “SDP pre-config” feature

Figure 6.X.3-2 is a signaling diagram of a UE using GEO satellite access makes a MO IMS call with another UE using TN access according, which includes the following steps:

0.	The UE successfully finished the pre-configured media configuration procedure with early SIP INVITE message.(see Figure 6.X.3-1.

1.	The UE-1 using GEO access initiates an IMS call to the UE-2 using TN access and send INVITE without SDP.

2~9.	The IMS CN will use “SDP pre-config” parameters stored in the IMS CN to trigger SIP interactions between IMS CN and UE-2, the SIP interaction between IMS CN and UE-2 will apply the traditional SDP offer/answer process thus the IMS CN forwards the INVITE to UE-2 with SDP offer, and UE-2 responds 183 Session Progress with SDP answer. All the SIP interactions will not be forwarded to UE-1 until UE-2 has sent out the 180 Ringing.

10.	The IMS CN activate Dedicated EPS Bearer Context for voice with the UE-1 using the stored “SDP pre-config” parameters.

11.	The IMS CN forwards 180 Ringing to UE-1 without SDP.

12~13.	UE-2 answers the call, send 200 OK to UE-1 and UE-1 responds ACK to UE2.

14.	The media paths between UE-1 and IMS CN, and between UE-2 and IMS CN have been established

### 6.X.4	Impacts on services, entities and interfaces

UE:

-	Support “SDP pre-config”

IMS Server

-	Support “SDP pre-config”

*** END of changes ***