3GPP TSG|WG-SA2 Meeting #168	S2-2503676

Goteburg, Sweden, April 07 – 11, 2025	(revision of S2-20xxxx)

Source:	Samsung

Title:	Discussion on removal of Replaced S-NSSAI from Configured NSSAI for AF triggered slice replacement

Agenda Item:	19.32

Document for:	Information

# 1.	Background

For the TEI19_SliceSel feature, there is an EN which is yet to be resolved though it was discussed in SA2#166ahe (S2-2500952) and SA2#167 (S2-2502097) meeting.

Editor's note:	It is FFS whether the Replaced S-NSSAI should be removed from the Configured NSSAI.

# 2.	Discussion

In case of AF triggered slice replacement, the Replaced S-NSSAI gets replaced with Alternative S-NSSAI when third party activates the feature and reverted (i.e. Alternative S-NSSAI is replaced with Replaced S-NSSAI) when third party deactivates/terminates the feature. It is expected that until unless third party does not deactivate the feature, the user shall continue with the Alterative S-NSSAI.

During feature activation, two approaches are getting discussed about handling of the Replaced S-NSSAI and that is whether the Replaced S-NSSAI should be removed from Configured NSSAI or should be added to Rejected NSSAI with cause set to “slice rejected due to RA”.

If the Replaced S-NSSAI removed from Configured NSSAI is considered then in the scenario when UE gets deregistered from the network and during this time (UE is in deregistered state) if third party deactivates or terminates the feature then question arises, how the network is going to add the removed Replaced S-NSSAI back to Configured NSSAI when the same UE trying to register with the network?

If the Replaced S-NSSAI added to Rejected NSSAI with cause set to “slice rejected due to RA” is considered then in the scenario when UE gets deregistered from the network, during this time (UE is in deregistered state) if third party deactivates or terminates the feature, UE gets Replaced S-NSSAI automatically when the same UE trying to register with the network because the Rejected cause of “slice is not supported in RA” gets reset. Is it really an advantage to consider over the other approach?

AF triggered slice replacement feature activation and deactivation happens using either 1) AM Policy Authorization and 2) AM Influence procedures.

AM Influence: In this procedure, the third party sends the slice replacement activation/deactivation request is sent to UDR. AM-PCF gets this update from UDR and inform to AMF. So if the feature deactivation information is received from PCF, AMF takes the desired action like AMF can add the Replaced slice back to Configured NSSAI or remove the slice from Rejected NSSAI which means UE can use the Replaced S-NSSAI.

Observation 1: When UE is in deregistered state, then irrespective of the approach like removal of Replaced S-NSSAI from Configured NSSAI or Replaced S-NSSAI is rejected due to RA by adding to Rejected NSSAI is used there is no problem as when UE subsequently tries to register with the network, PCF will get the information from UDR and provide to AMF.

AM Policy Authorization: In this procedure, the third party directly send the request to AM-PCF after getting the AM-PCF address from BSF which means it is aware of the UE’s registration status. If the UE is in deregistered state, then the third party neither can activate the feature nor deactivate the feature.

Observation 2: When UE is in deregistered state, the third party can not deactivate the feature using AM Policy Authorization procedure. Still, the UE gets Replaced S-NSSAI instead of Alternative S-NSSAI if Slice is rejected due to RA option is consider which means the feature gets reset/deactivated/broken. There is no such impact if removal of Configured NSSAI is used.

Observation 3: Third party does not want to deactivate the feature but still the UE gets Replaced S-NSSAI instead of Alternative S-NSSAI if Slice is rejected due to RA option is considered which means the feature gets reset/deactivated/broken. There is no such impact if removal of Replaced S-NSSAI from Configured NSSAI is considered.

Observation 4: Irrespective of feature is deactivated or not, the feature is always gets reset if the UE logged out of the network and login again (deregister and register) when Slice is rejected due to RA option is considered.

# 3.	Proposal.

Proposal: By looking at the all four observations, it is proposed to consider the proposal of removing Replaced S-NSSAI from Configured NSSAI and agree to S-241xxxx