SA WG2 Meeting #168	S2-2503793

Goteberg, Sweden, 07-11 April, 2025

# Title:	Update to Command procedure

# Source:	InterDigital Inc.

# Document for:	Approval

Agenda Item:	19.14.2

Abstract:	The contribution proposes an update to the Command procedure to address various scenarios of the result of the Inventory procedure.

1. Discussion

In the existing Command procedure, Inventory procedure (Step 7-11) is initiated by the AIOTF before fowarding the Command Request (Step 12a/12b) to the AIoT RAN. However, it is not clear how the Command Request in Step 12a/12b is handled depending on the result of the Inventory procedure. A few scenarios are possible:

Scenario 1: The original Command Request from the AF (Step 1) is associated with one single target device, i.e. the Command Request includes one single device ID. The result of Inventory procedure may be success or failure. In case of failure, the Command procedure should not continue.

Scenario 2: The original Command Request from the AF (Step 1) is associated with multiple target devices, i.e. the Command Request includes multiple device IDs or device filtering information. The Inventory responses from all the target devices may be received, or from a part of the target devices, or none of the target devices. The follow-up Command procedure should continue only for those devices that has returned succesful Inventory responses. If the original Command Request doesn’t contain specific device identifiers (i.e., only device filtering info is included), the new Command Request(s) after the Inventory reponses are received should contain the device identifiers received in the Inventory responses.

2. Proposal

It is proposed to adopt the proposed change to Clause 6.2.3 “Procedure for Command” in the draft TS 23.xyz.

* * * * First Change (All new text) * * * *

### 6.2.3	Procedure for Command

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], (AIoT data)) to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The NEF selects AIOTF(s) taking into account the target area information for AIoT service operation and Target AIoT device information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed. Inventory Report from multiple devices through multiple BS Readers may be received.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	For each successful Inventory response received, the AIOTF sends Command Request message (correlation identifier, AIoT Device ID(s), NAS Command Request) to the AIoT RAN from which the Inventory response is received. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request. If none of successful Inventory response is received, Step 12a -15 is not performed and the AIOT sends a failure report to the NEF in Step 16.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	For each successful Inventory response received, the AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.. If none of successful Inventory response is received, Step 12b -15 is not performed and the AIOT sends a failure report to the NEF in Step 16.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to AIoT RAN.

13.	The AIoT RAN sends the AS R2D message (NAS Command Request) to the AIoT device.

14.	The AIoT device sends the AS D2R message (NAS Command Response) to the AIoT RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct path as shown in clause 4.2.2.2, step 15a is performed.

15a.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect path via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Command Response (correlation identifier, Reader ID, NAS Command Response)) to the AIOTF.

16.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

17.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

* * * * End of Change * * * *