3GPP TSG-SA2 Meeting #168	S2-2504219

07 - 11 April, 2025, Goteborg                                                           (revision of S2-2503977)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1386 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements on the input and output of QoS and Policy Assistance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, ETRI, Samsung, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Some applications are sensitive to bandwidth and packet loss rate, such as cloud gaming, where RAN status may also affect the QoE value even under the same RAN signal power and quality. For these applications, the RAN status values need to be analyzed by the NWDAF, and the perfomance data collected from the OAM reused in this paper enables the NWDAF to more accurately predict the QoE. - In the ouput predictions of QoS and Policy Assistance Analytics, the "Applicable duration of QoS and Policy Assistance information" is redundant since the "Validity period" was already defined to indicate the applicable time duration in a time slot for the assistance information. the "Applicable duration of QoS and Policy Assistance information" is proposed to be removed from Table 6.23.3-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RAN performance data collected from the OAM defined in clause 6.6.2 in TS 23.288 is added as input. - Delete "Applicable duration of QoS and Policy Assistance information" in the output table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete input data may lead to unreliable QoE prediction. Duplicated information may lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.23.2, 6.23.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.23.2	Input Data

The NWDAF supporting QoS and policy assistance analytics shall be able to collect information from AF, OAM and 5GC NFs.

The following specified input data might be reused by QoS and policy assistance analytics:

-	The input data defined for Observed Service Experience related network data analytics in clause 6.4.2, e.g. the information of the application, Service Experience and QoE metrics.

-	The input data collected from the OAM for Network Performance analytics as defined in clause 6.6.2, e.g. the RAN status, load and performance per Cell Id for the traffic type of interest and in the Area of Interest.

In addition, more information collected by the NWDAF from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.23.2-1 and from OAM in Table 6.23.2-2.

NOTE:	The NWDAF may also collect the Observed Service Experience Analytics from other NWDAFs, based on NWDAF implementation.

Table 6.23.2-1: Input data from 5GC NF related to QoS and Policy Assistance information

| Information | Source | Description |
| --- | --- | --- |
| QFI allocation, QoS Flow change, or QFI deallocation events (1…max) | SMF | SMF provides such events (see clause 5.2.8.3.1 of TS 23.502 [3]). |
| > Used QoS parameter set(s) | SMF | The QoS parameter set(s) that have been already applied by SMF. As detailed in clause 6.23.1, the QoS parameter set includes 5QI or QoS characteristics attributes and optionally GFBR and MFBR for GBR flows. |
| > Used QoS profile | SMF | The QoS profile associated to the used QoS parameter set(s). |
| > Traffic descriptor | SMF | One of Application Identifier, IP Packet Filter Set or Ethernet Packet Filter Set |
| > S-NSSAI | SMF | Slice used to transport the QoS flow(s). |
| > DNN | SMF | DNN associated with the QoS flow(s). |
| > DNAI | SMF | Identifies the access to the DN that the PDU Session is connected to. |
| UE identifier | SMF, AMF | The identifier of UE, e.g. SUPI, UE IP address, etc. |
| UE Location | AMF | The UE location information, e.g. cell ID or TAI. |
| RAT Type | SMF | RAT Type used for the UE. |
| Time stamp | SMF, AMF | The time stamp associated to the collected data. |
| N6 Delay (NOTE 1) | UPF | The observed N6 delay between the UPF and measurement endpoint in the DN, as defined in clause 5.8.2.23 of TS 23.501 [2]. |
| Packet Loss Rate (NOTE 2) | UPF | The Packet Loss Rate for uplink and/or downlink. |
| NOTE 1: Whether and how N6 delay (independent of QoS profile) is considered to derive the analytics in comparison to delay between UE and UPF (dependent on the QoS profile) is left to implementation based on the operator's policy. NOTE 2: The Packet Loss Rate is applicable to the TCP traffic for which the UPF is able to provide the information. |  |  |

For the analysis of a QoE that must meet the provided Packet Delay Budget, the following input is collected to enable the latency analysis between UE and PSA UPF.

Table 6.23.2-2: Input data from OAM related to Packet Delay between UE and Anchor UPF

| Information | Source | Description |
| --- | --- | --- |
| UL/DL GTP packet delay | OAM | Average UL/DL packet delay measurement on N3 between NG-RAN and PSA UPF, as defined in clause 5.4.7 of TS 28.552 [8]. |
| Delay in RAN | OAM | Average Uplink and Downlink packet transmission delay over the air interface, as defined in clause 5.1.1.1 of TS 28.552 [8]. |

# * * * * 2nd change * * * *

6.23.3	Output Analytics

The NWDAF shall be able to provide predictions on QoS and Policy Assistance to consumer NFs, i.e. PCF, as defined in Table 6.23.3-1.

Table 6.23.3-1: QoS and Policy Assistance predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > QoS and Policy Assistance information (1…max) | List of QoS and Policy Assistance information. Max. is the number of the candidate QoS parameter set(s), if applicable. |
| >> QoS parameter set identifier (NOTE 6) | Identifies the QoS parameter set for which the entry applies to. |
| >> DNN | DNN for the PDU Session which contains the QoS flow of the application for which the predicted QoE and the candidate QoS parameter set are provided. |
| >> Application ID or SDF template(s) | Identifies the application or SDF template(s) for which the predicted QoE and the candidate QoS parameter set are provided. |
| >> Duration for the Application | Duration for the corresponding application ID. |
| >> Predicted QoE (NOTE 6) | The predicted QoE of the application when the corresponding QoS parameter set is applied, e.g. average, variance, maximum, minimum of QoE. |
| >> QoS parameter values (NOTE 4) | The parameters in the QoS parameter set and their corresponding values associated to the predicted QoE. |
| >>> 5QI | The reference to 5G QoS characteristics and QoS parameters. |
| >>> Priority level | The Priority Level associated with 5G QoS characteristics, as defined in clause 5.7.3.3 of TS 23.501 [2]. |
| >>> Resource type | The resource type of the corresponding QoS flow, e.g. GBR QoS flow, non-GBR QoS flow, delay-critical GBR QoS flow. |
| >>> Packet Delay Budget | Packet Delay Budget (PDB) indicates the upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [2]. |
| >>> Packet Error Rate | Packet Error Rate (PER) defines an upper bound for a rate of non-congestion related packet losses, as defined in TS 23.501 [2]. |
| >>> Bit Rates (NOTE 2) | The bit rates are only applied to GBR for a SDF template or Application IDs. |
| >>>> GBR | Guaranteed Bit Rate (GBR) for UL and/or DL. |
| >>>> MBR | Maximum Bit Rate (MBR) for UL and/or DL. |
| >>> Maximum Packet Loss Rate (NOTE 2) | The Maximum Packet Loss Rate (UL, DL) indicates the maximum rate for lost packets of the QoS Flow that can be tolerated in the uplink and downlink direction. This is provided to the QoS Flow if it is compliant to the GFBR. |
| >>> Averaging Window (NOTE 2) | The Averaging window is applied when the resource type is GBR QoS. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE), as defined in TS 23.501 [2]. |
| >>> Maximum Data Burst Volume (NOTE 3) | The Maximum Data Burst Volume (MDBV) applies to GBR QoS Flow with Delay-critical resource type. The MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, as defined in TS 23.501 [2]. |
| >> Applicable time window of the QoS and Policy Assistance information | The applicable time window of the QoS and Policy Assistance information within the validity period. |
| >> RAT Type | Indicating the list of RAT type(s) for which the QoS and Policy Assistance Information applies. |
| >> Frequency | Indicating the list of carrier frequency value(s) for which the QoS and Policy Assistance Information applies. |
| >> Validity period | The validity period within the time slot for the QoS and Policy Assistance information analytics as defined in clause 6.1.3. |
| >> Spatial validity | Area where the QoS and Policy Assistance information analytics applies. |
| >> (Optional) Usage Duration information (NOTE 5) | Maximum/Minimum/Average usage duration of the QoS Flows associated to candidate QoS parameter set. |
| >> (Optional) Number of usage (NOTE 5) | The number of times that the QoS Flows associated to candidate QoS parameter set to be used. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Reporting Thresholds". NOTE 2: The output analytics only applies to GBR for a SDF template or Application ID QoS Flow. NOTE 3: The output analytics only applies to GBR for a SDF template or Application ID QoS Flow with Delay-critical resource type. NOTE 4: Only the parameters that have more than one candidate values provided by the consumer shall be included in the NWDAF output and only the values provided by the consumer are allowed. NOTE 5: This parameter is included in analytics output if the Target of Analytics Reporting is a specific UE or Analytics Filter contains specific Application ID. It is determined by NWDAF using the SMF QFI allocation and deallocation events. For example, the duration equals to the timestamps difference between the QoS Flow change(s) (e.g. QFI allocation, change, and deallocation); the usage number counts the number of times the application traffic is allocated to the QoS Flow. NOTE 6: The predicted QoE is reported for the entire QoS parameter set (together with QoS parameter set identifier) if only one parameter value has been provided for each parameter in the set. Otherwise separate predicted QoE for each combination of different parameter values are provided. |  |

# * * * * End of changes * * * *