3GPP TSG-WG SA2 Meeting #168	S2-2504287

7th – 11th April 2025, Goteborg, Sweden	(revision of 2504067)

Source:	Lenovo, China Telecom, NTT DOCOMO, Futurewei, Nokia

Title:	NF descriptions update for AIOTF and NRF

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to add the descriptions of the functional entities in the clause 4.5 of the TS 23.369 for Ambient IoT.

- Introduction

This paper is prepared for the normative work of Ambient IoT, to add more descriptions about the AIOTF and NRF, which are lised as below.

For AIOTF:

- Add the support of aggregation, and providing aggregation assistance information to the AIoT reader, based on the latest conclusions in TR 23.700-13.

- Add the support of AIOTF itself inititaing and performing the inventory and command procedure based on the previously received AF request. The request can be stored at AIOTF as service context and this solution has been proposed in TR Solution 19.

2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an AIOT RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF's AIoT service operation request.

-	Triggering the AIoT RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the AIoT RAN.

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	AIoT RAN selection and optionally a list of AIoT RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the AIoT RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

-  Perform aggregation of the AIoT service responses, determine and provide assistant for the AIoT aggregation in AIoT RAN.

NOTE X: Authentication of AIoT Devices related to AIOTF is defined in TS 33.369[X].

# * * * * End of changes * * * *