3GPP TSG-WG SA2 Meeting #168                                       	S2-2502876

# Goteborg, Sweden, 7 - 11 April, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5380 | rev | --- | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for BSF invocation in URSP Rule Enforcement Information procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO, TEI18 |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 23.502 4.16.16.2 step 3 the (H-)PCF for the UE subscribes to the BSF to be notified when a PCF for the PDU Session for this SUPI is registered in the BSF, by invoking Nbsf_Management_Subscribe (SUPI; DNN). S-NSSAI has to be provided as well, since the same DNN can repeat itself in multiple slices, and is thus ambiguous. Another reason is S-NSSAI is a mandatory parameter. The proper invocation of the BSF Subscribe service operation should be: Nbsf_Management_Subscribe (SUPI; DNN, S-NSSAI). NOTE: It is true that in general we include in service invocation only key parameters. However, once we include DNN in Nbsf_Management_Subscribe, we need to include also S-NSSAI as otherwise it might be misleading (as both DNN and S-NSSAI are mandatory parameters). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to fix invocation of BSF Subscribe service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong/ambiguous invocation of the BSF Subscribe service operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.16.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.16.16.2	Forwarding of URSP Rule Enforcement Information (for non-roaming and HR roaming)

This procedure applies when the PCF serving the PDU session receives URSP rule enforcement information from the SMF and forwards this information to the (H-)PCF serving the UE (see clause 6.1.3.18 of TS 23.503 [20] for non-roaming and HR roaming).

Figure 4.16.16.2-1: Forwarding of URSP Rule Enforcement Information (for non-roaming or HR roaming)

1.	The UE Policy Association is established, as described in clause 4.16.11.

2.	If the (H-)PCF indicates the UE to send reporting of URSP rule enforcement as described in clause 6.6.2.4 of TS 23.503 [20], then depending on operator policies in the (H-)PCF, the (H-)PCF may subscribe to the BSF, then step 3 follows, or provides its PCF binding information to the AMF in step 1 with the indication to be notified about the PCF for the PDU Session for a UE, then step 4 follows.

3.	The (H-)PCF for the UE determines that URSP rules depend on the UE reporting of URSP rule enforcement, it then subscribes to the BSF to be notified when a PCF for the PDU Session for this SUPI is registered in the BSF, by invoking Nbsf_Management_Subscribe (SUPI, DNN, S-NSSAI). Steps 4 and 5a are repeated for each PCF registered for a PDU Session to a SUPI included in the Nbsf_Management.

4.	The (H-)SMF establishes a SM Policy Association as described in clause 4.16.4. The allocated UE address/prefix, SUPI, DNN, S-NSSAI and the PCF address is registered in the BSF, as described in clause 6.1.1.2.2 of TS 23.503 [20]. The SMF may provide the PCF binding information (address(es) of PCF for UE, instance id of PCF for UE) which receives from AMF to the PCF for session during SM Policy Association establishment procedure. If the (H-)SMF has received UE report of URSP rule enforcement via PDU session establishment as described in clause 4.3.2 (step 4), it includes the received traffic information in the SM Policy Association establishment request.

5a.	If the (H-)PCF for the UE subscribed to the BSF in step 3, then the BSF notifies that a PCF for the PDU Session is registered in the BSF, by invoking Nbsf_Management_Notify (UE address(es), PCF address, PCF instance id, PCF Set ID, level of binding). When there are multiple PDU Sessions to the same UE the BSF provides multiple notification to the PCF.

5b.	If the (H-)PCF for the UE sent the request to notify that a PCF for the PDU Session is available to the AMF in step 1, then the PCF for the PDU Sessions sends Npcf_PolicyAuthorization_Notify (EventID set to SM Policy Association established, UE address, PCF address, PCF instance is, PCF Set ID) to the PCF indicated in the PCF binding information provided by the SMF.

6.	The (H-)PCF for the UE subscribes to notifications of event "UE reporting Connection Capabilities from associated URSP rule" as defined in clause 6.1.3.18 of TS 23.503 [20], using Npcf_PolicyAuthorization_Subscribe (EventId set to "UE reporting Connection Capabilities from associated URSP rule", EventFilter set to at least "list of Connection Capabilities") and immediate reporting flag set to the PCF for the PDU Session. The response includes the NotificationCorrelationId and any Connection Capabilities if already available at the PCF for the PDU Session.

7.	If not already installed, the PCF installs the Policy Control Request Trigger to detect "UE reporting Connection Capabilities from associated URSP rule" in the SMF.

8.	When the (H-)SMF receives a UE report of URSP rule enforcement via PDU session modification as described in clause 4.3.3 (step 8a) and the Policy Control Request Trigger is met, it then reports the received traffic information to the PCF serving the PDU Session, by invoking Npcf_SMPolicyControl_Update as defined in clause 6.1.3.5 of TS 23.503 [20] (step 8b).

NOTE:	The case when the (H-)SMF receives a UE report of URSP rule enforcement via PDU session establishment is covered by steps 4-6a above and is described in clause 6.6.2.4 of TS 23.503 [20].

9.	The (H-)PCF for the UE is notified on the "UE reporting Connection Capabilities from associated URSP rule" by Npcf_PolicyAuthorization_Notify (NotificationCorrelationId, EventId set to "UE reporting Connection Capabilities from associated URSP rule", EventInformation including the Connection Capabilities) as defined in clause 6.1.3.18 of TS 23.503 [20].

10.	The (H-)PCF for the UE checks operator policies and then may make policy control decisions based on awareness of URSP rule enforcement as described in clause 6.1.1.5 of TS 23.503 [20].

11.	The SM Policy Association is terminated as described in clause 4.16.6. The allocated UE address/prefix, SUPI, DNN, S-NSSAI and the PCF address are deregistered in the BSF.

12a.	If the (H-)PCF for the UE subscribed to the BSF, then the BSF notifies that the PCF serving a PDU Session is deregistered in the BSF, by invoking Nbsf_Management_Notify (Binding Identifier for the PDU Session).

12b.	If the (H-)PCF for the UE sent the request to notify that a PCF for the PDU Session is available to the AMF in step 1, then the PCF for the PDU Session sends Npcf_PolicyAuthoritation_Notify (EventID set to SM Policy Association termination, Notification Correlation Id).

# *** End of Change ***