3GPP SA WG2 Meeting #168	S2-2503295

# Goteborg, Sweden, 07-11 April 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1622 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DC multiplexing capability negotiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The DC mupltiplexing capability negotiation between UE and IMS network during SIP restratration procedure is specified in clause AC.7.10.2.1. It still unclear how the negotiation result is applied for the consequent IMS sessions. Reference to Stage 3 specifications are required for the new Feature-Caps header field and media feature tag in the Contact header field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that, if UE doesn’t receive the Feature-Caps header field in the response to the REGISTER request, the UE shall not include the media feature tag in the INVITE request. A Note is added to clarify that the Feature-Caps header field and media feature tag in the Contact header field are specified in Stage 3 specifications. Update the functionalities of S-CSCF with supporting the new Feature-Caps header field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DC multiplexing capability negotiation would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.10.2.1, AC.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

#### AC.7.10.2.1	Capability negotiation between UE and IMS network

When the UE supports IMS data channel and the UE is allowed to use IMS data channel, the UE and IMS network may negotiate DC multiplexing capability as follows:

-	The UE and the IMS network mutually negotiate their capabilities of data channel multiplexing during registration procedure.

-	If the UE supports data channel multiplexing, when the UE registers on the IMS network, the UE shall include a media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing in the initial and any subsequent REGISTER request.

-	If the UE supports data channel multiplexing, when the UE initiates a IMS session using data channel multiplexing, the UE shall include the media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing in the INVITE request.

-	If the home IMS network, including the P-CSCF, the IMS-AGW, the MF, supports data channel multiplexing, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the 200 OK response to the initial and any subsequent REGISTER request.

NOTE:	The P-CSCF and IMS-AGW need to understand the multiplexed SDP to avoid gating the multiplexed media streams. The method used by the S-CSCF to discover the data channel multiplexing capabilities of other NFs (P-CSCF, IMS-AGW, MF) is based on network deployment.

NOTE:	The new Feature-Caps header field and media feature tag in the Contact header field indicating capability of supporting data channel multiplexing are specified in TS 26.114 [76].

-	If the UE supports and is allowed to use data channel multiplexing, the UE shall include the media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing in the initial INVITE or any re-INVITE request messag, regardless of data channels being multiplexed or not.

-	If the S-CSCF has not included the data channel multiplexing capability indication in the Feature-Caps header field in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message, the UE shall not include the media feature tag indicating its capability of supporting data channel multiplexing in the Contact header field of the initial INVITE request or any subsequent re-INVITE request message,

# >>>>NEXT CHANGE<<<<

### AC.2.2.5	S-CSCF

In addition to the functions defined in clause 4.6.3, the S-CSCF is enhanced to support the following functionalities:

-	The S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and to any subsequent REGISTER request from UE.

-	The S-CSCF includes a Feature-Caps header field indicating its data channel multiplexing capability in the 200 OK response to the initial and to any subsequent REGISTER request from UE.

# >>>>END OF CHANGES<<<<