3GPP SA WG2 Meeting #168	S2-2503362

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-250xxxx)

Source: 	LG Electronics

Title: 	Architectural Assumptions

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper proposes to add some descriptions into the clause on “Architectural Assumptions” of TR 23.700-19.

1	Discussion

From FS_5GSAT_Ph4_ARC SID (SP-250400),

The study items below aim at investigating on further 3GPP core network functions enhancements to support satellite access with the following work tasks:

-	WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC

NOTE 1:	both CP and UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	WT-1.1 Whether and how to support QoS for IMS voice service over GEO satellite,

-	WT-1.2 whether and how to define IMS enhancement to utilize NB IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements.

-	WT-1.3 Support of IMS emergency call via NB-IoT NTN.

-	WT-1.4 How to obtain UE Geo-location for IMS emergency call and other regulatory services.

NOTE 3:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 4:	There will be NO mobility enhancement developed for the NB-IoT NTN in this WT.

NOTE 5:	Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

-	WT-2: UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available.

-	WT-2.1 Support of UE-SAT-UE communication for roaming case considering the requirements for lawful interception.

NOTE 6:	The scenario will be restricted to two roaming UEs served by same VPLMN and HPLMN.

NOTE 7:	GSMA should be consulted with regard to the roaming requirement for UE-satellite-UE communication, and then further clarification may be needed to specify the detailed roaming scenario..

-	WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked.

NOTE 9: No UE impact is expected for WT-2.

Since the subtasks of WT-1 are planned to be documented as separated key issues, common assumptions in WT-1 are proposed to be captured in Architectural Assumptions.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-19.

* * * * Start of 1st Change * * * *

!! All New Texts !!

4	Architectural Assumptions

The following architecture assumptions are applied to the study:

-	The architecture for satellite access to 5GC as defined in TS 23.501 [x1] is used as a baseline.

-	The architecture for satellite access to EPC as defined in TS 23.401 [x2] is used as a baseline.

-	Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused.

-	No UE impact is expected for UE-SAT-UE communication enhancement.

-	A connection to the ground is always available.

-	GSMA input is required with regard to the roaming requirement for UE-satellite-UE communication before the start of the study for roaming scenario for UE-SAT-UE communication.

-	both CP and UP based NB IoT NTN solutions will be studied in the IMS voice call over GEO via NB-IoT NTN, checkpoint will be set before the normative work to align with RAN scope.

-	Only architecture options with data via SGi will be considered in the study of IMS voice call over GEO via NB-IoT NTN. The option with data via SCEF/T8 will not be considered.

-	Solutions for IMS voice call over GEO via NB-IoT NTN should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

-	There will be NO mobility enhancement developed for the NB-IoT NTN in IMS voice call over GEO via NB-IoT NTN.

-	Depending on operators’ policies, IMS enhancements defined in the IMS voice call over GEO via NB-IoT NTN may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

* * * * End of Changes * * * *