3GPP TSG-WG SA2 Meeting #168 	S2-2503407

Goteborg, Sweden, April 07 – 11, 2025	(revision of S2-25xxxxx)

Source:	Huawei, HiSilicon

Title:	Interaction between the I-SMF and SMF

Document for:	Approval

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract: this paper clarifies two issues related to the interaction between SMF and I-SMF for local offloading management.

1. Introduction

There are two issues related to the interaction between SMF and I-SMF for local offloading management and need be clarified.

2. Discussion

N4 Rule for the UPF controlled by the I-SMF

To generate the N4 rule for the UPF controlled by the I-SMF, the existing ETSUN mechanism is assumed to be used. Hence the I-SMF notifies the selected DNAI to the SMF. As the concrete traffic to be offloaded is controlled by I-SMF and not be aware by the SMF, the SMF need check all related information, e.g. EDI, AFTI, per the received DNAI information. If we do like this, following issue need be considered:

-	Unnecessary filter Setting: The filter to be added to L-PSA include the traffic which is not delivering but can be transferred via the same DNAI. This add unnecessary effort for not used filter checking at the L-PSA.

-	Filter checking per DNAI: the SMF need subscribe and maintain the latest EDI information for the indicated DNN/S-NSSAI. Also SMF need find all filter related to indicated DNAI. This add burden to the SMF.

On the other hand, the more straight forward way is to let the I-SMF directly provide the related SDF information of the offloaded traffic to SMF. SMF generates N4 rule per received SDF information and not need check per DNAI information.

One example can be seen as following:

There are one DNAI—AA in the network, three AF traffic can be offloaded via this DNAI-AA, i.e. the green enty in below table.  The related EDI information can be as following example table:

Table 1: EDI information example

| AS | FQDN | IP ADDRESS | DNAI |
| --- | --- | --- | --- |
| AS--1 | FQDN-A | 00.11.22.33 | DNAI-AA, |
|  |  | 11.22.33.44 | DNAI-BB |
| AS-2 | FQDN-B | 22.33.44.55 | DNAI-CC |
|  |  | 33.44.55.66 | DNAI-AA |
| AS-3 | FQDN-C | 44.55.66.77 | DNAI-DD |
|  |  | 55.66.77.88 | DNAI-AA |

Now per UE DNA query for AS-1 traffic, i.e. for FQDN-A, I-SMF notifies to the SMF that DNAI-AA has been selected for traffic offloading.

ETSUN based mechanism, the mechanism would like this,

1.	DNAI-AA is notified to SMF.

2.	SMF check above table and found that three entries related to DNAI-AA. So SMF notifies three related filter information to I-SMF, i.e. 00.11.22.33, 33.44.55.66, 55.66.77.88.

3.	I-SMF installs three filter information at the L-PSA. In the following handling, two additional filter, i.e., the filter 33.44.55.66, 55.66.77.88 also need be checked for each incoming packet.

So AS-2/AS-3 traffic filters are also installed at the L-PSA even no related traffics are transferring.

SDF based mechansim, the mechanism would like this,

1.	 IP address= 00.11.22.33 is notified to SMF by I-SMF.

2.	Per the received IP address information, SMF only ask to configure that IP address as IP filter.

3.	I-SMF now only configure one filter information at the L-PSA.

So only AS-1 traffic filters is installed at the L-PSA.

From above example it can be seen via the SDF based mechanism, the additional burden to SMF and unnecessary effort at the L-PSA are not added. Hence we suggest to clarify that SDF based mechanism is used.

Proposal 1: The I-SMF notifies the SDF (Service Data Flow) Filter(s) of the offloaded traffic to the SMF. Per the received SDF Filter(s) information, the SMF generates N4 information and sends it to the I-SMF to configure N4 rules on UL CL and Local PSA.

LOM Supported/Allowed Indication

During the PDU session establishment, the LOM Supported indication is indicated by I-SMF to A-SMF. And LOM Allowed Indication is sent back to I-SMF.  After the PDU session is allowed to be applied as LOM, it is suggested later the LOM Allowed Indication is sent from I-SMF to A-SMF, i.e. the LOM Allowed Indication (not LOM Supported Indication) is sent from I-SMF to A-SMF.

By doing this, when the I-SMF receive the Allowed Indication from AMF, the I-SMF need send different indication to SMF per whether it is in PDU session establishement procedure or later mobility procedure even it uses same operation from I-SMF to SMF, e.g. Nsmf_PDUSession_Create service operation. This add additional logic to the I-SMF.

Also, it is unclear what is the SMF behavior if it receives the Allowed Indication from I-SMF? Should it sent Allowed Indicaton back to I-SMF to confirm the LOM is applied now?  We think it is better to let SMF send back Allowed Indication to I-SMF to confirm that now DNS query (and others.) is handled by the I-SMF.

Considering above two issues it is suggested not to make I-SMF behaviour complexity and unify its behaviour.

Proposal 2: When I-SMF receive the LOM Allowed Inidcation from AMF and it support LOM, it indicates the LOM Supported indication to SMF. When SMF receives the LOM supported indication from I-SMF, it sends back LOM Allowed indication to I-SMF.

3. Proposal

It is proposed to discuss and agree above Proposal. The related accompanied paper (S2-2503408/2503409) are also proposed on the concrete changes.