3GPP SA WG2 Meeting #168	S2-2504174

07 - 11 April, 2025, Goteborg, Sweden	(revision of S2-2503614)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0308 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for Local offloading management policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The usage of Offload Identifier is unclear in Local offloading management scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify how the Offload Identifier is used in PCF and SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not completed and hard to be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.10.3.2	Local offloading management policy handling

The Local Offloading Management Policy may be dynamically generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.10.2. The Local offloading Management Policy may be provided with a Offload Identifier. In this case the Offload Identifier is assigned by PCF.

The SMF (including SMF which may act as I-SMF for Local Offloading Management) for Local Offloading Management may also be preconfigured with Local Offloading Management Policy and tagged with related Offload Identifier(s). In this case, the PCF only provide the Offload Identifier to the SMF to indicate the Local Offloading Management Policy applied for the PDU session.

The Offload Identifier is unique within the PLMN

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4].The SMF sends the Local Offloading Management Information to the I-SMF.

The Local Offloading Management Information includes:

-	Offload Identifier (mandatory);

-	IP range(s) (optional);

-	FQDN(s) (optional).

End of CHANGES