SA WG2 Meeting #168                                              S2-2504290

Goteborg, Sweden, 07 – 11April 2025                            (was S2-2504272)

Source:	vivo, Samsung

Title:	AMF selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: It proposes the update the RAN selection and introduce AIOTF/AMF selection for AIoT operation.

1 Discussion

Current specification only covers RAN Reader selection. AMF and AIOTF discovery and selection are still missing.

In clause 6.2.2, there is an EN:

Editor's note: How the NEF selects the AIOTF needs further definition.

Proposal1: It is proposed to introduce AIOTF selection as 5.3.X2 and AMF selection in 5.3.X3, remove the AIOTF selection detail from the reader selection clause and remove the EN in clause 6.2.2.

In clause 5.3, there is an EN:

Editor’s note: It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

According to LS S2-2502832 (R3-250905), “FFS on RAN3 signalling for the other A-IoT RAN information”. However, for indirect case, it is unclear whether A-IoT RAN information can be OAM configuration in the AIOTF. Event it is possible, the AIOTF doesn’t know whether the NG-RAN has setup NGAP connection with the AMF, e.g. which NG-RAN can be routed by which AMF. Hence, after NGAP connection setup with NG-RAN, the AMF need to notify the NG-RAN ID at least to the AIOTF. Whether the other AIoT RAN information also provided by NG-RAN via AMF need to further coordinated with the RAN WG3.

Figure-1 Different NG-RANs may setup NGAP connection with different AMFs

Proposal2: For indirect path, the AIOTF obtains the NG-RAN ID of the NG-RAN which setup NGAP connection with the AMF via AMF (see clause 6.X AIoT information transfer per RAN node from AMF and AIOTF) and the AIOTF obtains the other AIoT information of the NG-RAN via AMF or by OAM configuration.

2 Proposal

It is proposed to update the following contents in TS 23.369.

START OF CHANGES (All new text)

5.3	Discovery and Selection of AIoT node(s)

5.3.X3 	AMF Discovery and Selection

For indirect Connectivity via AMF (see clause 4.2), AMF discovery and selection functionality is implemented in AIOTF.

When the AIOTF performs AMF discovery and selection in order to forward AIoT service operation messages the NG-RAN via AMF, the following applies:

-	After the NG-RAN selection for the AIoT service operation (see clause 5.3.X4), the AIOTF selects the AMF that has association with the selected NG-RAN nodes in order to forward the AIoT service operation messages.

Editor’s note: 	It is FFS whether and how the procedure is performed between AMF and AIOTF in order to provide the NG-RAN ID of NG-RAN from the AMF to the AIOTF.

END OF CHANGES