3GPP TSG-WG SA2 Meeting #168	S2-2504319

Goteborg, Sweden, April 7 - 11, 2025	(revision of S2-2504011, S2-2503200)

Source:	Ericsson, Huawei, HiSilicon, OPPO, Deutsche Telekom

Title:	Terminology Alignment

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC/Rel-19

Abstract: This pCR proposes some updates for terminology alignment.

1. Introduction

In TS 23.369 v0.2.0, there was an EN about the terminology alignment with RAN3:

Editor's note:	The term AIoT RAN needs to be aligned with RAN WGs throughout the TS.

In RAN3 LS (R3-250905), RAN3 informed SA2 the A-IoT RAN is a gNB:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

[Proposal-1] Based on the LS, it is proposed to update AIoT RAN to NG-RAN for terminology alignment and resolve the EN.

Inside the specification, both “RAN Reader” and “AIoT RAN reader” are used to refer to the readers in topology 1, which are opposite to “UE reader” in topology 2. In RAN WID (RP-250796), “reader” is used as a general term. To align the terminology for readers in topology 1 in the specification, it is proposed to align them to “RAN reader”, instead of “AIoT RAN reader” or “NG-RAN reader” which follows [Proposal-1].

[Proposal-2] It is proposed to align “RAN Reader” and “AIoT RAN reader” to “RAN reader”.

In SA2#167, it was agreed to update the “direct/indirect path” to “direct/indirect connectivity” to align with RAN3 where “connectivity” is used. However, some clauses kept the old terminologies.

[Proposal-3] It is proposed to update “direct path” to “direct connectivity” and “indirect path” to “indirect connectivity” to align with RAN3. For simplicity and alignment, it is further proposed to remove the “path” in “direct connectivity path” and “indirect connectivity path”.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.2.0:

* * * Start of Change * * * *

## 4.2	Architecture

### 4.2.1	General

The 5GS System architecture for AIoT includes core network functions, different AIoT Reader architectures and AIoT Devices. The different AIoT Reader architectures allow for different deployment options. The following AIoT Reader architectures are defined:

-	NG-RAN (which supports AIoT Reader), which includes either a direct connectivity between NG-RAN and the AIOTF or an indirect connectivity between NG-RAN and the AIOTF via an AMF.

The NG-RAN in this specification refers to the gNB which supports AIoT related functionalities, as specified in TS 38.300 [5]. The gNB may only support communication with AIoT Devices.

The architecture for Network Exposure Function, using reference point representation, defined in clause 4.2.3 of TS 23.501 [3] is applicable for AIoT, with a southbound interface from the NEF to AIOTF.

### 4.2.2	Architecture for NG-RAN connectivity

#### 4.2.2.1	General

Editor's note:	Whether the connection to the AIOTF should be Nx or N2 is to be determined.

5GS system architecture for AIoT supports the following connectivities to access an NG-RAN:

-	Direct Connectivity: AIOTF communicates with NG-RAN directly.

-	Indirect Connectivity via AMF: NG-RAN and the AIOTF communicate indirectly via an AMF.

Figure 4.2.2.1-1 depicts the complete non-roaming architecture showing the overall 5GS architecture for support of AIoT, including both the Indirect Connectivity and Direct Connectivity.

Figure 4.2.2.1-1: Non-roaming AIoT System Architecture

NOTE 1:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na and N2 as depicted using dashed lines, as all deployments might not use them.

Figure 4.2.2.1-2 depicts the complete non-roaming AIoT system architecture, using the reference point representation.

Figure 4.2.2.1-2: Non-roaming AIoT System Architecture (RAN Readers) in reference point representation

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDR, NEF and NRF as necessary.

NOTE 3:	For clarity, the UDR and its connections with other NFs, e.g. ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

NOTE 4:	For the sake of clarity and to depict the complete reference point architecture, the AMF, Na, N2 and Nx as depicted using dashed lines, as all deployments might not use them.

The architectures in the following clauses showing parts of the overall AIoT architecture specific to each connectivity option:

-	Direct Connectivity: the AIOTF uses Nx to access NG-RAN, and is described in clause 4.2.2.2.

-	Indirect Connectivity via AMF: the AIOTF uses an AMF which uses N2 to access NG-RAN, and is described in clause 4.2.2.3.

NOTE 5:	A deployment that only uses, e.g. the Direct Connectivity only does not need to deploy the NFs, reference points and service-based interfaces associated with the Indirect Connectivity, and vice-versa.

#### 4.2.2.2	Direct connectivity between AIOTF and NG-RAN

In the Direct Connectivity architecture, the AIOTF uses Nx to communicate directly with NG-RAN.

Figure 4.2.2.2-1 depicts the AIoT architecture, using the service-based interfaces, showing only the parts of the AIoT architecture for an AIOTF connecting to NG-RAN directly. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.2-1: NG-RAN - AIOT Direct connectivity Architecture

Figure 4.2.2.2-2 depicts the AIoT architecture, using the reference point representation, showing only the parts of the AIoT architecture for an AIOTF access NG-RAN. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.2-2: NG-RAN - AIOT Direct connectivity Architecture in reference point representation

#### 4.2.2.3	Indirect connectivity between AIOTF and NG-RAN via an AMF

Figure 4.2.2.3-1 depicts the AIoT architecture, using the service-based interfaces showing only the parts of the AIoT architecture for an AIOTF connects indirectly to NG-RAN via an AMF. The remaining parts of the AIoT Architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.3-1: NG-RAN - AIOT Indirect connectivity Architecture

Figure 4.2.2.3-2 depicts the AIoT architecture, using the reference point representation showing only the parts of the AIoT architecture for an AIOTF connects to NG-RAN via an AMF. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.3-2: NG-RAN - AIOT Indirect connectivity Architecture in reference point representation

* * * Next Change * * * *

## 4.3	Reference points

Editor's note:	The reference point names (e.g. Ny, etc.) need to be finalised.

The AIoT Architecture contains the following reference points:

Ny:	Reference point between the AIoT Device and the AIOTF.

Nx:	Reference point between the NG-RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

Na:	Reference point between the AIOTF and the AMF.

Nb:	Reference point between the AIOTF and the NEF.

Nc:	Reference point between the AIOTF and the NRF.

Nd:	Reference point between the AIOTF and the ADM.

Ne:	Reference point between the ADM and the UDR.

Nf:	Reference point between the ADM and the NEF.

In addition to the relevant reference points defined in TS 23.501 [3], in the case of AIoT, these reference points are as follows:

N2:	Reference point between the NG-RAN and the AMF.

N33:	Reference point between NEF and AF.

* * * Next Change * * * *

### 4.5.2	AIoT Readers

Editor’s note:	NG-RAN and AIoT Reader will be aligned later with RAN WGs. It is FFS whether to specify the relationship between NG-RAN and AIoT Reader and which term to use in this TS.

* * * Next Change * * * *

### 4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an NG-RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF's AIoT service operation request.

-	Triggering the NG-RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the NG-RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	NG-RAN selection and optionally a list of RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the NG-RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

* * * Next Change * * * *

### 4.5.7	AMF

The AMF performs the following functions when the NG-RAN and the AIOTF communicate indirectly via an AMF:

-	Relaying signalling for AIoT service between AIoT-RAN and AIOTF.

-	Providing transparent transport for AIoT NAS messages between AIoT Device and AIOTF.

-	Providing transport for NG-RAN Node’s serving area information between NG-RAN and AIOTF.

Editor’s note:	Further functions on AMF is FFS.

* * * Next Change * * * *

## 4.6	Protocol Stacks

### 4.6.1	General

This clause specifies the protocol stacks between entities used for AIoT. The protocol stacks include the following:

-	Protocol stacks between AIoT Device and AFs, including the protocol stacks among AIoT Device, NG-RAN, AMF, AIOTF, NEF and AFs.

Editor's note:	The protocol stacks between RAN and 5GC in the following clauses need to align with RAN specifications.

### 4.6.2	Protocol Stack between AIoT Device and AF

#### 4.6.2.1	General

The general protocol stack between an AIoT Device, NG-RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN is specified in TS 38.300 [5].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack Between AIoT Device and AF

The NG-RAN may communicate to AIOTF via different connectivities, see clause 4.2.2.1. The NG-RAN protocol stack remains the same and is transparent to the difference connectivities. The protocol stacks and routing of information between an NG-RAN and the AIOTF is defined in other clauses depending upon the architecture used:

-	Direct Connectivity: When the NG-RAN communicates with AIOTF directly, the protocol stack is specified in clause 4.6.2.2.

-	Indirect Connectivity: When the NG-RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

The AIoT NAS protocol supports the inventory response and command related signalling.

#### 4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity

Legend:

-	AIoT Reader Control: It is between the AIOTF and NG-RAN.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Connectivity option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN transparently.

#### 4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN and AMF transparently.

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

* * * Next Change * * * *

## 5.3	AIoT Reader Selection

The AIOTF selects NG-RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the NG-RAN node(s) based on the internal area information.

Editor’s note:	Whether and how the AIOTF chooses the RAN reader(s) is FFS.

Editor’s note:	How AIOTF selects the AMF and the NG-RAN in the indirect Connectivity is FFS.

The AF may provide the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information. The NEF queries the NRF to obtain serving AIOTF(s) based on internal area information. Then, the NEF forwards the AIoT service request, including the internal area information, to the AIOTF.

Editor’s note:	How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

The AIOTF may have NG-RAN information and the optional RAN reader information. For direct Connectivity, the AIOTF obtains the information from NG-RAN by the NGAP or OAM configuration.

Editor’s note:	NG-RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of NG-RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how NG-RAN transfers and updates its information to the AIOTF in indirect connectivity.

NOTE:	RAN reader information is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) to determine targeted RAN reader(s) directly.

* * * Next Change * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF provides the assistance information to the NG-RAN together with the service operation requests. The AIOTF determines the assistance information provided to the NG-RAN based on the information received from the AF. The Assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation.

The following assistance information may be provided:

-	AIoT service type (e.g. Inventory, Command);

-	Optionally, approximate number of AIoT devices based on AF request;

-	Optionally, approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

* * * Next Change * * * *

### 6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires additional details, alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

* * * Next Change * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices])  service operation request to the NEF.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF selects the AIOTF to handle the request.

Editor's note:	How the NEF selects the AIOTF needs further definition.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by NG-RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, and assistance information to the selected NG-RAN.

Editor's note:	AIoT Device Identification information needs further definition.

Editor's note:	Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The NG-RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s).

Editor's note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

* * * Next Change * * * *

### 6.2.3	Procedure for Command

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], (AIoT data)) to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The NEF selects AIOTF(s) taking into account the target area information for AIoT service operation and Target AIoT device information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs NG-RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the NG-RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct connectivity as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the NG-RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect connectivity via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (NG-RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to NG-RAN.

13.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT device.

14.	The AIoT device sends the AS D2R message (NAS Command Response) to the NG-RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct connectivity as shown in clause 4.2.2.2, step 15a is performed.

15a.	The NG-RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect connectivity via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The NG-RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Command Response (correlation identifier, Reader ID, NAS Command Response)) to the AIOTF.

16.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

17.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

* * * Next Change * * * *

### 7.2.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.2.1-1.

Table 7.2.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect connectivity.

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

* * * Next Change * * * *

## 7.3	AMF services

### 7.3.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.3.1-1. The Namf_AIoT AMF service is used when NG-RAN and the AIoTF communicate indirectly via an AMF.

Table 7.3.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

### 7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data (i.e. AIoT inventory/command messages) towards NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages to deliver to NG-RAN.

2)	NG-RAN ID.

Outputs, Required: AIoT messages received from NG-RAN, Result indication (Success or Failure), Failure Cause in case of Failure.

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

### 7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages received from NG-RAN.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

Editor's note:	AIoT messages is FFS and coordination with RAN3 is needed.

Editor's note:	It is FFS how to make AIOT be aware of the report is finished in case there are multiple notify for the same operation.

Editor's note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

* * * End of Change * * * *