3GPP TSG-SA2 Meeting #168	S2-2504324

Goteborg, SE, Apr 7th – 11th 2025	revision of S2-2504261

Source:	Huawei, HiSilicon, Offino, Lenovo, Deutsche Telekom

Title:	Terms of Sensing study

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The architecture assumptions for FS_Sensing_ARC are proposed.

# 1. Introduction

This pCR proposes the definition of terms for FS_Sensing_ARC.

The definition in TS 22.137 related to sensing are the following:

3GPP sensing data: data derived from 3GPP radio signals impacted (e.g., reflected, refracted, diffracted) by an object or environment of interest for sensing purposes, and optionally processed within the 5G system.

5G Wireless sensing: 5GS feature providing capabilities to get information about characteristics of the environment and/or objects within the environment (e.g., shape, size, orientation, speed, location, distances or relative motion between objects, etc) using NR radio frequency signals, which, in some cases, can be extended by information created via previously specified functionalities in EPC and/or E-UTRAN.

non-3GPP sensing data: data provided by non-3GPP sensors (e.g., video, LiDAR, sonar) about an object or environment of interest for sensing purposes.

Sensing assistance information: information that is provided to the 5G system from a trusted third-party and can be used to support the derivation of a sensing result. This information does not contain 3GPP sensing data.

NOTE 1:	Examples of sensing assistance information are map information, area information, a UE ID attached to or in the proximity of the sensing target, UE position information, UE velocity information etc.

Sensing contextual information: information that is exposed with the sensing results by 5G system to a trusted third-party which provides context to the conditions under which the sensing results were derived. This information does not contain 3GPP sensing data.

NOTE 2:	Examples includes map information, area information, time of capture, UE location and ID. This contextual information can be required in scenarios where the sensing result is to be combined with data from other sources outside the 5GS.

Sensing group: a set of sensing transmitters and sensing receivers whose location is known and whose sensing data can be collected synchronously.

Sensing receiver: a sensing receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver is part of a RAN node or a UE. A Sensing receiver can be located in the same or different entity as the Sensing transmitter.

Sensing result: processed 3GPP sensing data requested by a service consumer.

Sensing signals: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

NOTE 3:	This definition refers to NR radio frequency signals which, in some cases, can be extended by information created via previously specified functionalities in EPC and/or E-UTRAN.

Sensing transmitter: a sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A Sensing transmitter is part of a RAN node or a UE. A Sensing transmitter can be located in the same or different entity as the Sensing receiver.

Target sensing service area: a cartesian location area that needs to be sensed by deriving characteristics of the environment and/or objects within the environment with certain sensing service quality from the impacted (e.g., reflected, refracted, diffracted) 3GPP radio signals. This includes both indoor and outdoor environments.

Sensing operation in TS 23.137

Sensing operation can be implemented in a couple of different ways, from radar like sensing where the sensing transmitter and sensing receiver are co-located in the same entity (figure 4.2-1), called Monostatic sensing, to have the sensing receiver and sensing transmitter in different entities (figure 4.2-2), also called Bistatic sensing. A more advanced scenario with multiple sensing transmitters and receivers is also possible, called Multistatic sensing. The reflections of the sensing signal sent from the sensing transmitter are received by the sensing receiver and processed to obtain characteristics of the sensed object and its environment (e.g., location). Below figure 4.2-1 and figure 4.2-2 show examples of the sensing operation and the nomenclature used in this specification.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 22.137: " Service requirements for Integrated Sensing and Communication; Stage 1".

# * * * * Second change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

The terms defined in TS 22.137 [X1] apply to this study.

Sensing Service Consumer: the entity that may request and/or consume the Sensing Result

Sensing Entity: the Sensing Entity referring to a Sensing Transmitter or to a Sensing Receiver

Sensing Function: indicating the logical function which is involved to support Sensing Service.

Note: The Sensing Function cannot be a Sensing Entity

Sensing Service capability to collect and provide information about object and/or characteristics of the environment using radio signals,

Editor’s note: the terminology will be further revised during the study work, if needed.

# * * * * End of changes * * * *