3GPP TSG-SA WG2#168	S2-2504347

Gothenburg, Sweden, 7-11 April 2025	(was S2-2504315)

Source:	Qualcomm Incorporated, vivo

Title:	New Solution to WT#2: Enabling dynamic QoS handling for energy efficiency lterby use of Energy Saving QoS Profiles

Document for:	Discussion/Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to enable dynamic QoS handling based on whether energy efficiency mechanisms are being used, by defining an Energy Efficiency QoS profile.

# 1.	Discussion

This contribution proposes to enable dynamic QoS handling by define Energy Saving QoS profiles.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-67:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

## 6.X	Solution #X: Enabling dynamic QoS handling for energy efficiency by use of Energy Saving QoS Profiles

## 6.X.0	High-level solution principles

-	AF provides Energy Saving Service Requirements along with legacy QoS requirements to PCF, the Energy Saving Service Requirements which indicates the service requirement(s) for the AF session if energy saving is applied by the 5GS.

-	PCF generates the Energy Saving QoS profile(s) and the priority list for the QoS flow based on AF’s request and operator policy. The Energy Saving QoS profile(s) can be applied to both GBR and Non-GBR QoS flow.

-	PCF sends the Energy Saving QoS profile(s) and the priority list to SMF and then to NG-RAN along with the legacy QoS profile.

-	NG-RAN selects the Energy Saving QoS profile only if NG-RAN decides to downgrade the QoS parameters due to energy saving.

-	NG-RAN notifies the selected Energy Saving QoS profile to SMF, SMF will further inform the selected Energy Saving QoS profile to PCF and to UPF for charging purpose.

Note: 	The energy-related information at UE or NF level used in KI#2 and KI#3 should be based on robust and accurate enough estimates to be fit for purpose.

## 6.X.1	Description

This solution addresses KI#2.

This solution proposes that PCF generates the Energy Saving QoS profile for the service flow. AF may provide the Energy Saving Service Requirements in addition to the QoS Reference, QoS parameters or Alternative Service Requirements for a service flow. The Energy Saving Service Requirements indicates the service requirement(s) for the AF session if energy saving is applied by the 5GS. PCF authorizes the service requirement(s) from the AF and generates the corresponding PCC rules which include the list of Energy Saving QoS profile(s) and the priorities. The AF uses the procedures described in clause 6.1.3.22 in TS 23.503 to provide the Energy Saving Service Requirements to PCF. If AF does not provide the Energy Saving Service Requirements, PCF may generate the Energy Saving QoS profile and the priority based on operator policy. PCF does not need to collect any energy related information from other 5GC entities to generate the Energy Saving QoS profile.

The Energy Saving QoS profile represents a combination of QoS parameters (e.g., 5QI, ARP, PDB etc.), in the same format as the existing QoS profile as described in clause 5.7.2 in TS 23.501 but with different values for each parameter. There is no new QoS parameter / characteristics proposed for the Energy Saving QoS profile. The Energy Saving QoS profile(s) can be applied to both GBR and non-GBR QoS flows, and is only used for the specific QoS flow when the NG-RAN actives the energy saving.

The PCF shall enable QoS Notification Control and include the derived Energy Saving QoS profile with other QoS profile in the PCC rule sent to the SMF. SMF shall provide the Energy Saving QoS profile together with other QoS profile to the NG-RAN and accordingly NG-RAN may use the Energy Saving QoS profile based on the procedure not defined in SA2, for example NG-RAN interaction with OAM, or local policy at the NG-RAN.

If the Energy Saving QoS profile is received from SMF for the QoS flow, and NG-RAN decides to downgrade the QoS parameters due to energy saving, the NG-RAN selects the Energy Saving QoS profile to apply and notify the SMF that the Energy Saving QoS profile is applied. SMF may indicate the used Energy Saving QoS profile to PCF and also forwards to UPF for charging purpose.

## 6.X.2	Procedures

The solution is proposed with the following principles:

AF provides the Energy Saving Service Requirements to PCF by reusing the setting up an AF session with required QoS procedure as described in clause 4.15.6.6 in TS 23.502, with the following addition:

- AF may provide Energy Saving Service Requirements to PCF.

- PCF generates a list of Energy Saving QoS profile(s) and the priority list for the QoS flow based on AF’s request and operator policy. the Energy Saving QoS profile can be applied to both GBR and non-GBR QoS flows. The Energy Saving QoS profile represents a combination of QoS parameters (e.g., 5QI, ARP, PDB etc.), in the same format as the existing QoS profile as described in clause 5.7.2 in TS 23.501 but with different values for each parameter.

PCF provides the Energy Saving QoS profile to SMF and NG-RAN by reusing the PDU session establishment as described in clause 4.3.2 in TS 23.502 and PDU session modification in clause 4.3.3 in TS 23.502 with the following addition:

- PCF provides the Energy Saving QoS profile(s) and the priority list along with other QoS profile to SMF.

- SMF provides the Energy Saving QoS profile(s) and the priority list along with other QoS profile to NG-RAN.

NG-RAN may use the Energy Saving QoS profile based on the procedure not defined in SA2, for example NG-RAN interaction with OAM, or local policy at the NG-RAN. NG-RAN sends the notification control with the selected Energy Saving QoS profile to SMF as described in clause 4.3.3.2 and clause 4.3.3.3 in TS 23.502 with the addition:

- NG-RAN sends the notification with the selected Energy Saving QoS profile

- SMF sends the selected Energy Saving QoS profile to PCF.

- SMF sends the selected Energy Saving QoS profile to UPF for charging purpose.

## 6.X.3	Impacts on services, entities and interfaces

PCF

-	Generates Energy Saving QoS profile which used when the energy saving is activated in 5GS.

SMF

- Indicate Energy Saving QoS Profile to NG-RAN

- Receive notification of selected Energy Saving QoS Profile from NG-RAN, and propagating that information to PCF and UPF.

NG-RAN

-	Use of Energy Saving QoS Profile for energy efficiency if Energy saving is applied to the QoS flow.

-	Notification of use of Energy Saving QoS Profile to SMF.

UPF

- Indication of Energy Saving QoS Profile usage in CDR.

# >>>>END CHANGE<<<<