SA WG2 Meeting #169	S2-2504874

19 – 23 May, 2025, Fukuoka, Japan

Title:	Discussion paper on UE data collection solution between UE and 5GC

Source:	Ericsson

Document for:	Discussion

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/Rel-20

Abstract of the contribution: the paper provides clarifications about requirements on UE data collection and existing solution between UE and 5GC.

# 1.	Introduction

Based on the RAN requirements, SA2 works on solution for KI#1: Data collection over user plane for UE side model training. Solution: UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. The following figure depicts an example of the solution (described in S2-2504872) using an existing generic EVEX architecture for collecting and reporting data:

# 2.	Discussion

There have been some offline discussions and questions regarding the solution above. The intention of this discussion paper is to provide clarifications on the requirements and applicability of an existing EVEX solution to KI#1 in the TR 23.700-04.

## 2.1. 	EVEX solution based on reference architecture defined in TS 26.531 and procedures defined in TS 23.288 clause 6.2.8

This is a list of requirements from RAN, also included in TR 23.700-04 5.2	Key Issue #1: Transfer of data over UP for UE data collection and how EVEX defined in TS 23.288 clause 6.2.8 and TS 26.531 can fulfil those requirements, as such one of the possible solutions to be considered.

- How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection? The following is a list of references to indicate that EVEX supports this requirement and how it is supported:

    - used over UP connection between UE and 5GC (TS 26.531 clause 4.4, TS 23.288 clause 6.2.8)

    - HTTP/HTTPS based and supports required client-server mechanisms

    - supports standardized data transfer, and it can be extended with the new use cases (e.g. CSI compression data, CSI prediction data, Beam management, or UE Positioning)

    - supports initiation/termination and managing of data transfer

      - supports data reporting assistance procedures (TS 26.531 clauses 4.4 and 5.4, TS 23.288 clause 6.2.8) to provide configuration information to UE (e.g. how to obtain its data collection, how that data is to be sampled, how the collected data is to be reported)

      - supports data reporting procedures used by the client to send data reports to the DCAF (TS 26.531 clauses 4.4 and 5.5, TS 23.288 clause 6.2.8)

2-	How to support in 5GC the full visibility of standardized UE data contents? The following is a list of references to indicate that EVEX supports this requirement and how it is supported:

    - uses standardised data content in current standardised use cases (TS 26.532 clause 7.3)

    - extendable with the new data records for the new use cases (e.g. as part of data records defined for data reporting in TS 26.532 clause 7.3.2.3).

3- How 5GC can support the full controllability of standardized UE data transfer? The following is a list of references to indicate that EVEX supports this requirement and how it is supported:

    - supports MNO control with Data Collection AF (DCAF) used as a trusted function

    - we propose DCAF to trigger establishment of the UP between UE and 5GC when UE data collection is requested and when there is not an existing session that can be reused. (e.g. using standardized Namf_Communication_N1N2MessaageTransfer procedures)

    - supports UE data collection assistance control and data reporting procedures (TS 26.531 clauses 4.4, 5.4. and 5.5)

    - supports event-based exposure of data that can be extended with new events for the new use cases

    - supports information security model (TS 26.531 clause 4.5) for ensuring that collected data is securely transferred and with data integrity and confidentiality

4- Whether and how the UE knows about what the standardized data to be transferred are?

    - supports UE data collection assistance procedures that can be reused for the new use cases (TS 26.531 clauses 4.4 and 5.4)

# 3.	Conclusions

Based on the analysis performed in this paper on applicability of an existing EVEX based solution for the UE data collection new use cases we conclude the following:

- The EVEX solution already meets all necessary requirements, and its existing procedures can be reused. It features a generic design that can be extended to accommodate new data collection use cases and required data contents. Any new solution would need to replicate the same procedures and support the same requirements. We propose considering an existing EVEX-based solution as proposed in S2-2504872.