3GPP TSG-SA2 Meeting #169	S2-2505191

Fukuoka, JP, May 19th – 23rd 2025

Source:	Huawei, HiSilicon, CAICT

Title:	KI#2, New Sol: Authorization and Revocation to Support Sensing Service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Authorization and Revocation to Support Sensing Service are proposed.

1. Introduction

This pCR proposes the solution for Key issue#2 Authorization and Revocation to Support Sensing Service.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  | X |  |  |  |  |

# * * * * Second change (All text new) * * * *

6.X	Solution #X: Authorization and Revocation to Support Sensing Service

6.X.0	High level Principle

This solution is based on the high-level procedure and architecture documented in Sol#X and addresses KI#2. The principles of this solution are:

-	The NEF, based on the AF ID, determines whether to authorizes the AF to request Sensing Service.

-	The Sensing Control Function performs authorization of the sensing service request taking into consideration the request parameters e.g., Target Sensing Service area, Sensing Service type.

6.X.1	Description

6.X.1.1	Terms used within this solution

For the purposes of the present solution the following definition applies:

Sensing Service type: indicating the Sensing scenarios, it includes Object Detection within a Target Sensing Service Area, Border Crossing Detection, Path Monitoring and Path Tracking.

Sensing Control Function: Indicating the logical function which is involved to support Sensing Service and responsible for selecting the Sensing Entities and for providing the related configuration information.

Sensing Processing Function: responsible for receiving the 3GPP Sensing Data and for performing the elaboration for determining the Sensing Result.

Sensing Context: data maintained by the Sensing Control Function containing information relative to a sensing operation (e.g., Sensing Service Consumer, requested Sensing Service type).

6.X.1.2	Solution Description

This solution addresses KI#2.

The Sensing Service authorization consists of two steps: authorisation of the AF performed by the NEF and authorisation of the specific sensing service request performed by the Sensing Control Function.

It is assumed that the general framework used by the NEF to authorise an AF is applied without modification.

If the AF is authorised to request sensing service, upon requesting a sensing service, the NEF forwards the request to the Sensing Control Function which determines whether the sensing service request can be authorised based on the AF identity and request parameters.

The NEF may at any time revoke the AF authorisation: the general framework used by the NEF to revoke AF authorisation is applied without modifications.

The Sensing Control Function may at any time revoke the authorisation granted to an AF to receive sensing results. The Sensing Control Function notifies the AF that the authorisation has been revoked and cancels all relevant sensing procedures related to the sensing service request whose authorisation has been revoked.

6.X.2	Procedure

6.X.2.1	Procedure for Sensing Service authorization

Figure 6.X.2.1-1: Sensing Service authorization procedure.

NOTE 1:	This sub-clause provides the details of Sensing Service authorization and invocation procedure, and corresponds to step 1-2 and step 4-7 of the high-level procedure and architecture (in Sol#X).

1.	The AF sends Nnef_SensingService_Subscribe Request message to the NEF. The request contains the Sensing Service requirement(s) (e.g. Target Sensing Service Area, one-time report/periodical report/event-triggered, etc), Sensing Service type and corresponding information to obtain required Sensing Result from network.

2.	Upon receiving request from AF, the NEF authorizes the AF for usage of the Sensing Service based on AF ID. If the authorization is not granted, the rest of the procedure (steps 3-6) is skipped and the NEF responds to the AF indicating the failure of the Sensing Service authorization in step 7.

3.	NEF may discover and select the candidate Sensing Control Function(s). This step is outside the scope of this solution description and it is addressed by KI#3 solutions.

4.	If the authorization succeeds, then the NEF sends the Nscf_SensingServiceExposure_Subscribe Request message to the Sensing Control Function. The request message contains the Sensing Service type and associated parameters.

5.	Sensing Control Function creates the Sensing Context containing information relative to a sensing operation (e.g., Sensing Service Consumer, requested Sensing Service type).

6.	Upon receiving request from NEF, the Sensing Control Function performs authorization based on the received parameters from NEF (e.g., AF ID, Target Sensing Service Area, Sensing Service Type).

NOTE 2:	For example, the Sensing Control Function determines whether the sensing can be performed in the requested Target Sensing Service area and/or Sensing Service type.

7.	If the Sensing Control Function does not grant the authorization to perform the sensing service request in step 5, steps 8 to step 15 of the overall Sensing procedure (see Sol#X) are skipped and Sensing Control Function reports the authorization failure in the Nscf_SensingServiceExposure_Subscribe response. If the authorization in step 5 succeeds, the Sensing Control Function performs the Sensing procedure to obtain the Sensing Result.

8.	The NEF sends Sensing Service subscription response message to the AF, the response message indicates whether the Nnef_SensingService_Subscribe response is successful or not.

6.X.2.2	Procedure for Sensing Service authorization revocation

6.X.2.2.1	General

As for the authorisation to use a Sensing Service, revocation may be performed at NEF level, that is, the NEF revokes the AF permission to invoke sensing services or performed at the SCF level in case the SCF decides to revoke the authorisation to perform a specific sensing service.

For the revocation at NEF level the same framework used by the NEF to revoke AF authorisation is re-used as is. Revocation at SCF level may happen for example if the AF requests sensing a certain area which is not authorised to sense during certain times.

6.X.2.2.2	NEF revoke the AF capability to invoke sensing service

Figure 6.X.2.2.2-1: Procedure of NEF revoking the AF capability to invoke sensing service

1.	The NEF may decide to revoke the capability of the AF to invoke Sensing Services (e.g., the subscription of AF is not authorized any more) as described in TS 23.501 [3].

2.	The NEF sends the Nsf_SensingService_Unsubscribe request message to the Sensing Control Function to cancel subsequent reporting of the Sensing Result of the requester.

3.	The Sensing Control Function enforces the revocation of the sensing procedure, and cancels the reporting of related Sensing Result according to Nnef_SensingService_Unsubscribe request. The Sensing Control Function determines whether the sensing request towards the Sensing Entity needs to be updated or the Sensing Control Function determines the cancellation of the Sensing procedure of the downstream nodes.

NOTE: 	For example, if the Target Sensing area of the cancelled Sensing Service overlaps with the Target Sensing area of the still-valid sensing service, in that case, the Sensing procedure regarding that area might be maintained. If the removal of Target Sensing area does not have any overlaps, the Sensing Function may interact with the Sensing Entity to update the Sensing procedure.

4.	The Sensing Control Function also removes the Sensing Context associated to the Sensing requests from AF.

5.	The Sensing Control Function sends a Sensing Task Update Request to Sensing Processing Function if it determines to update the task to stop processing data related to the sensing request whose authorisation was revoked, or the Sensing Control Function sends a Sensing Task Release Request to the Sensing Processing Function to cancel the sensing task if it determines to cancel the sensing service.

6.	The Sensing Function sends Nscf_SensingService_Unsubscribe response message to the NEF to indicate that the Sensing Service has been cancelled.

7.	The NEF sends Nnef_SensingService_Notify message to the AF to indicate that the Sensing Service has been cancelled.

6.X.2.2.3	Sensing function triggers to revoke authorisation to perform a specific Sensing Service

Figure 6.X.2.2.3-1: Procedure of Sensing function revoking authorisation to perform a specific Sensing Service.

- The Sensing Control Function determines whether the sensing request towards the Sensing Entity needs to be updated or to revoke the Sensing procedure according to SLA change or the requested Sensing Service is no longer allowed.

NOTE: 	For example, if the Target Sensing area of the cancelled Sensing Service overlaps with the Target Sensing area of the still-valid sensing service, in that case, the Sensing procedure regarding that area might be maintained. If the removal of Target Sensing area does not have any overlaps, the Sensing Function may interact with the Sensing Entity to update the Sensing procedure.

2.	The Sensing Control Function also removes the Sensing Context regarding the Sensing requests from AF.

3.	The Sensing Control Function sends a Sensing Task Update Request to Sensing Processing Function if it determines to update the task or the Sensing Control Function sends a Sensing Task Release Request to the Sensing Processing Function to cancel the sensing task if it determines to cancel the sensing service.

4.	The Sensing Function sends Nscf_SensingServiceExposure_Notify message to the NEF to indicate that the Sensing Service has been cancelled.

5.	The NEF sends Nnef_SensingService_Notify message to the AF to indicate that the Sensing Service has been revoked.

6.X.3	Impacts on services, entities and interfaces

AF:

-	Support Sensing Service request and revocation Sensing Service

NEF:

-	Support Sensing Service authorization and revocation

Sensing function with Sensing Control Function and Sensing Processing Function:

-	Support Sensing Service authorization and revocation

# * * * * End of changes * * * *