SA WG2 Meeting ##169	S2-2504567

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

# Title:	[Draft] Reply LS on Questions on stage 2 requirements for AIML_CN

# Response to:	LS on Questions on stage 2 requirements for AIML_CN (S2-2504575/ C4-250630)
LS on Clarification on SCP data collection(S2-2504509/ C4-251477)
LS on AIML_CN issues(S2-2504505/ C3-251653)

# Release:	Rel-19

# Work Item:	AIML_CN

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

# Name:	Dr. Thomas Belling

E-mail Address:	 thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 1432 for TS 23.288, CR 5429 for TS 23.502, CR 0741 for TS 23.273

1. Overall Description:

A. LS C4-250630

SA2 thanks CT4 for their LS. SA2 has agreed the attached CRs to address the comments of CT4 and would also like to answer the questions raised by CT4 as follows:

1) Table 6.22.2-5 (Data collection from NRF/OAM) in TS 23.288 contains the following Note: " NOTE: If this information is not available in the OAM, it can be obtained from the NRF if available." CT4 observes that the above Note would result in overlapping functionalities in OAM and NRF APIs. Q1: Given that the metrics can be reported by OAM APIs, is it necessary for the NRF APIs to also support reporting the same metrics, and if so, can SA2 explain why?

SA2 Answer A1:  SA2 agrees that all the information listed in table 6.22.2-5 is available via OAM, see clause 5.10 in TS 28.552. However, the majority of companies preferred to also allow collection of the information from the NRF. SA2 thus maintained the NOTE.

2) if the answer to Q1 is yes, TS 23.502 suggests using the Nnrf_NFManagement_NFStatusSubscribe service operation for subscribing and reporting the aforementioned metrics: "- NF load status information, capacity and priority information of NFs and NF Services, etc as described in Table 6.x.2-2 and Table 6.x.2-5 of TS 23.288 [50]." The service operation description states: " Description: Consumer can subscribe to be notified of the following: - Newly registered NF along with its NF services. - Updated NF profile. - Deregistered NF." Q2: If the answer to Q1 is yes, this required functionality does not fit with the current scope of the Nnrf_NFManagement_NFStatusSubscribe service operation. CT4 kindly asks SA2 to clarify which service operation is suitable for this purpose?

SA2 Answer A2: SA2 agrees that the intention of the Nnrf_NFManagementStatusSubscribe service operation is a subscription for information available in NF profiles, but the information listed in table 6.22.2-5 is not part of any NF profile. While the information about load, capacity, and priority related to NFs and NF service instances as listed in Table 6.22.2-2 is part of NF profiles, SA2 does not see a need to introduce related filters in the subscription. SA2 thus removed the quoted bullet. However, SA2 instead defined a new Nnrf_EventExposure service that enables the NRF to expose the information in table 6.22.2-5.

3) Table 6.22.2-1 (UE related Context Data collection) of TS 23.288 specifies: An NF instance may comprise multiple NF service instances supporting the same and/or different service names, e.g. for different network slices, possibly deployed in different Data Centers. Q3: Accordingly, should NF metrics related to signalling storm information be reported per Service Name (i.e. with the NF instance aggregating metrics of all NF service instances supporting the same service name) or per NF service instance?

SA2 Answer A3: SA2 agrees that it is appropriate to report metrics per service instance rather than per service name if such information is available. An NF instance can comprise multiple service instances supporting the same service (name) e.g. for different network slices, and so aggregating all the metrics per service (name) would not allow the consumer to have a correct representation of the traffic handled by all these service instances, e.g. per network slice. SA2 understands that the peer NF can know the NF service instance of the producer via the information provided as part of the binding mechanism. Thus SA2 clarified in Table 6.22.2-1 that Request type designates a NF service instance when available and otherwise NF service name.

4) Clause 5.2.29.2.1 of TS 23.502 specifies: " Abnormal Signalling characteristics: An NF, NF set, Service Type, may be provided as target. A reporting threshold (in terms of absolute transaction or deviation from average, or in terms of a minimum percentage failed transactions) " Q4: Does “Service Type” refer to “service name”? If the answer is yes, should SCP metrics related to signalling storm information be reported per "Service Name" or "Service instance"? If the answer is no, what does "Service Type" refer to?

SA2 Answer A4: SA2 agrees that it is desirable to support reporting per service instance rather than per service name if such information is available for reasons outlined in response to question 3. SA2 understands that the SCP can know the NF service instance of the producer via the information provided as part of the binding mechanism. Thus SA2 clarified in Clause 5.2.29.2.1 of TS 23.502 that the subscription can be per NF service name or per NF service instance and that the reported “Request type” designates a NF service instance when available and otherwise NF service name.

B. LS C4-251477

SA2 also thanks CT4 for their LS. SA2 has agreed the attached CRs to address the comments of CT4 and would also like to answer the questions raised by CT4 as follows:

TS 23.502 clause 5.2.29.2 defines the event(s) supported by Nscp_EventExposure. In this clause one event is defined, which is "Service Signalling characteristics". Furthermore, TS 23.288 clause 6.22.2 defines the list of information which may be reported to NWDAF as the consumer via SCP Event Exposure service. CT4 would appreciate SA2 insights on a few questions related to data collection from SCP:

Q1) Regarding the "UE related Context Data collection" specified in Table 6.22.2-1 of TS 23.288:

Requiring the SCP to provide information with UE granularity means that the NFs in the network would be required to be configured to use the custom header 3gpp-Sbi-Correlation-Info which is optional to support and may be subject to security/regulatory requirements (see TS 29.500). Furthermore, different NFs may use different UE identities, e.g. SUPI, GPSI, etc, for the same UE within this header Also the NFs of earlier releases that do not support this header do not provide UE information to the SCP, which would impact the accuracy of the statistics.

Considering these limitations, CT4 would like to ask SA2 to confirm whether the information with UE granularity is required to be obtained by the SCP for the event exposure purpose.

SA2 Answer B1:  SA2 agrees to remove requirements related to per-UE reporting from the SCP. An SCP can collect information on a per NF instance or NF service instance level. Related IEs have thus been transferred to Table 6.22.2-2,

Q2) According to TS 23.288 clause 6.22.2, the SCP is required to distinguish between different types of communications and signallings and provide information, for example, to collect the data of "Number of redundant signalling of NF" and "A posterior Request type of NF (0..max)" in Table 6.22.2-1, and "SCP Signalling statistics" in Table 6.22.2-3 which are depicted bellow. Currently there is no mechanism to enable SCP to differentiate between different types of communication SA2 refers to, e.g. whether the signalling is redundant or an "A posterior Request".

Therefore, CT4 would like to ask SA2 to consider reverting the requirements on "A posterior Request type of NF (0..max)" and "Number of redundant signalling of NF", unless SA2 could provide clarification on how these requirements can be implemented.

SA2 Answer B2:  SA2 agrees that this information cannot be observed by an SCP. The intention of Note 3 is rather to clarify that an SCP can perform retransmissions. Note 3 was clarified accordingly.

Q3) CT4 would also appreciate SA2 to clarify what "different types of signalling" corresponds to for "SCP Signalling statistics".

SA2 Answer B3:  The “different types of signalling” relate to HTTP request type and addressed service. The wording in the table has been clarified accordingly.

Q4) "Heart-beat related information", "NF load status information", "Capacity and priority information" which are listed in TS 23.288 Table 6.22.2-2, may be provided by NRF. If required, SCP may get this information from NRF. Therefore, CT4 would like to ask for clarification on why the SCP is also required to provide this information.

SA2 Answer B4:  SA2 agrees to remove requirements to expose heart-beat related information from the SCP and defined procedures to expose this information from the NRF instead. However, it is the understanding of SA2 that the SCP can be aware of overload of peer NFs (e.g. for load balancing) based on information received in signalling messages received from the peer NFs and can expose such information

C. LS C3-251653

SA2 thanks CT3 for their LS. SA2 has agreed the attached CRs to address the comments of CT3 and would also like to answer the questions raised by CT3 as follows:

CT3 noticed that the SCP, the NRF, and the UDM, which are analytics consumers of the Signalling Storm analytics according to 3GPP TS 23.288, do not appear in Table 8.1-1 and Table 9.1-1 as NF service consumers of the DCCF and the MFAF.

Question 1:

Can Signalling Storm analytics be subscribed (by the SCP, the NRF, the UDM) via the DCCF (potentially using the MFAF)?

SA2 Answer C1:  SA2 confirms that Signalling Storm analytics be subscribed by the SCP, the NRF, and/or the UDM via the DCCF, and thus agreed to add NRF, SCP, UDM as service consumers of the Ndccf_DataManagement and Nmfaf_3caDataManagemen services.

Further, CT3 noticed that the DCCF is also not explicitly mentioned in 3GPP TS 23.273, which defines the Nlmf_DataExposure service. CT3 also noticed that 3GPP TS 23.288 clause 5A.1 says that "Data Collection Coordination and Delivery is applicable to NWDAFs that request data from a Data Source (e.g. for use in computing analytics)".

Question 2:

Can the DCCF be used to collect data using the Nlmf_DataExposure service?

SA2 Answer C2:  SA2 agrees that the DCCF can be used to collect data using the Nlmf_DataExposure service and added it as service consumer of that service.

Finally, CT3 noticed that information about the "used QoS Profile" are needed from the SMF in order to compute QoS and Policy Assistance analytics, according to 3GPP TS 23.288 Table 6.23.2-1. However, no further details about the exposed QoS Profile contents are provided, while the QoS Profile is not specified to be exposed by any event in the definition of the Nsmf_EventExposure service in 3GPP TS 23.502 clause 5.2.8.3.1.

Question 3:

Can information about the used QoS Profile be exposed by the QFI allocation, change, and deallocation events of the Nsmf_EventExposure service?

SA2 Answer C3:  Yes, the QoS profile is exposed.

Question 4:

If the answer to Question 3 is "yes", can SA2 clarify which QoS Profile parameters are relevant to be exposed for QoS and Policy Assistance analytics?

SA2 Answer C4:  SA2 has the understanding that the QoS profile information signalled to the AN is also exposed via the event. SA2 updated the related text to clarify that the QoS information exposed by the Nsmf_EventExposure Service via the QFI allocation event consist of the QoS profile as defined in Clause 5.7.1.2 of TS 23.501, including 5QI and QoS characteristics according to Clause 5.7.3 of TS 23.501.

2. Actions:

To CT3 and CT4 groups.

ACTION: 	SA2 kindly asks CT3 and CT4 to take the provided answers into account.

# 3. Date of Next SA2 Meetings:

TSG SA WG2 Meeting #170	25th August – 29th August 2025	Goteborg, Sweden

TSG SA WG2 Meeting #171	13th October – 17th October 2025	TBD, China