3GPP TSG-WG SA2 Meeting #169	S2-2504870

Fukuoka, JP, 19th May – 23rd May, 2025

Source:	NTT DOCOMO

Title:	Discussion on structure of AIoT data for read/write/disable

Document for:	Discussion

Agenda Item:	19.14.2

Work Item / Release:	Architecture support of Ambient power-enabled Internet of Things (AmbientIoT-ARC) / Rel-19

Abstract: This paper discusses a potential way forward to resolve an Editor’s Note related to structure of AIoT data for command type read and write.

# 1. Introduction

During SA2#168 discussions took place in attempt to resolve the following Editor’s Note:

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

Unfortunately, there was no consensus on how to proceed. This discussion paper will focus on beneficial aspects for an operator and propose a way forward to resolve the issue.

# 2. Discussion

## 2.1	Current situation

In the current version of TS 23.369 clause 7.2.3, the AF includes the command type e.g., read, write or disable as per the conclusion in clause 8.3.5 of TR 23.700-13.

Up for discussion are three different options.

Option 1: Keep the current level of detail as it has been stable for a while and is aligned with TR conclusions.

Option 2: Generalize and remove details of read/write, making command content less transparent to the core network.

Option 3: Add more details to the command to specify starting memory address to read/write from. In the case of write, the data to be written. In the case of read, the length of data to be read.

### 2.1.1	Option 1

Awareness of the command type may be useful for the core network and access network. For example, the access network may find it useful to distinguish between read and write as the amount of data in uplink/downlink may differ.

The operator of the core network may want to differentiate between commands for charging purposes.

The operator of the core network may want to authorize some AFs to read from the device while other AFs are allowed to write.

Observation 1: The current level of detail gives value to an operator.

### 2.1.2	Option 2

Making the command completely proprietary is not desirable as the advantages in Observation 1 would be eliminated.

### 2.1.2	Option 3

Option 3 can improve interoperability between devices from different vendors as command will work in a standardized way reading and writing from a specified address. However, some more analysis is required to get the interoperability.

Figure 2.1.2-1 Memory illustration of AIoT devices

As per Figure 2.1.2-1 reading from the same location using devices from different vendors may not necessarily mean the right information is retrieved from both devices. It may be so that, in one device a memory pointer provided by an AF may target sensitive internal data of the Ambient IoT device. Hence, an operator has to be aware of each manufacturer’s memory location for user data before authorizing a request.

Observation 2: Specifying starting memory address does not automatically lead to interoperability.

Instead to increase interoperability, it is proposed to do an abstraction, leaving memory storage to implementation but support addressing data reliably by introducing banks. A bank can be reached with a pointer. The Ambient IoT device will based on the pointer retrieve data from a bank or write data to the bank. The physical memory location can then be an implementation specific location. As illustrated in Figure 2.1.2-2.

Observation 3: By abstracting away the need for exact memory addresses, higher interoperability can be achieved.

Figure 2.1.2-2 Illustration of using a bank

# 3. Conclusion and proposal(s)

Based on the observations in clause 2, it is proposed to add support for a memory bank with corresponding pointer to the command procedure of TS 23.369.

If no consensus can be reached it is proposed to resolve the pending Editor’s Note without any changes to avoid confusion.

The proposed changes are reflected in S2-2504881.