SA WG2 Meeting ##169	S2-2505001

19 - 23 May, 2025, Fukuoka, Japan

# Title:	Response LS on Handling of UE Radio Capability for Paging

# Response to:	S2-2504518/ R3-252276

# Release:	Rel-16

# Work Item:	NR_newRAT-Core

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:

#### Name:	ZTE

#### Tel. Number:

#### E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks RAN3 for the LS on Handling of UE Radio Capability for Paging. SA2 would like to provide the following answers.

RAN3 would like to ask SA2 that when the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message, and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then should the NG-RAN node send to the AMF the paging information on the missing NG-RAN RATs or all NG-RAN RATs that the UE supports in the UE RADIO CAPABILITY INFO INDICATION message?

SA2 answer: It is SA2 understanding that the AMF stores the UE Radio Capability for Paging Information without needing to understand its contents. Therefore the NG-RAN shall provide all NG-RAN RATs that the UE supports in the UE RADIO CAPABILITY INFO INDICATION message to AMF. See TS 23.501 clause 5.4.4.3 for more details.

RAN3 would also like to ask if the “that PLMN” in TS 23.501 clause 5.4.4.3 is interpreted as the serving PLMN?

SA2 answer: “that PLMN” in the TS 23.501 clause 5.4.4.3 refers to serving PLMN.

2. Actions:

To RAN3.

ACTION: 	SA2 kindly asks RAN3 to take the above answer into account.

3. Date of Next TSG RAN WG3 Meetings:

TSG-SA2 Meeting #170		Aug 25-28, 2025				Goteborg , SE

TSG-SA2 Meeting #171		Oct 13-17, 2025				China(TBD), CN