3GPP TSG-SA2 Meeting #169	S2-2505115

# Fukuoka, Japan, 19th May 2025 – 23rd May 2025

Source: 	Nokia

Title: 	Interim conclusion for KI#2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes interim conclusion for KI#2 of FS_5GSAT_Ph4_ARC.

# 1	Discussion

For UEs in communication with each other via 5G-LAN services, that are served by the same NGSO satellite, if local data switching via UPF on-board needs to be enabled, then the communication path between two UEs can be significantly shortened by avoiding using ISL and feeder link on the way towards the PSA on the ground. If some UEs are served by different NGSO satellites and require local data switching, then on-board UPFs in different satellites can have N19 towards each other for each VN group, as shown in Figure 1. If UEs are served by different satellites, in order to enable the local data switching between them, N19 forwarding between UPFs on-board should be supported as shown in the Figure 2.

Figure 1: Local Data Switching via multiple UPFs on-board

The different possible interactions via the satellite between UEs are shown in the Figure 2. The Local Data Switching via a UPF on-board is shown as the blue line, where both the UEs are being served by same satellite and same on-board PSA-UPF. The Local Data Switching via multiple UPFs on-board is shown as the red line, where both UPFs are connected via N19 for VN group switching. The Local Data Switching via the ground PSA-UPF is shown as the green line, a case, where N19 is possible between 2 satellites, e.g. due to absence of ISL.

Figure 2: Local Data Switching via multiple UPFs on-board

The existing rel-18 satellite backhaul feature supporting N19 and N6 based forwarding can be reused for NGSO scenario. To counter the possible packet loss, the N6 forwarding can be avoided by restricting the UE-Sat-UE 5GVN group to be handled by a single SMF or single SMF set.

For change of satellite, the rel-19 UE-Sat-UE can be reused to use DNAI based satellite change detection and N19 reestablishment for service continuity. The fallback option can also be supported in the similar way, when N19 forwarding to other satellite is missing.

The existing 5G-VN services allows all SSC mode for session continuity. The same is applicable if the 5G-VN is supported via N6 forwarding. The change of IP address due to change of PSA-UPF can be communicated to other VN members via techniques which are not in scope of this study.

# 2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change (all new)* * *

# 7	Interim Agreements

## 7.X	Interim Conclusions on UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

-	The UE-to-UE traffic whenever possible be locally routed by UPF on-board to the target UE without traversing back to the satellite gateway on the ground;

-	A single SMF or a single SMF set is supported for 5G LAN-type services via UPF on-board, i.e. both UEs are served by the same SMF or same SMF set. The usage of multiple SMF set for same VN group is out of scope.

-	N19 tunnel may be established between two UPFs on-board different satellites to locally switch UE to UE traffic.

-	The SMF is configured with DNAI corresponding to each satellite ID. The SMF determines DNAI based on the configuration and satellite ID received from AMF.

-	On-board UPF selection is done based on the DNAI during initial PDU session setup and Handover.

-	The SMF creates an group-level N4 session on the UPF for a 5G VN group when N19-based forwarding is applied.

-	In the case of N19-based forwarding is required, the followings apply for the PDU Sessions targeting the (DNN, S-NSSAI) associated with a 5G VN group:

-	The PDU Sessions targeting the same 5G VN group are anchored at different UPFs on-board different satellite, the SMF creates the group-level N4 Session in each involved UPF on-board different satellite via N4 Session Establishment procedure in order to establish N19 tunnel(s) between the UPFs across different satellite connected via ISL and install the N4 rules.

-	For the on-board UPF acting as a PSA case, SMF creates N4 rules on the on-board UPF to enable local data switching as following:

-	Based on pre-configuration, the SMF establishes the N19 tunnel between the two UPFs and corresponding UPF-level N4 rules to support local data switch. The IP address(es) in UPF address pools are utilized in the UPF-level N4 session to detect UE-to-UE communication packets which are sent to pre-configured N19 for local data switch.

-	For on-board UPF acting as ULCL/BP/local PSA case:

-	SMF configures Local PSA(s) based on the IP address(es) of target UEs, then UE-to-UE communication packets are detected and sent to the internal interface and optional N19 for local data switch. The local data forwarding rules on L-PSA can be configured based on 5G-VN user plane handing mechanism.

-	SMF configures UL-CL/BP to route data traffic toward the L-PSA.

# * * * * End of changes * * * *