3GPP TSG-SA2 Meeting #169	S2-2505116

# Fukuoka , Japan, 19th May 2025 – 23rd May 2025	(revision of S2-2503078)

Source: 	Nokia

Title: 	FS_5GSAT_Ph4_ARC Key issue for WT#2: UE-SAT-UE communication enhancement for NR NTN NGSO constellation with feeder link available

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes KIs for WT#2 of FS_5GSAT_Ph4_ARC.

# 1	Discussion

The Work Task#2 of the FS_5GSAT_Ph4_ARC SID is:

“-	WT-2: UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available.

-	WT-2.1 Support of UE-SAT-UE communication for roaming case considering the requirements for lawful interception.

NOTE 6:	The scenario will be restricted to two roaming UEs served by same VPLMN and HPLMN.

NOTE 7:	GSMA should be consulted with regard to the roaming requirement for UE-satellite-UE communication, and then further clarification may be needed to specify the detailed roaming scenario.

-	WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked.

NOTE 9: No UE impact is expected for WT-2.”

Based on different aspects of WT#2, different Key issues need to be studied independently as proposed in this paper.

# 2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change (all new)* * *

# 5	Key Issues

## 5.X Key Issue #X: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

### 5.X.1 Description

This key issue studies the UE-Satellite-UE communication services in LEO/MEO satellite for non-IMS services, e.g., 5G-LAN for IP and Ethernet PDU session types.

-	How to support UE-SAT-UE communication for non-IMS services (e.g. 5G-LAN services with IP and Ethernet PDU Session type).

# * * * * End of changes * * * *